TEST CASES

1. RQ-013 → Dokumentenprüfung (parallel, jederzeit)

2. RQ-004/006, RQ-007, RQ-014, RQ-009(Web) → Web/API-Block (keine Abhängigkeiten)

3. RQ-001 → Netzwerk-Scan/Sniffing (kein Custom-Tooling nötig)

4. RQ-008/009 (Netzwerk-Teil) → I&M0 auslesen — VOR Schritt 8!

5. RQ-011 → Physischer F-Adress-/Mode-Test + Power-Cycle Prüfen: löscht dieser Cycle bereits das Log? Falls ja → Schritt 4 muss davor liegen (bereits erledigt ✓)

6. [Vorarbeit] Scapy-PROFIsafe-Layer entwickeln

7. RQ-002/005 → CRC-Kollisionstests (gebündelt)

8. RQ-010 → Flood/Fuzzing-Test

9. RQ-003 → Siegel-/Tamper-Test (separates Muster, falls "stealth"-Zugriff getestet wird) 10. RQ-012 → Erzwungener Cold-Start (ZULETZT – zerstört Log-Zustand für dieses Gerät)

| TC-001 A: Non-safe path isolation & service exposure (nmap CM-001) |  |
| --- | --- |
| Objective | Verify the device exposes only documented services (HTTP only; HTTPS/Websocket/MQTT explicitly unsupported), i.e., test the firewall/isolation assumption at device level |
| Setup | Pi eth0 on OT segment (192.168.0.100); SRIO at 192.168.0.2; PLC in RUN |
| Steps | - sudo nmap -sS -sV -p- -T3 -oA rq001a_srio 192.168.0.2 - sudo nmap -sU --top-ports 200 -oA rq001a_srio_udp 192.168.0.2 |
| Expected | Only HTTP (IoT-Core) + PROFINET-related ports open; no TLS/WS/MQTT; no unexpected services |
| Pass/Fail | PASS if open ports match the architecture port matrix; any extra service = FAIL. |
| Evidence | rq001a_srio.nmap/.xml |

| TC-001 B: Field presence verification in normal operation (Wireshark) |  |
| --- | --- |
| Objective | Confirm F_Dest_Add, F_Source_Add and the 4‑byte CRC are actually present and consistent in live cyclic traffic, as documented. |
| Setup | Laptop NIC1 on switch mirror port (no IP); display filter pn_rt ¦¦ pnio. |
| Steps | - Capture ≥ 2 min of PLC ↔ SRIO PROFIsafe exchange - inspect F-parameter fields and CRC via the PROFIsafe dissector |
| Expected | Every safety frame carries the documented F_Dest_Add + CRC; F_Source_Add is present |
| Pass/Fail | PASS if fields present/consistent with spec; deviation = FAIL. |
| Evidence | rq001b.pcap |

| TC-001 C: F_Source_Add spoofing (Scapy) GAP |  |
| --- | --- |
| Objective | Demonstrate that a frame with a wrong/foreign F_Source_Add is still accepted, because Address Type 1 does not check the source address. |
| Setup | Pi side‑inject on OT segment; SCPU/PLC operating state + output channels monitored; hardware watchdog on target (active tools only with monitoring). |
| Steps | - Capture/rebuild a valid PROFIsafe frame. - Set an incorrect F_Source_Add. - Recompute the 4‑byte CRC over the modified frame (do not keep the original CRC). - Inject via eth0 → switch → SRIO; observe SCPU acceptance and safety reaction. |
| Expected | Frame is processed / no source‑based rejection → proves authorization relies solely on the external firewall assumption |
| Pass/Fail | PASS if Gap‑confirmation test (not a compliance pass/fail) — record observed behavior.present/consistent with spec; deviation = FAIL. |
| Evidence | rq001c.pcap, Scapy script, SCPU/PLC state log |

| TC-001 D: F_WD_Time watchdog fallback MITIGATION |  |
| --- | --- |
| Objective | Verify that on loss/staleness of communication the device passivates to the safe state within F_WD_Time |
| Setup | Pi reconfigured as 2‑port in‑path bridge between PLC and SRIO; F_WD_Time known (default 150 ms) |
| Steps | sudo ip link set br0 down; sleep 1; sudo ip link set br0 up to interrupt the stream longer than F_WD_Time; observe SRIO outputs + PLC diagnosis |
| Expected | Outputs go Low / qualifier = bad within F_WD_Time; re‑integration only after acknowledgment |
| Pass/Fail | PASS if passivation occurs within the configured watchdog time; FAIL if it does not |
| Evidence | rq001d.pcap + PLC diagnostic log + timestamp correlation |

| TC-002-A: CRC-collision on cyclic process data (scapy) GAP |  |
| --- | --- |
| Objective | Prove the SCPU accepts a modified safety frame when the 4‑byte CRC is recomputed to match → CRC gives no cryptographic authentication; "intentional corruption" (RQ‑002) not met. |
| Setup | Pi side‑inject on OT segment (eth0 → switch → SRIO); PLC in RUN; SCPU operating state + F‑DO output channels + PLC diagnosis monitored; hardware watchdog on target (active tools only with monitoring). |
| Steps | - Capture a valid cyclic PROFIsafe frame (Wireshark, mirror port) as reference. - In Scapy, rebuild the frame and alter the process‑data payload (e.g., flip a DO channel bit). - Recompute the 4‑byte CRC (CRC‑Seed24/32) over the modified frame, do not keep the original CRC. - Inject the frame; observe whether the SCPU accepts it (state stays Operate, output follows the manipulated data) or passivates. |
| Expected | Modified‑but‑CRC‑valid frame is accepted → integrity relies on a non‑cryptographic checksum. |
| Pass/Fail | Gap‑confirmation (not compliance pass/fail); record observed SCPU reaction |
| Evidence | rq002a.pcap, Scapy script, SCPU/PLC state log with timestamps |

| TC-002-B: Parameter CRC (iPar/F-Parameter) manipulation (Scapy) |  |
| --- | --- |
| Objective | Test whether modified safety parameters with a recomputed iParCRC/F_ParCRC are accepted during parametrization → extends the CRC‑collision claim to configuration data. |
| Setup | Pi side‑inject; target in parametrization phase (param‑begin/param‑end sequence); acyclic PROFINET record write. |
| Steps | - Capture the parameter record transfer (index 0x200 F‑Parameters / 0x30n iPar). - Modify a safety‑relevant parameter value. - Recompute F_ParCRC / iParCRC accordingly. - Send between param‑begin and param‑end; observe whether the SCPU validates and applies it |
| Expected | Parameter accepted with recomputed CRC → no cryptographic parameter authentication. |
| Pass/Fail | Gap‑confirmation; record whether SCPU enters Operate with manipulated parameters |
| Evidence | rq002b.pcap, Scapy script, SCPU parametrization log |

| TC-002-C: Physical seal integrity (manual) |  |
| --- | --- |
| Objective | Verify the rotary/DIL switch housing cannot be opened or switch positions changed without visibly damaging the seal, and that no covert access path exists |
| Setup | Separate physical bench; one SRIO unit; camera for documentation |
| Steps | - Attempt to detach the seal and re‑apply it without visible damage. - Attempt to change a rotary/DIL switch position while keeping the seal intact. - Attempt housing access via alternative paths (drilling, heat) without disturbing the seal. |
| Expected | Any tampering leaves visible evidence; no covert switch change possible. |
| Pass/Fail | PASS if all attempts produce visible seal damage / are infeasible; FAIL if covert access succeeds |
| Evidence | Photographic before/after documentation, test log |

| TC-003-A: Negative audit proof for physical switch change (curl + Wireshark) GAP |  |
| --- | --- |
| Objective | Show a physical intervention on switch G produces no attributable audit record in the device (error log or I&M0 REVISION_COUNTER) → RQ‑003 not met. |
| Setup | SRIO on OT segment (192.168.0.2); IoT‑Core reachable; laptop on mirror port with pn_rt ¦¦ pnio filter; I&M0 readable via PROFINET acyclic record 0xAFF0 |
| Steps | - Read baseline: curl http://192.168.0.2/devicestatus/errorlog and capture I&M0/REVISION_COUNTER (Wireshark, record 0xAFF0). - Physically change a rotary switch position on G (record your own timestamp). - Immediately re‑read the error log and I&M0/REVISION_COUNTER. - Compare before/after for any entry attributable to the physical action. |
| Expected | No log entry references the physical intervention (REVISION_COUNTER may reflect a parameter change but carries no actor/timestamp; error log shows nothing) → negative proof delivered. |
| Pass/Fail | Gap‑confirmation (not compliance pass/fail); record before/after states. |
| Evidence | baseline + post-action curl outputs, rq003a.pcap, your action timestamp |

| TC-003-B: Debug-port non-logging (JTAGulator/logic analyzer) GAP |  |
| --- | --- |
| Objective | Demonstrate that access via the fused debug/test interface (if any residual access exists) generates no audit entry, reinforcing the "avoids visible damage / unattributable" claim |
| Setup | Separate physical bench; JTAGulator + logic‑analyzer (sigrok/PulseView) on the debug/test contacts |
| Steps | - Probe the debug/test interface for residual accessibility after fusing. - If any interaction is possible, attempt a switch/mode-relevant action via that path. - Read the error log and I&M0 afterward (via Zone B) to check for any entry |
| Expected | Either the port is fully fused (no access, confirms design exclusion, not logging) or access produces no log entry (confirms gap). Both outcomes support the Gap statement. |
| Pass/Fail | Gap‑confirmation; document which outcome occurred. |
| Evidence | logic‑analyzer capture, photos, post‑action curl/pcap |

| TC-004-A: Identification-function conformance & completeness (curl) GAP |  |
| --- | --- |
| Objective | Verify the device identifies safety‑critical software (D) via the API, and determine whether configuration/data (B) is identified at all → tests RQ‑004 completeness. |
| Setup | SRIO on OT segment (192.168.0.2), IoT‑Core reachable; PLC in RUN (device in Operate) |
| Steps | - curl http://192.168.0.2/deviceinfo/hwversion - curl http://192.168.0.2/deviceinfo/swrevision - curl http://192.168.0.2/deviceinfo/swinfo/scpuversion - curl http://192.168.0.2/deviceinfo/swinfo/cpubootloaderversion and /scpubootloaderversion - Search the IoT‑Core tree for any endpoint that identifies configuration/parameter data (B) (e.g., safe parameter set / iPar identification) |
| Expected | Steps 1–4 return valid software/HW identification (D identified). Step 5 finds no endpoint identifying safety‑relevant configuration/data (B) → gap confirmed (identification incomplete) |
| Pass/Fail | PASS for the software‑identification part if all versions returned; gap‑confirmation for the missing data/config identification |
| Evidence | curl outputs saved as rq004a_*.txt |

| TC-004- B: Cleartext transmission of identification data (Wireshark) GAP |  |
| --- | --- |
| Objective | Confirm the identification data is transmitted without TLS and without access control (confidentiality weakness). |
| Setup | Laptop NIC1 on mirror port; filter http ¦¦ tcp.port==80; trigger the same GET requests as TC‑004‑A from the Pi. |
| Steps | - Start capture on mirror port. - Issue the /deviceinfo/* GET requests. - Inspect the captured stream for a TLS handshake vs. cleartext HTTP |
| Expected | No TLS handshake; full FW/HW versions visible in cleartext; no authentication challenge |
| Pass/Fail | Gap‑confirmation (confidentiality weakness demonstrated); note this is a secondary finding to RQ‑004 |
| Evidence | rq004b.pcap showing cleartext /deviceinfo responses |

| TC-005- A: Parameter CRC-collision (Scapy) GAP |  |
| --- | --- |
| Objective | Prove the SCPU accepts modified safety parameters when iParCRC/F_ParCRC are recomputed to match → CRC ≠ cryptographic integrity for configuration data |
| Setup | Pi side‑inject on OT segment; target driven into the parametrization phase (param‑begin/param‑end); acyclic record write (F‑Parameters index 0x200, iPar 0x30n); SCPU state + PLC diagnosis monitored; watchdog on target (company safety rule) |
| Steps | - Capture the parameter transfer as reference (Wireshark, mirror port). - In Scapy, modify a safety-relevant parameter value (e.g., a filter/symmetry/F_WD_Time field). - Recompute F_ParCRC (and iParCRC if the iPar block is touched) over the modified data. - Send between param‑begin and param‑end; observe whether the SCPU validates and enters Operate with the manipulated parameter. |
| Expected | Parameter accepted with recomputed CRC → no cryptographic parameter authentication. |
| Pass/Fail | Gap‑confirmation; record whether SCPU reaches Operate with the manipulated value. |
| Evidence | rq005a.pcap, Scapy script, SCPU parametrization/state log |

| TC-005- B: Firmware integrity/secure-boot absensce (documentation + structural review) |  |
| --- | --- |
| Objective | Establish, as a negative proof, that no firmware signing / secure-boot verification protects asset D against corruption. |
| Setup | Manufacturer documentation (COM/FW‑Update spec, security requirements); optionally a firmware container obtained via the IoT‑Core /firmware upload path (FSTM Stage 2), if permitted |
| Steps | - Review the FW‑Update and security documentation for any signature/secure‑boot requirement (expected: none documented). - Inspect the IoT‑Core /firmware/install + /firmware/container behavior for integrity verification beyond size/chunk checks. - Optional (if image available): binwalk -E firmware.bin (entropy — signed/encrypted vs. plain) and header inspection; note checksec is N/A for the bare‑metal STM32 image |
| Expected | No documented or observable signature/secure‑boot verification → part D structurally unaddressed |
| Pass/Fail | Gap‑confirmation via structural/documentation review (SVV‑1 style); no active exploit required. |
| Evidence | documentation references, IoT‑Core behavior notes, optional binwalk entropy output. |

| TC-005- C: Malformed firmware upload rejection (IoT-Core, part D, only if upload permitted) |  |
| --- | --- |
| Objective | Check whether a deliberately corrupted firmware container is rejected on integrity grounds vs. only on size/format |
| Setup | IoT‑Core /firmware/container upload in a controlled test window; device in Update state; no rollout to a production unit (company rule) |
| Steps | - Upload a container with a modified payload but valid size/chunk framing via start_stream_set/stream_set; - trigger /firmware/install; observe accept/reject and reason. |
| Expected | If accepted/installed → confirms absence of cryptographic image verification; if rejected, capture the rejection reason (size/CRC vs. signature). |
| Pass/Fail | Gap‑confirmation depending on outcome; document precisely which check rejected it (if any). |
| Evidence | upload log, device state transitions, IoT‑Core responses. |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |

| TC-006- A: |  |
| --- | --- |
| Objective |  |
| Setup |  |
| Steps |  |
| Expected |  |
| Pass/Fail |  |
| Evidence |  |