Safe Remote I/O (SRIO) — Security Test Catalogue

Revised & Environment-Validated Edition (IEC 62443-4-1 SVV / 62443-4-2)

14 Requirements · Two-Level Catalogue · CM-001…CM-010 integrated

Prepared for: N. Nuber — Degree Apprentice Safety Products, ifm (Kressbronn)

0. How to read this document

Every test case opens with a STATUS banner:

- Feasible — runs as written in your lab after the global IP/interface substitution.

- Adapted/Modified — runs only after a specific change (switch mirror, spare unit, or method swap); the change is stated in “Why”.

- Not Feasible (Environment/Device Limitation) — cannot be executed in this lab; retained as an analytical / documented residual-risk finding.

1. Test environment & global conventions

1.1 Topology and address plan

| Node | IP / Port | Role in tests |
| --- | --- | --- |
| Kali Linux PC | 192.168.0.80 / P3 | Attacker host (USB3-GbE dongle) — origin of all active tests |
| SRIO (DUT) | 192.168.0.2 / P2 | IoT-Core HTTP:80 target; PROFIsafe consumer |
| PLC (F-Host) | 192.168.0.1 / P1 | Monitored every active test; DO read-back source |
| Company Laptop | 192.168.0.7 / P4 | TIA Portal / engineering + I&M reads |
| Managed Switch | 192.168.0.91 | Mirror source (P1+P2 → P3) — see 1.2 |

Set these once per shell so no command is mistyped:

export SRIO=192.168.0.2 PLC=192.168.0.1 LAPTOP=192.168.0.7

export IFACE=$(ip -br link | awk '/enx|eth1/{print $1; exit}')   # USB3-GbE dongle

echo "target=$SRIO  attacker-iface=$IFACE"

1.2 The “mirror” dependency (answers: “Was für mirror?”)

PROFINET/PROFIsafe cyclic frames (EtherType 0x8892) between PLC↔SRIO are L2 unicast. On a switch they reach only the destination port, so Kali on P3 will NOT passively see them. Every “Setup A (passive capture)” step therefore requires ONE of:

- Option A (preferred): configure port mirroring on switch 192.168.0.91, mirroring Port 1 AND Port 2 → Port 3.

- Option B: transparent inline bridge on Kali (needs two wired NICs + re-cabling PLC–Kali–SRIO). This is also the ONLY way to attempt the in-path tests.

1.3 The 2-connection HTTP cap (cross-cutting)

IoT-Core allows only 2 concurrent HTTP connections (SRIO-7909). Extra connections return resets/timeouts that are NOT vulnerabilities. Throttle every HTTP tool: Nmap --max-parallelism 2; Nikto -Pause 1 single-thread; ZAP threads=1-2; Siege -c 2 (compliance run); boofuzz single connection + sleep_time; OpenVAS expect low yield.

1.4 Standing safety rule (live SIL3 unit)

-p- / -sV / OpenVAS / fuzzing against the live DUT wired to the PLC can trip EC3/EC4 diagnosis or passivation. Run only in an approved window, keep the PLC/PROFIsafe connection monitored, and prefer a spare/standalone unit for aggressive tests.

2. Integration of company baseline tests CM-001 … CM-010

| CM | Tool / topic | Your finding | Disposition in this catalogue |
| --- | --- | --- | --- |
| CM-001 | Nmap exposure | Executed, evaluable | Feasible — drives TC-RQ001-02 (fix IP + --max-parallelism 2). |
| CM-002 | Nmap segmentation | N/A (no zone model) | Adapted — flat L2 lab; repurposed to port-deactivation / MRP check (component-level). |
| CM-003 | Hydra creds | N/A (no login) | Not Feasible — no auth endpoint (SRIO-8299). Evidence = CM-001 (no login ports). |
| CM-004 | ZAP RBAC | N/A (single anon role) | Adapted — ZAP repurposed to prove read-only enforcement (TC-RQ011-01). |
| CM-005 | SSLscan TLS | N/A (443 closed) | Adapted — kept as NEGATIVE evidence (no TLS) in RQ004-03 / RQ014-02. |
| CM-006 | Lynis hardening | N/A (embedded RTOS) | Not Feasible — no OS/shell; substitute Nmap + curl -I + manual baseline. |
| CM-007 | Nikto debug iface | Executed | Feasible — single-thread; supports RQ001-02 / RQ010. |
| CM-008 | OpenVAS/GVM | Failed to run | Fixed — see §5 (feed sync + Consider Alive); low yield caveat. |
| CM-009 | Siege load | Executed 50 & 3 conn | Feasible — -c 2 = compliance, -c 50 = availability (RQ007-03 / RQ010-04). |
| CM-010 | boofuzz | 0x1000 device error | Fixed — see §6; 0x1000 decoded as COM diagnosis event, corrected script. |

2.1 CM-008 (OpenVAS/GVM) — working procedure for this lab

Root cause is almost always feed sync / service state + ICMP alive-test on a device that may not ping. Run in order:

sudo gvm-setup            # first run only — COPY the admin password it prints ONCE

sudo gvm-feed-update      # wait until NVT + SCAP + CERT feeds FULLY finish

sudo gvm-check-setup      # every line must be green

sudo gvm-start            # UI: https://127.0.0.1:9392

# Create target with a NON-ICMP alive test (SRIO may not answer ping):

gvm-cli --gmp-username admin --gmp-password 'YOUR_SETUP_PW' socket --xml \

"<create_target><name>cm008_srio</name><hosts>192.168.0.2</hosts>\

<alive_tests>Consider Alive</alive_tests></create_target>"

Config: “Full and fast”, default port list, scanner “OpenVAS Default”. Checklist if empty: (1) feeds not synced [#1 cause]; (2) ospd-openvas/Redis/notus down → check-setup; (3) lost admin pw → sudo runuser -u _gvm -- gvmd --user=admin --new-password='…'; (4) ICMP-only alive test; (5) wrong subnet. Caveat: against an embedded RTOS exposing only HTTP:80 + PROFINET/DCP, OpenVAS will find almost nothing — the real CVE/patch view must come from a manufacturer SBOM (TC-RQ004-02 / RQ006-02).

2.2 CM-010 (boofuzz) — the 0x1000 event

0x1000 falls inside the SRIO manufacturer-specific diagnosis range 0x100–0x7FFF (SRIO-7825) and the IoT-Core lives on the non-safety COM/QM CPU which logs ErrorEvents (SRIO-10665). Most likely reading: the HTTP layer REJECTED malformed input and logged a COM-side diagnosis — i.e. input-validation working, potentially a PASS — provided (a) COM did not watchdog-reset/hang, (b) IoT-Core recovered, and (c) the PROFIsafe/PLC connection stayed up (qualifier good). If (c) held you also have positive RQ-010 domain-separation evidence. Decode the exact code from the error log and correlate with the manual’s HEX table (SRIO-15221):

curl -s http://$SRIO/devicestatus/errorlog/loglist   # decode 0x1000 vs manual table

Also rule out the mundane cause: with only 2 HTTP connections, an un-throttled fuzzer can exhaust the pool — the corrected script in TC-RQ010-03 removes that ambiguity with a per-case liveness monitor.

3. RQ-001 — Connection of another device shall not lead to a hazardous situation

IEC 62443-4-2 CR mapping: CR 3.1, CR 3.1 RE(1), CR 3.6, CR 1.2

TC-RQ001-01 — Baseline safety exchange

STATUS: Adapted/Modified

Why: Original uses passive PROFIsafe capture, which needs the switch mirror (§1.2). Method swapped to PLC-side verification via TIA watch table + DO read-back, so it runs even without the mirror.

Level 1 — Abstract Test Scenario

| Traceability | RQ-001 → CR 3.1 / CR 3.6 → Asset A, C |
| --- | --- |
| Target Threat | N/A (functional baseline) |
| SVV Category | SVV-1 |
| Objective | Show that with correct F_Dest_Add and valid CRC the consumer exchanges cyclic data, qualifier=good, DO follows the control byte, no passivation. |
| Abstract Steps | - Configure controller with matching F_Dest_Add; establish safe connection. - Toggle a safe input and command a safe output. - Observe qualifier + DO read-back over a sustained interval. |
| Pass / Fail | PASS if qualifier stays good and DO mirrors the command with no passivation; FAIL on qualifier=bad or command/output mismatch. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | TIA on laptop (192.168.0.7). If mirror configured: also capture on Kali. DO looped back to a spare PLC F-DI for independent read-back. |
| --- | --- |
| Tools | TIA Online & Diagnose; (optional) tshark on mirror; camera. |

Procedure / Commands:

# Optional passive capture ONLY if switch mirror P1+P2->P3 is active:

sudo tshark -i "$IFACE" -f "ether proto 0x8892" -w cm_rq001-01_baseline.pcap

# Primary evidence: TIA watch table monitoring F-DI/F-DO value + PROFIsafe qualifier;

# toggle DI button, command DO from PLC, record DO read-back.

Expected / Evidence: qualifier=good throughout; DO follows control byte; DO-readback mirrors state; no passivation. Evidence: TIA watch-table screenshot, DO-lamp video, (opt.) pcap. [PLACEHOLDER: Result]

TC-RQ001-02 — Attack-surface enumeration (non-safety path)

STATUS: Feasible

Why: Direct Nmap from Kali to SRIO; this is exactly CM-001 with corrected IP and the 2-connection throttle.

Level 1 — Abstract Test Scenario

| Traceability | RQ-001 → CR 3.1 → Asset F |
| --- | --- |
| Target Threat | Spoofing / Tampering |
| SVV Category | SVV-3 |
| Objective | Enumerate services a connecting device reaches on the non-safety path; compare to the approved port matrix. |
| Pass / Fail | PASS if only design-approved services reachable (TCP/80 + PROFINET/DCP); FAIL if any port outside the matrix (Telnet/FTP/SSH/HTTPS). Plaintext HTTP recorded as defence-in-depth only. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali on P3. Monitor PLC during scan. |
| --- | --- |
| Tools | Nmap (CM-001), Nikto (CM-007). |

Procedure / Commands:

sudo nmap -sS -sV -p- -T3 --max-parallelism 2 -oA cm001_srio $SRIO

sudo nmap -sU --top-ports 200 -oA cm001_srio_udp $SRIO

nikto -h http://$SRIO -Pause 1 -maxtime 600 -o cm007_srio.html -Format html

Expected / Evidence: Only TCP/80 + PROFINET/DCP infrastructure; no 22/23/21/443. Any extra port = FAIL. Evidence: cm001_srio.*, cm007_srio.html, port-matrix diff. [PLACEHOLDER: Result]

TC-RQ001-03 — Codename / CRC-mismatch rejection (parallel injection)

STATUS: Adapted/Modified

Why: Parallel injection is possible from Kali, but frame reference-capture needs the mirror; treat as boundary evidence (parallel injection must NOT hijack the consumer). Full success is not expected — it proves the consecutive-number collision drives passivation.

Level 1 — Abstract Test Scenario

| Traceability | RQ-001 → CR 3.1(+RE1)/CR 3.6 → Asset A, C |
| --- | --- |
| Target Threat | Spoofing + Tampering |
| SVV Category | SVV-4 |
| Objective | Prove a mis-addressed / bit-flipped telegram is rejected and resolves to the safe state, not to uncommanded output. |
| Pass / Fail | PASS if crafted telegrams are rejected / device passivates (DO de-energised, qualifier bad) with no uncommanded actuation; FAIL if any crafted frame causes an uncommanded output. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali injects on P3; mirror capture if available; PLC in OPERATE. |
| --- | --- |
| Tools | Scapy (pnio, pnio_rtc contrib), tshark, DO read-back via PLC. |

Procedure / Commands:

# Reference frame (needs mirror). Then craft & send parallel frames from Kali:

python3 - <<'PY'

from scapy.all import *

load_contrib('pnio'); load_contrib('pnio_rtc')

# (a) mis-addressed: alter PROFIsafe F-address/consumer-ID

# (b) bit-flip PD leaving CRC stale

# sendp(frame_bad, iface=''+__import__('os').environ.get('IFACE','eth1')+'', loop=1, inter=0.002)

PY

Expected / Evidence: Parallel injection does NOT hijack the consumer; consecutive-number collision drives passivation or frames ignored; DO unchanged/de-energised = PASS. Evidence: inject pcap, PLC diag buffer, DO-lamp video. [PLACEHOLDER: Result]

TC-RQ001-04 — In-path adversary-in-the-middle boundary

STATUS: Not Feasible (Environment/Device Limitation)

Why: Requires a transparent inline bridge AND forging a valid PROFIsafe frame with consecutive-number takeover inside F_WD_Time — not achievable in this lab with a single unit and a USB dongle. Retained as documented residual-risk boundary (G-2), not scored as a device FAIL.

Level 1 — Abstract Test Scenario

| Traceability | RQ-001 → CR 3.1(+RE1) → Asset C, F |
| --- | --- |
| Target Threat | Spoofing + Tampering |
| SVV Category | SVV-4 |
| Objective | Characterise the residual boundary: a full in-path takeover (suppress producer + continue sequence + valid-CRC frame) that the consumer cannot distinguish from legitimate. |
| Pass / Fail | Boundary characterisation (NOT a device FAIL). A device FAIL is recorded only if a PARALLEL injection (TC-RQ001-03, no suppression) had succeeded. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Would require inline bridge (Kali NIC1↔P1, dongle↔SRIO). Not built. |
| --- | --- |
| Tools | bridge-utils, ebtables, NetfilterQueue + Scapy (documented, not executed). |

Procedure / Commands:

# NOT EXECUTED in this environment. Documented method only:

# brctl addbr br0; brctl addif br0 eth0; brctl addif br0 $IFACE; ip link set br0 up

# ebtables + NFQUEUE script to suppress producer and inject valid-CRC frame.

Expected / Evidence: Documented residual risk covered by the network-segregation control (SRIO-9402). No lab result. [ANALYTICAL FINDING]

TC-RQ001-05 — Channel-disruption watchdog response

STATUS: Feasible

Why: Physical link interruption of the SRIO fieldbus patch cable + DO read-back via TIA needs no mirror. Timing precision improves with a mirror but is not required for PASS/FAIL.

Level 1 — Abstract Test Scenario

| Traceability | RQ-001 → CR 3.1/CR 3.6 → Asset A, C, E |
| --- | --- |
| Target Threat | Denial of Service |
| SVV Category | SVV-2 |
| Objective | Verify that interruption beyond F_WD_Time causes passivation to the safe state and that re-integration needs explicit acknowledgement. |
| Pass / Fail | PASS if passivation within the specified response time, DO de-energised, qualifier bad, and re-integration gated by acknowledgement; FAIL if outputs stay energised beyond F_WD_Time or auto-re-integration occurs. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Pull SRIO P2 patch cable (hold > F_WD_Time, default 150 ms). TIA diagnostics + DO read-back. |
| --- | --- |
| Tools | TIA, stopwatch/DO-readback, (opt.) tshark timestamps on mirror. |

Procedure / Commands:

# If mirror active, timestamp the last valid frame:

sudo tshark -i "$IFACE" -Y "pn_io.ps" -w cm_rq001-05_wd.pcap -t a &

# Interrupt: unplug SRIO Port-2 patch cable; hold > F_WD_Time; measure DO de-energise;

# restore link; confirm re-integration requires PLC acknowledgement.

Expected / Evidence: Passivation within response time; DO de-energised, qualifier bad; no auto re-integration. Evidence: DO-lamp video, PLC diagnostic (passivation + ack), (opt.) pcap delta. [PLACEHOLDER: Result]

4. RQ-002 — Protect hardware transmitting connection/access-relevant data against corruption

IEC 62443-4-2 CR mapping: CR 3.1, CR 3.1 RE(1), CR 3.4, EDR 3.11

TC-RQ002-01 — Parameter-CRC rejection (iParCRC / F_ParCRC)

STATUS: Feasible

Why: Executed entirely from TIA on the laptop (192.168.0.7) — no injection, no mirror. A wrong F_iPar_CRC is entered and the connection establishment observed.

Level 1 — Abstract Test Scenario

| Traceability | RQ-002 → CR 3.4 → Asset C |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-1 |
| Objective | Show the device rejects a configuration whose parameter integrity value does not match and stays in the safe/parametrization state (EC3). |
| Pass / Fail | PASS if mismatch → configuration error, device stays in Parametrization, qualifier bad, no OPERATE handshake; FAIL if a mismatched config is accepted. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | TIA + baseline valid config. Optional capture on mirror. |
| --- | --- |
| Tools | TIA Portal; (opt.) tshark. |

Procedure / Commands:

# In TIA: set valid config, note correct F_iPar_CRC; then enter a WRONG F_iPar_CRC,

# compile, download. Optional capture of connection establishment:

sudo tshark -i "$IFACE" -Y "pn_io" -w cm_rq002-01_crc.pcap

Expected / Evidence: EC3-class F-parameter error; stays in Parametrization; qualifier bad; no OPERATE = PASS. Evidence: TIA diagnostic-buffer screenshot, (opt.) pcap. [PLACEHOLDER: Result]

TC-RQ002-02 — Stale integrity-value fault detection

STATUS: Adapted/Modified

Why: Requires crafting/replaying a frame with stale CRC from Kali; feasible to send, but clean verification needs the mirror + a live reference frame. Treat as effectiveness-under-fault evidence.

Level 1 — Abstract Test Scenario

| Traceability | RQ-002 → CR 3.1 → Asset C |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-2 |
| Objective | Modify a payload byte while leaving the CRC stale → detectable mismatch → passivation (or frame ignored). |
| Pass / Fail | PASS if mismatch detected and device passivates or ignores the corrupted frame; FAIL if the modified payload is acted upon. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali injection on P3 + mirror capture; OPERATE. |
| --- | --- |
| Tools | Scapy (pnio_rtc), tshark, DO read-back. |

Procedure / Commands:

# Capture one cyclic frame (mirror), modify one PD byte leaving CRC unchanged, replay:

python3 - <<'PY'

from scapy.all import *

load_contrib('pnio'); load_contrib('pnio_rtc')

# sendp(frame_stale_crc, iface=__import__('os').environ['IFACE'], loop=1, inter=0.002)

PY

Expected / Evidence: CRC mismatch detected → passivate/ignore; DO not driven by tampered data = PASS. Evidence: pcap, PLC diagnostic, DO video. [PLACEHOLDER: Result]

TC-RQ002-03 — Valid-integrity injection over modified data

STATUS: Not Feasible (Environment/Device Limitation)

Why: Needs single-stream substitution on an inline bridge (suppress genuine frame, forward re-CRC’d frame). No bridge / single unit. Retained as gap-characterisation (checksum ≠ authenticity, G-1).

Level 1 — Abstract Test Scenario

| Traceability | RQ-002 → CR 3.1(+RE1) → Asset C |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-4 |
| Objective | Show that a recomputed-CRC telegram over modified PD is accepted — protection against accidental but not intentional corruption. |
| Pass / Fail | Gap-characterisation. Expected acceptance confirms authenticity rests on the black channel. A rejection would indicate an undocumented crypto control (investigate). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Would require inline bridge + PROFIsafe CRC routine (public CRC-Seed24/32). Not built. |
| --- | --- |
| Tools | Scapy + CRC routine, NetfilterQueue (documented). |

Procedure / Commands:

# NOT EXECUTED. Method: implement PROFIsafe F-CRC over modified PD with public seed;

# on bridge, suppress genuine frame and forward the re-CRC'd frame.

Expected / Evidence: Documented residual risk (checksum ≠ authenticity). No lab result. [ANALYTICAL FINDING]

TC-RQ002-04 — Parameter-integrity forgeability assessment

STATUS: Feasible

Why: Pure desk analysis on Kali/laptop — no DUT interaction. Enumerate the 16-bit F_ParCRC space and show iParCRC is deterministically recomputable from the public GSD layout.

Level 1 — Abstract Test Scenario

| Traceability | RQ-002 → CR 3.4 → Asset C |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-3 |
| Objective | Establish analytically that F_ParCRC (16-bit) is exhaustively searchable and iParCRC (32-bit) is recomputable — both error-detection, not authentication. |
| Pass / Fail | Analytical finding: both values forgeable (no cryptographic resistance). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Desk analysis. |
| --- | --- |
| Tools | GSD documentation (SRIO-2998), short enumeration script. |

Procedure / Commands:

python3 -c "print('F_ParCRC 16-bit space =', 2**16, 'values — exhaustively enumerable')"

# iParCRC = 32-bit CRC over PUBLIC parameter layout via the certified CRC-Tool algorithm;

# neither depends on a secret → error-detection, not authentication.

Expected / Evidence: Written keyspace argument + enumeration output. [PLACEHOLDER: Result]

TC-RQ002-05 — Physical tamper-resistance of the address/mode interface

STATUS: Not Feasible (Environment/Device Limitation)

Why: Requires defeating the moulded seal on a spare unit and cold-restart verification. No spare / intact production seal. Documented basis: rotary switch read only at init (SRIO-8803, SRIO-10812); seals are passive (SRIO-11000/11001).

Level 1 — Abstract Test Scenario

| Traceability | RQ-002 → EDR 3.11 → Asset G |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-4 |
| Objective | (a) confirm runtime change is ignored (temporal protection); (b) determine if the stored/next-boot address is corruptible with the seal as the only barrier. |
| Pass / Fail | Gap-characterisation: runtime change ignored (credit); post-restart altered value applies (corruptible); only passive seal resists (partial EDR 3.11). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Spare SRIO required (not available). |
| --- | --- |
| Tools | Multimeter, camera, TIA (documented). |

Procedure / Commands:

# NOT EXECUTED. Documented basis: SRIO-8803 / SRIO-10812 (switch read only at init).

Expected / Evidence: Documented resistance-only physical protection. No lab result. [ANALYTICAL FINDING]

5. RQ-003 — Collect evidence of any legitimate/illegitimate intervention in connection-relevant hardware

IEC 62443-4-2 CR mapping: CR 2.8-2.13, CR 3.9, EDR 3.11, CR 6.1

TC-RQ003-01 — Baseline: what evidence the device actually collects

STATUS: Feasible

Why: Direct HTTP read from Kali + I&M read via TIA. No mirror, no spare.

Level 1 — Abstract Test Scenario

| Traceability | RQ-003 → CR 2.8/CR 6.1 → Asset I |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-1 |
| Objective | Enumerate readable evidence (error log, REVISION_COUNTER, I&M4) and inspect for actor identity + absolute timestamp. |
| Pass / Fail | PASS if log/records readable and change reflected; documented shortfall if no record carries actor identity or absolute timestamp. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali (IoT read) + laptop TIA (I&M). |
| --- | --- |
| Tools | curl, TIA Online & Diagnose. |

Procedure / Commands:

curl -s http://$SRIO/devicestatus/errorlog/loglist  | tee cm_rq003-01_errorlog.json

# In TIA: read I&M0 REVISION_COUNTER and I&M4 signature (before/after one param change).

Expected / Evidence: Log + records readable (accessibility PASS); REVISION_COUNTER increments; I&M4 changes; shortfall: no actor identity / no absolute timestamp. Evidence: json + TIA screenshots. [PLACEHOLDER: Result]

TC-RQ003-02 — Authorized intervention evidence under normal use

STATUS: Feasible

Why: Authorized F-address/iPar change via TIA + reboot, then re-read log — all within your lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-003 → CR 2.8/CR 2.11 → Asset I, G |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-2 |
| Objective | Determine whether an authorized change + restart generates retained, time-referenceable evidence. |
| Pass / Fail | PASS if a retained, time-referenceable record exists after restart; expected shortfall: detail cleared on cold start, no absolute timestamp. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + TIA. |
| --- | --- |
| Tools | curl, TIA. |

Procedure / Commands:

curl -s http://$SRIO/devicestatus/errorlog/loglist > cm_rq003-02_before.json

# Perform authorized rotary/F-address or iPar change (TIA) + reboot, then:

curl -s http://$SRIO/devicestatus/errorlog/loglist > cm_rq003-02_after.json

Expected / Evidence: Post-reboot detail cleared (cold start), no absolute timestamp; only counter delta persists. Evidence: before/after json + TIA I&M. [PLACEHOLDER: Result]

TC-RQ003-03 — Illegitimate physical intervention & anti-forensic clearing

STATUS: Not Feasible (Environment/Device Limitation)

Why: Requires defeating the seal on a spare unit. No spare / intact seal. Retained as FAIL vs RQ-003 (no tamper switch, EDR 3.11 detection limb unmet).

Level 1 — Abstract Test Scenario

| Traceability | RQ-003 → EDR 3.11/CR 2.8/CR 3.9 → Asset G, I |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-4 |
| Objective | Show a seal-preserving intervention leaves no tamper record and a cold start clears incidental log content. |
| Pass / Fail | Gap-characterisation → FAIL vs RQ-003. A persistent attributed timestamped tamper record would indicate an undocumented capability. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Spare SRIO required. |
| --- | --- |
| Tools | Multimeter, camera, curl, TIA (documented). |

Procedure / Commands:

# NOT EXECUTED. Requires spare unit + seal defeat. Documented gap only.

Expected / Evidence: Documented missing-detection + unprotected-evidence gap. No lab result. [ANALYTICAL FINDING]

TC-RQ003-04 — Time-attribution failure of collected evidence

STATUS: Feasible

Why: Two events across a cold start, reading /systemtime/systick via curl — fully in-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-003 → CR 2.11/CR 3.9 → Asset I |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-3 |
| Objective | Show the device exposes only uptime (resets on cold start) with no synchronized RTC, so interventions across a power cycle cannot be dated/ordered. |
| Pass / Fail | PASS if entries carry a retained absolute timestamp; expected FAIL: time base resets, no absolute timestamp. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + power cycle. |
| --- | --- |
| Tools | curl. |

Procedure / Commands:

curl -s http://$SRIO/systemtime/systick > cm_rq003-04_systick_1.txt

# generate a loggable event; cold-restart the device; then:

curl -s http://$SRIO/systemtime/systick > cm_rq003-04_systick_2.txt

Expected / Evidence: systick resets to ~0 on cold start; no absolute timestamp on any entry → not time-referenceable (CR 2.11 unmet). Evidence: two systick files + log excerpts. [PLACEHOLDER: Result]

TC-RQ003-05 — Excluded (development-only) interface produces no evidence

STATUS: Feasible

Why: A single curl POST to the production-gated FIT service; expect rejection on a production unit. Documented basis: FIT is WO/(P) production-only; test interface fused after development (SRIO-8331).

Level 1 — Abstract Test Scenario

| Traceability | RQ-003 → EDR 2.13 → Asset G, I |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-3 |
| Objective | Confirm the FIT function is rejected on a production unit and produces no log entry. |
| Pass / Fail | PASS (exclusion valid) if rejected; documented gap: excluded interface produces no evidence. Silicon-fuse claim out of scope (no invasive PA). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali → production-configured SRIO. |
| --- | --- |
| Tools | curl. |

Procedure / Commands:

curl -s -X POST http://$SRIO/fit/setfit -d '{"type":1}' -w "%{http_code}\n" | tee cm_rq003-05_fit.txt

curl -s http://$SRIO/devicestatus/errorlog/loglist   # check for absence of an entry

Expected / Evidence: FIT rejected on production unit; no log entry for the attempt. Evidence: fit.txt + error-log excerpt. [PLACEHOLDER: Result]

6. RQ-004 — Identify software and data critical for EHSR compliance

IEC 62443-4-2 CR mapping: CR 7.8, CR 4.1

TC-RQ004-01 — Identification-surface inventory (SW + data)

STATUS: Feasible

Why: curl enumeration of /deviceinfo/* + TIA I&M read. Fully in-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-004 → CR 7.8 → Asset D (SW) + Asset B (data via I&M4) |
| --- | --- |
| Target Threat | Information Disclosure |
| SVV Category | SVV-1 |
| Objective | Enumerate the full identification set; confirm SW versions + data signature; diff vs expected EHSR-critical set. |
| Pass / Fail | PASS (partial baseline) if top-level SW identification + I&M4 signature present; shortfall if no SBOM of third-party components and no explicit safety-critical-data classification. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + TIA. |
| --- | --- |
| Tools | curl, TIA, OWASP FSTM Stage 1. |

Procedure / Commands:

for p in deviceinfo/hwversion deviceinfo/hwrevision deviceinfo/swrevision \

deviceinfo/swinfo/cpubootloaderversion deviceinfo/swinfo/scpuversion \

deviceinfo/swinfo/scpubootloaderversion firmware/version fieldbussetup/fieldbusfirmware; do

echo "== $p =="; curl -s http://$SRIO/$p; echo; done | tee cm_rq004-01_inventory.txt

Expected / Evidence: Top-level SW identification + I&M4 present; no third-party SBOM / no EHSR-data classification. Evidence: inventory.txt, TIA I&M, diff table. [PLACEHOLDER: Result]

TC-RQ004-02 — SBOM completeness & third-party CVE correlation

STATUS: Feasible

Why: Desk analysis using TC-RQ004-01 output + device docs. Note (per your comment): SR 7.8 wants an inventory of IDs/capability/revision + a config-management process, not a cryptographic SBOM — so this is reported as an SDL/CRA (SM-9/10) shortfall, not a device-conformance FAIL.

Level 1 — Abstract Test Scenario

| Traceability | RQ-004 → CR 7.8 / 62443-4-1 SM-9–SM-10 → Asset D |
| --- | --- |
| Target Threat | Information Disclosure |
| SVV Category | SVV-3 |
| Objective | Show the inventory is product-level only; third-party components (µC/OS-II, SIL3 PROFIsafe stack, STM32F7 STL, netX90 PROFINET) cannot be enumerated or CVE-correlated. |
| Pass / Fail | Expected: no component-level SBOM → SDL/CRA shortfall (reported separately from the CR 7.8 conformance verdict). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Desk analysis on Kali/laptop. |
| --- | --- |
| Tools | TC-RQ004-01 output, NVD, documentation. |

Procedure / Commands:

# From inventory.txt list exposed version strings; from docs list constituents:

#   µC/OS-II (SRIO-1260), SIL3 PROFIsafe stack (SRIO-1929), STM32F7 STL (SRIO-10766),

#   netX90 PROFINET (SRIO-1768). Attempt to derive each version; correlate derivable vs NVD.

Expected / Evidence: Third-party components not individually exposed; no machine-readable SBOM → SM-9/10 / TR-03183-2 shortfall. Evidence: component-vs-exposed table. [PLACEHOLDER: Result]

TC-RQ004-03 — Plaintext exposure of identification data

STATUS: Feasible

Why: Kali makes the HTTP request itself, so a local tcpdump on the Kali interface captures the cleartext — NO mirror needed. sslscan confirms no TLS.

Level 1 — Abstract Test Scenario

| Traceability | RQ-004 → CR 4.1 (observation) → Asset D |
| --- | --- |
| Target Threat | Information Disclosure |
| SVV Category | SVV-3 |
| Objective | Confirm identification data is served without transport encryption and recoverable in cleartext. |
| Pass / Fail | Observation only (not a safety-security FAIL under EN 50742 FR4=none). Expected: cleartext recoverable, no TLS. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali self-capture on its own iface + a curl read. |
| --- | --- |
| Tools | tshark/tcpdump, sslscan (CM-005). |

Procedure / Commands:

sudo tshark -i "$IFACE" -f "host $SRIO and tcp port 80" -w cm_rq004-03_http.pcap &

curl -s http://$SRIO/deviceinfo/swrevision >/dev/null; sleep 1; sudo pkill tshark

sslscan --show-certificate $SRIO:443 > cm_rq004-03_sslscan.txt 2>&1  # expect: refused/no TLS

Expected / Evidence: Cleartext identification recoverable; no encrypted transport. Evidence: pcap (Follow HTTP stream), sslscan.txt. [PLACEHOLDER: Result]

TC-RQ004-04 — Data-identification signature uniqueness

STATUS: Feasible

Why: Two configs loaded via TIA; I&M4 read each time. In-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-004 → CR 7.8 → Asset B |
| --- | --- |
| Target Threat | N/A (functional identification) |
| SVV Category | SVV-1 |
| Objective | Confirm distinct configurations yield distinct I&M4 signatures (data is identified via fingerprint). |
| Pass / Fail | PASS if distinct configs → distinct signatures; documented limitation: signature identifies that a config is loaded, not which elements are EHSR-critical. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali/laptop + TIA; config A/B projects. |
| --- | --- |
| Tools | TIA (I&M4 read). |

Procedure / Commands:

# Load config A (e.g. filter 10 ms); read I&M4 in TIA.

# Load config B (e.g. filter 5 ms + symmetry change); read I&M4; confirm signatures differ.

Expected / Evidence: Distinct configs → distinct I&M4 signatures. Documented limitation as above. Evidence: two TIA I&M4 screenshots + config notes. [PLACEHOLDER: Result]

7. RQ-005 — Protect identified safety-critical software and data against corruption

IEC 62443-4-2 CR mapping: CR 3.3, CR 3.4, EDR 3.10, 3.12, 3.13, 3.14

TC-RQ005-01 — Parameter integrity-value rejection (⇄ TC-RQ002-01)

STATUS: Feasible

Why: Re-uses the TIA-only TC-RQ002-01 procedure under RQ-005 traceability.

Level 1 — Abstract Test Scenario

| Traceability | RQ-005 → CR 3.4 → Asset B |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-1 |
| Objective | Verify a mismatched parameter integrity value is rejected (device stays safe). |
| Pass / Fail | As TC-RQ002-01 — PASS on rejection of the mismatched configuration. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | As TC-RQ002-01. |
| --- | --- |
| Tools | TIA. |

Procedure / Commands:

# Execute TC-RQ002-01 procedure; record under RQ-005. Reuse cm_rq002-01_* evidence.

Expected / Evidence: PASS on rejection. Evidence: reuse cm_rq002-01_*. [PLACEHOLDER: Result]

TC-RQ005-02 — Valid-integrity forgery over modified parameters

STATUS: Not Feasible (Environment/Device Limitation)

Why: Needs inline substitution or a spare unit to safely present a forged parameter record. No bridge/spare. Retained as gap-characterisation (integrity values ≠ authentication).

Level 1 — Abstract Test Scenario

| Traceability | RQ-005 → CR 3.4 → Asset B |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-4 |
| Objective | Show a config with modified parameters + recomputed CRCs is accepted. |
| Pass / Fail | Gap-characterisation. Expected acceptance; rejection would indicate an undocumented crypto control. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Spare unit / inline bridge required. |
| --- | --- |
| Tools | Scapy/TIA + CRC routine (documented). |

Procedure / Commands:

# NOT EXECUTED. Method: modify a safety iParameter, recompute F_iPar_CRC/F_Par_CRC,

# present forged config; observe accept vs configuration error.

Expected / Evidence: Documented residual risk. No lab result. [ANALYTICAL FINDING]

TC-RQ005-03 — Unsigned / modified firmware installation

STATUS: Adapted/Modified

Why: This is the DECISIVE gap (EDR 3.10/3.14 — update is an unsigned BLOB, SRIO-2953). It requires a SPARE unit (installing an intentionally corrupted image on the live DUT is unsafe). If a spare is obtained → Feasible & highest-value; if not → primary evidence shifts to the offline TC-RQ005-04.

Level 1 — Abstract Test Scenario

| Traceability | RQ-005 → EDR 3.10/3.14 (3.12/3.13) → Asset D, F |
| --- | --- |
| Target Threat | Tampering + Spoofing |
| SVV Category | SVV-4 |
| Objective | Show a modified/unsigned update package is accepted for installation (no signature/integrity check before install). |
| Pass / Fail | PASS if rejected by a signature/integrity check; expected FAIL: accepted → EDR 3.10/3.14 unmet. Spare unit only; re-flash validated image after. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | SPARE SRIO (rotary=999 update state). Keep validated container + hash for recovery. |
| --- | --- |
| Tools | hex editor (xxd/bless), IoT-Core upload, sha256sum. |

Procedure / Commands:

cp UpdateContainer.bin recovery/UpdateContainer.bin

sha256sum recovery/UpdateContainer.bin > recovery/hash.txt

cp UpdateContainer.bin mod.bin

# flip one byte to create a modified/unsigned image:

printf '\xFF' | dd of=mod.bin bs=1 seek=1024 count=1 conv=notrunc

# set rotary=999, power-cycle, upload mod.bin via IoT-Core /firmware, invoke /firmware/install

Expected / Evidence: Modified container accepted for installation, no signature/integrity check (EDR 3.10/3.14 unmet). Recovery: re-upload validated container, power-cycle, confirm hash. Evidence: upload pcap, IoT response, before/after /firmware/version. [PLACEHOLDER: Result]

TC-RQ005-04 — Update-package static analysis (signing / boot-chain artefacts)

STATUS: Feasible

Why: Fully offline on Kali — no DUT. Elevated to PRIMARY evidence for the firmware-authenticity gap if no spare unit exists for TC-RQ005-03.

Level 1 — Abstract Test Scenario

| Traceability | RQ-005 → EDR 3.12/3.14 → Asset D |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-3 |
| Objective | Characterise absence of signature/secure-boot artefacts by static analysis of the update container. |
| Pass / Fail | Expected: no signature/trust-anchor artefacts → supports the firmware-authenticity gap. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Offline on Kali/laptop (no DUT). |
| --- | --- |
| Tools | binwalk, xxd, strings, entropy. |

Procedure / Commands:

binwalk UpdateContainer.bin        > cm_rq005-04_binwalk.txt

binwalk -E UpdateContainer.bin     > cm_rq005-04_entropy.txt

strings -n 8 UpdateContainer.bin   > cm_rq005-04_strings.txt

# look for signature blocks / certificates / trust-anchor references — expect none

Expected / Evidence: No signature/trust-anchor artefacts; entropy consistent with unsigned image. Evidence: cm_rq005-04_*. [PLACEHOLDER: Result]

TC-RQ005-05 — Analytical: safety self-tests ≠ authenticity

STATUS: Feasible

Why: Desk analysis, no DUT. Explains why 1oo2 cross-check + STM32F7 STL cannot detect a coherent identical tamper on both channels.

Level 1 — Abstract Test Scenario

| Traceability | RQ-005 → CR 3.3 → Asset A, D |
| --- | --- |
| Target Threat | N/A (architectural analysis) |
| SVV Category | SVV-1 |
| Objective | Argue that random-fault detection (safety) and coherent-tamper detection (security) are disjoint; only signed-image/secure-boot (EDR 3.14) catches the latter. |
| Pass / Fail | Analytical finding: integrity-of-execution (safety) ≠ authenticity (security). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Desk analysis. |
| --- | --- |
| Tools | Documentation (dual-SCPU 1oo2, STL), IEC 61508 vs 62443. |

Procedure / Commands:

# Written argument: fault model (random, independent, channel divergence) vs

# attack model (coherent identical modification of both channels). Reference TC-RQ005-03/04.

Expected / Evidence: Written argument. [PLACEHOLDER: Result]

8. RQ-006 — Identify installed software necessary for safe operation

IEC 62443-4-2 CR mapping: CR 7.8, CR 4.1

TC-RQ006-01 — Installed-software inventory enumeration

STATUS: Feasible

Why: curl enumeration + TIA I&M. In-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-006 → CR 7.8 → Asset D |
| --- | --- |
| Target Threat | Information Disclosure |
| SVV Category | SVV-1 |
| Objective | Enumerate host FW, both bootloaders, SCPU FW and the safety-protocol stack; map each to the safety function. |
| Pass / Fail | PASS (partial baseline) if all retrievable and consistent; shortfall where any safety-relevant component is not individually identified. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + TIA. |
| --- | --- |
| Tools | curl, TIA. |

Procedure / Commands:

for p in deviceinfo/swinfo/cpubootloaderversion deviceinfo/swinfo/scpuversion \

deviceinfo/swinfo/scpubootloaderversion deviceinfo/swrevision firmware/version \

fieldbussetup/fieldbusfirmware; do echo "== $p =="; curl -s http://$SRIO/$p; echo; done \

| tee cm_rq006-01_installed.txt

Expected / Evidence: Host FW, both bootloaders, SCPU FW, stack version retrievable + consistent. Evidence: installed.txt, TIA I&M, mapping table. [PLACEHOLDER: Result]

TC-RQ006-02 — SBOM completeness & vulnerability correlation (⇄ TC-RQ004-02)

STATUS: Feasible

Why: Desk analysis; reuse TC-RQ004-02 under RQ-006 traceability.

Level 1 — Abstract Test Scenario

| Traceability | RQ-006 → CR 7.8 / SM-9–SM-10 → Asset D |
| --- | --- |
| Target Threat | Information Disclosure |
| SVV Category | SVV-3 |
| Objective | Show no component-level SBOM exists; versions cannot be enumerated/CVE-correlated. |
| Pass / Fail | As TC-RQ004-02 — no component-level SBOM (SDL/CRA shortfall). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Desk analysis. |
| --- | --- |
| Tools | As TC-RQ004-02. |

Procedure / Commands:

# Execute per TC-RQ004-02; record under RQ-006. Do not re-derive.

Expected / Evidence: No component-level SBOM. Evidence: reuse TC-RQ004-02 artefacts. [PLACEHOLDER: Result]

TC-RQ006-03 — Plaintext inventory exposure (defence-in-depth observation)

STATUS: Feasible

Why: Kali self-capture of its own inventory read + sslscan — no mirror needed.

Level 1 — Abstract Test Scenario

| Traceability | RQ-006 → CR 4.1 (observation) → Asset D |
| --- | --- |
| Target Threat | Information Disclosure |
| SVV Category | SVV-3 |
| Objective | Confirm inventory read is served without transport encryption. |
| Pass / Fail | Observation only (not scored). Expected: cleartext, no TLS. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali self-capture. |
| --- | --- |
| Tools | tshark, sslscan. |

Procedure / Commands:

sudo tshark -i "$IFACE" -f "host $SRIO and tcp port 80" -w cm_rq006-03_http.pcap &

curl -s http://$SRIO/deviceinfo/swrevision >/dev/null; sleep 1; sudo pkill tshark

sslscan --show-certificate $SRIO:443 > cm_rq006-03_sslscan.txt 2>&1

Expected / Evidence: Cleartext inventory, no TLS. Evidence: cm_rq006-03_*. [PLACEHOLDER: Result]

9. RQ-007 — Provide identification of safety-relevant software at all times, easily accessible

IEC 62443-4-2 CR mapping: CR 6.1, CR 7.1, CR 7.2, CR 7.7, CR 7.8

TC-RQ007-01 — Identification availability & accessibility baseline

STATUS: Feasible

Why: Browser to IoT-Core Visualizer + curl — in-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-007 → CR 7.8 / CR 6.1 → Asset H, D |
| --- | --- |
| Target Threat | N/A (accessibility) |
| SVV Category | SVV-1 |
| Objective | Confirm safety-relevant identification is retrievable and human-readable via the browser visualizer. |
| Pass / Fail | PASS if interface loads and all identifiers human-readable; FAIL if unreadable/inaccessible in normal operation. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Laptop browser + Kali curl. |
| --- | --- |
| Tools | Browser (Visualizer), curl. |

Procedure / Commands:

# Browser (laptop): http://192.168.0.2/  → device info (human-readable versions)

curl -s http://$SRIO/deviceinfo/swrevision | tee cm_rq007-01_swrev.txt

Expected / Evidence: Visualizer loads without special tooling; identifiers human-readable. Evidence: screenshot, swrev.txt. [PLACEHOLDER: Result]

TC-RQ007-02 — Identification availability across operating states

STATUS: Adapted/Modified

Why: Init/power-cycle and FatalError windows are reachable in-lab; the SCPU-flash window needs the Update state (ideally a spare unit). Poll continuously and annotate outages.

Level 1 — Abstract Test Scenario

| Traceability | RQ-007 → CR 7.8 / FR 7 → Asset H, D |
| --- | --- |
| Target Threat | N/A (inherent unavailability) |
| SVV Category | SVV-2 |
| Objective | Show identification is not available “at all times” by polling through Init, Update (install restart) and FatalError. |
| Pass / Fail | Gap-characterisation → FAIL vs “at all times”. Continuous availability would refute the gap. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali poll + TIA state control (+ spare for Update/flash window). |
| --- | --- |
| Tools | timestamped curl loop, TIA. |

Procedure / Commands:

while true; do echo "$(date +%H:%M:%S.%3N) \

$(curl -s -o /dev/null -w '%{http_code}' --max-time 1 http://$SRIO/deviceinfo/swrevision) \

$(curl -s -o /dev/null -w '%{http_code}' --max-time 1 http://$SRIO/deviceinfo/swinfo/scpuversion)";

sleep 0.5; done | tee cm_rq007-02_poll.log

# During the loop: power-cycle (Init), enter Update+install restart, force FatalError.

Expected / Evidence: Measurable outage windows during Init, install-restart, FatalError; SCPU identifier absent during flash. Evidence: annotated poll.log. [PLACEHOLDER: Result]

TC-RQ007-03 — Connection-hold self-DoS on the identification interface

STATUS: Feasible

Why: Directly exploits the documented 2-connection cap (SRIO-7909) from Kali — no special access.

Level 1 — Abstract Test Scenario

| Traceability | RQ-007 → CR 7.1 / CR 7.2 → Asset H |
| --- | --- |
| Target Threat | Denial of Service (self-inflicted) |
| SVV Category | SVV-4 |
| Objective | Show that holding the 2 permitted connections blocks a 3rd legitimate identification read. |
| Pass / Fail | PASS if a 3rd read still succeeds while 2 held; expected FAIL: 3rd blocked, restored only after release. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali. |
| --- | --- |
| Tools | bash /dev/tcp holders, curl. |

Procedure / Commands:

for i in 1 2; do (exec 3<>/dev/tcp/$SRIO/80; printf 'GET /deviceinfo HTTP/1.1\r\nHost: x\r\n\r\n' >&3; sleep 600) & done

time curl -s --max-time 5 http://$SRIO/deviceinfo/swrevision   # expect timeout/refusal

kill %1; time curl -s --max-time 5 http://$SRIO/deviceinfo/swrevision  # restored after release

Expected / Evidence: 3rd read blocked/timeout while 2 held; restored after release. Evidence: hold log, curl timing before/after. [PLACEHOLDER: Result]

TC-RQ007-04 — Availability inversion under gateway/internal-comms interruption

STATUS: Not Feasible (Environment/Device Limitation)

Why: Requires injecting a COM/SysCom fault (FIT or internal path) while the safety function persists — needs dev/FIT access (production-gated) or a spare. Retained as analytical no-redundancy gap.

Level 1 — Abstract Test Scenario

| Traceability | RQ-007 → CR 7.2 → Asset H, F |
| --- | --- |
| Target Threat | N/A (reliability/redundancy) |
| SVV Category | SVV-3 |
| Objective | Show a COM/SysCom fault removes identification availability while the safety function persists. |
| Pass / Fail | Gap-characterisation. Expected: identification unavailable while safety persists (availability inversion). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | FIT/spare required. |
| --- | --- |
| Tools | curl poll, TIA, DO read-back (documented). |

Procedure / Commands:

# NOT EXECUTED. Requires COM-side fault injection (FIT, production-gated) or spare unit.

Expected / Evidence: Documented availability-inversion gap. No lab result. [ANALYTICAL FINDING]

10. RQ-008 — Collect evidence of any intervention in installed software

IEC 62443-4-2 CR mapping: CR 2.8-2.13, CR 3.9, CR 6.1

TC-RQ008-01 — Software-event evidence baseline

STATUS: Feasible

Why: curl + TIA read at baseline and after a benign software event. In-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-008 → CR 2.8 / CR 6.1 → Asset I, D |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-1 |
| Objective | Establish which software-intervention evidence is produced/readable and inspect for identity + absolute timestamp. |
| Pass / Fail | PASS if log + SW records readable and induced event captured; shortfall if no actor identity/absolute timestamp. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + TIA. |
| --- | --- |
| Tools | curl, TIA. |

Procedure / Commands:

curl -s http://$SRIO/devicestatus/errorlog/loglist > cm_rq008-01_before.json

# trigger a benign self-detected software event, then:

curl -s http://$SRIO/devicestatus/errorlog/loglist > cm_rq008-01_after.json

Expected / Evidence: Event captured; no actor identity, no absolute timestamp. Evidence: before/after json. [PLACEHOLDER: Result]

TC-RQ008-02 — Self-erasing firmware install

STATUS: Adapted/Modified

Why: A real FW update forces a cold start that clears the log; installing on the live DUT is unsafe → requires a spare unit. With a spare it is Feasible.

Level 1 — Abstract Test Scenario

| Traceability | RQ-008 → CR 3.9 / CR 2.11 → Asset I, D |
| --- | --- |
| Target Threat | Repudiation (+Tampering) |
| SVV Category | SVV-2 |
| Objective | Show the install restart clears the audit log and resets uptime — the install erases its own evidence. |
| Pass / Fail | Expected FAIL vs RQ-008: post-install log cleared, uptime reset. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | SPARE SRIO; recovery container ready. |
| --- | --- |
| Tools | curl, IoT-Core upload, TIA. |

Procedure / Commands:

curl -s http://$SRIO/devicestatus/errorlog/loglist > cm_rq008-02_before.json

curl -s http://$SRIO/systemtime/systick > cm_rq008-02_systick_before.txt

# rotary=999, upload legitimate container, /firmware/install (reboot), then re-read:

curl -s http://$SRIO/devicestatus/errorlog/loglist > cm_rq008-02_after.json

Expected / Evidence: Post-install log cleared, uptime reset → install erased its own evidence. Evidence: before/after json + systick. [PLACEHOLDER: Result]

TC-RQ008-03 — Legitimate vs illegitimate: evidence cannot classify

STATUS: Adapted/Modified

Why: Depends on TC-RQ005-03 (unsigned install) → requires a spare unit. With a spare, compare evidence footprints.

Level 1 — Abstract Test Scenario

| Traceability | RQ-008 → CR 2.8 → Asset D, I, F |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-4 |
| Objective | Show a modified/unsigned install produces the same audit/version footprint as a genuine update. |
| Pass / Fail | Expected FAIL: indistinguishable footprint (same completion semantics + version change, no authenticity flag). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | SPARE SRIO. |
| --- | --- |
| Tools | curl, TIA. |

Procedure / Commands:

# Record legitimate-update delta (from TC-RQ008-02), then install the modified image

# (TC-RQ005-03); compare log entries + version change for any 'illegitimate' marker.

Expected / Evidence: Indistinguishable footprint; no authenticity flag. Evidence: side-by-side legit vs illegit json. [PLACEHOLDER: Result]

TC-RQ008-04 — Time-attribution across the install cold start (⇄ TC-RQ003-04)

STATUS: Adapted/Modified

Why: Reuses the systick method around a real FW install → needs a spare unit for the install; the systick method itself is trivially feasible.

Level 1 — Abstract Test Scenario

| Traceability | RQ-008 → CR 2.11 / CR 3.9 → Asset I |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-3 |
| Objective | Show the install cold start resets uptime and no absolute timestamp is attached to the install event. |
| Pass / Fail | Expected: uptime resets, no absolute timestamp (CR 2.11 unmet). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | SPARE SRIO + Kali poll. |
| --- | --- |
| Tools | curl, TIA. |

Procedure / Commands:

curl -s http://$SRIO/systemtime/systick > cm_rq008-04_before.txt

# perform FW install (spare), then:

curl -s http://$SRIO/systemtime/systick > cm_rq008-04_after.txt

Expected / Evidence: Uptime resets, no absolute timestamp on the install event. Evidence: systick before/after. [PLACEHOLDER: Result]

TC-RQ008-05 — Excluded (development-only) interface produces no evidence (⇄ TC-RQ003-05)

STATUS: Feasible

Why: Single curl POST to FIT on a production unit; reuse of TC-RQ003-05.

Level 1 — Abstract Test Scenario

| Traceability | RQ-008 → EDR 2.13 → Asset D, I |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-3 |
| Objective | Confirm the production-gated FIT path is rejected and produces no log entry. |
| Pass / Fail | As TC-RQ003-05 — excluded interface produces no evidence. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali → production unit. |
| --- | --- |
| Tools | curl. |

Procedure / Commands:

curl -s -X POST http://$SRIO/fit/setfit -d '{"type":1}' -w "%{http_code}\n"

curl -s http://$SRIO/devicestatus/errorlog/loglist   # confirm no entry

Expected / Evidence: FIT rejected; no log entry. Evidence: reuse cm_rq003-05_*. [PLACEHOLDER: Result]

11. RQ-009 — Collect evidence of any modification of installed software or its configuration

IEC 62443-4-2 CR mapping: CR 2.8-2.12, CR 3.4, CR 3.9, CR 6.1

TC-RQ009-01 — Config-modification evidence-surface baseline

STATUS: Feasible

Why: curl error log + TIA I&M read at baseline. In-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-009 → CR 2.8 / CR 3.4 / CR 6.1 → Asset B, I |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-1 |
| Objective | Enumerate readable config-modification evidence (REVISION_COUNTER, I&M4, error log); inspect for identity/timestamp. |
| Pass / Fail | PASS (partial baseline) if indicators readable; shortfall where no field carries actor identity/absolute timestamp. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + TIA. |
| --- | --- |
| Tools | curl, TIA. |

Procedure / Commands:

curl -s http://$SRIO/devicestatus/errorlog/loglist > cm_rq009-01_log.json

# TIA: read I&M0 REVISION_COUNTER + I&M4 signature; record identity/timestamp presence.

Expected / Evidence: Indicators readable; no actor identity or absolute timestamp. Evidence: json + TIA screenshots. [PLACEHOLDER: Result]

TC-RQ009-02 — Config-modification evidence & asymmetric persistence

STATUS: Feasible

Why: Authorized iPar change via TIA + power cycle; re-read counter/signature/log. In-lab. (Note per your comment: verify empirically whether REVISION_COUNTER survives cold start — current hypothesis: cleared, like the log.)

Level 1 — Abstract Test Scenario

| Traceability | RQ-009 → CR 2.8 / CR 2.11 / CR 3.9 → Asset B, I |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-2 |
| Objective | Show a change increments the counter + changes the signature but carries no timestamp/actor; measure counter persistence across cold start vs log clearing. |
| Pass / Fail | Gap-characterisation: counter increments, signature changes, no time/actor; durable/volatile asymmetry reported as MEASURED, not assumed. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + TIA + power cycle. |
| --- | --- |
| Tools | curl, TIA. |

Procedure / Commands:

# Baseline counter/signature/log/uptime (TIA + curl). Perform one authorized iPar change.

# Re-read (counter +1, signature changed?). Cold-start. Re-read: counter retained or cleared?

curl -s http://$SRIO/devicestatus/errorlog/loglist > cm_rq009-02_after_coldstart.json

Expected / Evidence: Counter +1, signature changes, no time/actor; counter persistence recorded empirically; log cleared + uptime reset. Evidence: TIA before/after/after-coldstart. [PLACEHOLDER: Result]

TC-RQ009-03 — Actor attribution is delegated, not device-resident

STATUS: Feasible

Why: Authorized change from a known TIA user; inspect device-side fields vs the TIA/PLC project. In-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-009 → CR 2.12 → Asset B, F |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-3 |
| Objective | Show the device holds no actor identity for a config change; attribution lives in the controller/engineering project (delegated CR 2.12). |
| Pass / Fail | Expected: no device-side actor identity → CR 2.12 delegated to the controller/engineering layer. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + TIA (known user). |
| --- | --- |
| Tools | curl, TIA. |

Procedure / Commands:

# Perform authorized config change from a known TIA user.

# Dump every device-side evidence field (curl I&M/log) — confirm no actor attribute;

# inspect the TIA/PLC project for where the identity resides.

Expected / Evidence: No device-side actor identity → CR 2.12 delegated. Evidence: device-side field dump + TIA project user record. [PLACEHOLDER: Result]

TC-RQ009-04 — Post-configuration-phase write gate

STATUS: Adapted/Modified

Why: Attempting an acyclic PROFINET write after param-end from Kali is possible but crafting the acyclic record is non-trivial; needs Scapy PROFINET contrib and ideally the mirror to confirm rejection. Feasible with effort.

Level 1 — Abstract Test Scenario

| Traceability | RQ-009 → CR 3.4 → Asset B |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-3 |
| Objective | Validate the temporal write-gate: an acyclic config write after the configuration phase is rejected (SRIO-7749); check whether the blocked attempt produces evidence. |
| Pass / Fail | PASS (write-gate valid) if post-phase write rejected; observation: blocked attempt produces no evidence entry (expected). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali (active) during OPERATE + mirror capture. |
| --- | --- |
| Tools | Scapy (pnio acyclic), tshark. |

Procedure / Commands:

# After param-end (OPERATE), attempt an acyclic write to a config record index (e.g. iPar):

python3 - <<'PY'

from scapy.all import *

load_contrib('pnio')

# craft acyclic write to record index; send from Kali; observe rejection

PY

curl -s http://$SRIO/devicestatus/errorlog/loglist   # check for any entry

Expected / Evidence: Post-phase write rejected (SRIO-7749); blocked attempt produces no evidence entry. Evidence: pcap + error-log excerpt. [PLACEHOLDER: Result]

TC-RQ009-05 — Analytical: change-counter wrap-around bounds the evidence

STATUS: Feasible

Why: Desk computation — 16-bit counter over the 20-year mission (SRIO-706) with the mandated ≤1-year reboot (SRIO-1812).

Level 1 — Abstract Test Scenario

| Traceability | RQ-009 → CR 2.9 → Asset I |
| --- | --- |
| Target Threat | N/A (lifecycle analysis) |
| SVV Category | SVV-1 |
| Objective | Show the fixed-width counter wraps within the mission lifetime under frequent re-parameterization → bounded evidence. |
| Pass / Fail | Analytical: wrap reachable within mission lifetime → bounded, not unbounded, evidence. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Desk analysis. |
| --- | --- |
| Tools | Documentation (16-bit counter; SRIO-706). |

Procedure / Commands:

python3 -c "print('counter width 16-bit =', 2**16-1, 'max; wrap reachable within 20y mission')"

Expected / Evidence: Written computation showing wrap within mission lifetime. [PLACEHOLDER: Result]

12. RQ-010 — Withstand reasonably foreseeable malicious attempts leading to hazard

IEC 62443-4-2 CR mapping: CR 3.5, 3.6, 3.7, 7.1(1), 7.2

TC-RQ010-01 — Baseline resilience under rated conformant load

STATUS: Adapted/Modified

Why: Reproducing certified Net Load Class III conformant load exactly needs a PROFINET load generator you may not have; hping3 shaped to a conformant rate is an approximation. Monitor PLC.

Level 1 — Abstract Test Scenario

| Traceability | RQ-010 → CR 7.1 / CR 7.2 → Asset A, H |
| --- | --- |
| Target Threat | N/A (functional baseline) |
| SVV Category | SVV-1 |
| Objective | Confirm stable safe communication under (approximated) rated conformant load. |
| Pass / Fail | PASS if safe comms + stability maintained under rated load; FAIL if it destabilises within rated load. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali load source + PLC monitor + (opt.) mirror. |
| --- | --- |
| Tools | hping3 (shaped), tshark, DO read-back. |

Procedure / Commands:

sudo hping3 -i u1000 -d 100 $SRIO   # ~1 kpps approximation; keep BELOW disruptive rate

# monitor PLC/PROFIsafe qualifier + DO read-back throughout

Expected / Evidence: Safe communication + stability maintained under rated load. Evidence: pcap, DO read-back, PLC diagnostic. [PLACEHOLDER: Result]

TC-RQ010-02 — Network / Layer-2 flood resilience

STATUS: Feasible

Why: hping3 flood from Kali is directly runnable. CAUTION: the USB3-GbE dongle may be the bottleneck, not the DUT — sanity-check with a short capture. Monitor PLC.

Level 1 — Abstract Test Scenario

| Traceability | RQ-010 → CR 7.1(RE1)/7.2/3.6 → Asset H, A, E, F |
| --- | --- |
| Target Threat | Denial of Service |
| SVV Category | SVV-4 |
| Objective | Determine behaviour under high-rate flood: maintain safe comms, passivate safely, or enter an undefined/hazardous state; check continuous forced passivation (availability). |
| Pass / Fail | PASS if safe comms hold to rated limits and beyond it passivates safely (DO de-energised, qualifier bad) with no undefined output; FAIL on any undefined/hazardous output. Continuous passivation recorded as availability finding. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali flood source; PLC monitor; capture on mirror/inline or annotate loss. |
| --- | --- |
| Tools | hping3, Scapy pnio storm, tshark, DO read-back. |

Procedure / Commands:

sudo hping3 --flood --rand-source -d 100 $SRIO   # escalate stepwise; WATCH the PLC

# optional PROFINET-path storm via Scapy pnio broadcast; stop and confirm acknowledged re-integration

Expected / Evidence: Safe comms to rated limits, then safe passivation with no undefined output; continuous passivation = availability finding. Evidence: flood pcap, health log, DO video, PLC diagnostic. [PLACEHOLDER: Result]

TC-RQ010-03 — Protocol fuzzing robustness (integrates CM-010)

STATUS: Feasible

Why: Corrected HTTP-aware boofuzz against IoT-Core with a per-case liveness monitor + throttle (removes the 2-connection ambiguity behind the 0x1000 event). PROFINET/DCP field fuzzing part is Adapted (needs Scapy + ideally inline).

Level 1 — Abstract Test Scenario

| Traceability | RQ-010 → CR 3.5 / 3.7 / 3.6 → Asset H, A, E |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-4 |
| Objective | Determine whether malformed IoT/DCP/PROFIsafe inputs are safely rejected or cause a crash/hang/undefined state. |
| Pass / Fail | PASS if all malformed inputs safely rejected or resolve to passivation, no crash into undefined/hazardous state; FAIL on hang/uncommanded output/unrecoverable non-safe state. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + PLC monitor; (PROFINET part on inline bridge). |
| --- | --- |
| Tools | boofuzz (HTTP), Scapy pnio/pnio-dcp, tshark. |

Procedure / Commands:

# Corrected HTTP fuzzer (single connection, throttled, liveness monitor):

python3 cm010_http_fuzz.py     # see script below; UI at http://127.0.0.1:26000

# decode any device event afterwards:

curl -s http://$SRIO/devicestatus/errorlog/loglist   # map 0x1000 vs manual table

Expected / Evidence: All malformed inputs safely rejected/passivate; no crash. 0x1000 = COM-side diagnosis (input-validation), not a safety FAIL if PROFIsafe stayed up. Evidence: boofuzz-results/, fuzz pcap, health log. [PLACEHOLDER: Result]

TC-RQ010-04 — Application-layer flood & domain-separation confirmation (CM-009)

STATUS: Feasible

Why: Siege against IoT-Core from Kali while monitoring the safety path. -c 2 = compliance, -c 50 = availability. Direct in-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-010 → CR 7.2 / CR 5.1 → Asset F, H |
| --- | --- |
| Target Threat | Denial of Service |
| SVV Category | SVV-3 |
| Objective | Apply an HTTP request flood to the non-safety interface and confirm it does NOT propagate to the safety path (domain separation). |
| Pass / Fail | PASS if safety path unaffected (no induced passivation, no propagation); FAIL if the non-safety flood disturbs the safety function. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + PLC monitor + DO read-back. |
| --- | --- |
| Tools | Siege/ab (CM-009), tshark, DO read-back. |

Procedure / Commands:

siege -c 50 -t 5M http://$SRIO/deviceinfo   # availability run; MONITOR PLC

siege -c 2  -t 5M http://$SRIO/deviceinfo   # compliance run (matches 2-conn cap)

# monitor PROFIsafe qualifier + DO read-back throughout

Expected / Evidence: Safety path unaffected → domain separation holds. Evidence: siege log, safety-path pcap, DO read-back. [PLACEHOLDER: Result]

TC-RQ010-05 — Outcome-verification aggregation harness

STATUS: Adapted/Modified

Why: Aggregates outcomes of TC-RQ010-02/03 + imported TC-RQ001-03/04 and TC-RQ005-03; the firmware-install class depends on a spare unit, so the matrix is completed only for the feasible classes.

Level 1 — Abstract Test Scenario

| Traceability | RQ-010 → CR 3.6 → Asset A, E |
| --- | --- |
| Target Threat | N/A (aggregate outcome) |
| SVV Category | SVV-2 |
| Objective | For every attack class, verify no uncommanded/unsafe output; surface the firmware class that escapes the fail-safe design. |
| Pass / Fail | PASS (per class) if every outcome is safe-continue or safe-passivate; FAIL on any uncommanded actuation. Expected: flood/fuzz/spoof PASS; firmware-install = documented exception. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Independent DO→PLC-DI read-back across imported cases. |
| --- | --- |
| Tools | TIA read-back logic, spreadsheet. |

Procedure / Commands:

# For each executed attack case, sample output state + qualifier; classify

# {safe-continue, safe-passivate, hazardous}; consolidate into cm_rq010-05_matrix.xlsx.

Expected / Evidence: Flood/fuzz/spoof → safe outcome (PASS); firmware-install → documented hazard exception (if spare tested). Evidence: outcome matrix. [PLACEHOLDER: Result]

13. RQ-011 — Prevent hazardous modifications to safety-relevant settings/rules

IEC 62443-4-2 CR mapping: CR 1.1, 1.2, 2.1(2), 3.4, EDR 3.10

TC-RQ011-01 — Read-only enforcement on the non-safety interface (integrates CM-004)

STATUS: Feasible

Why: curl write attempts + http-methods NSE + ZAP (repurposed from CM-004 escalation to read-only proof). In-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-011 → CR 2.1 → Asset B |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-1 |
| Objective | Confirm safety-relevant parameters cannot be written via the information interface (read-only). |
| Pass / Fail | PASS if all safety-parameter write attempts refused; FAIL if any safety parameter is writable via that interface. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali. |
| --- | --- |
| Tools | curl, OWASP ZAP (CM-004), http-methods NSE. |

Procedure / Commands:

nmap -p80 --script http-methods --script-args http-methods.test-all $SRIO -oN cm_methods.txt

for m in POST PUT DELETE; do curl -s -o /dev/null -w "$m %{http_code}\n" \

-X $m http://$SRIO/fieldbussetup/network/ipaddress; done | tee cm_rq011-01_write.txt

# ZAP spider + attempt writes (threads=1-2) — show all rejected

Expected / Evidence: All safety-parameter write attempts refused/read-only. Evidence: write.txt, methods, ZAP report. [PLACEHOLDER: Result]

TC-RQ011-02 — Unauthenticated configuration-protocol factory reset

STATUS: Feasible

Why: DCP identify/set/reset-to-factory from Kali via python pnio_dcp/Scapy — no credentials, no physical access. Monitor PLC (impact = passivation).

Level 1 — Abstract Test Scenario

| Traceability | RQ-011 → CR 1.2 / CR 2.1 → Asset G, F, B |
| --- | --- |
| Target Threat | Spoofing + Elevation of Privilege |
| SVV Category | SVV-4 |
| Objective | Determine whether a peer node can issue a DCP set/reset-to-factory of communication parameters without any credential. |
| Pass / Fail | PASS if the set/reset requires authentication/refused; expected FAIL: reset succeeds unauthenticated. Bound: impact on communication parameters (→ passivation), not silent safety-iPar modification. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali (no credentials) + PLC monitor; (opt.) mirror capture. |
| --- | --- |
| Tools | python pnio_dcp / Scapy, tshark, DO read-back. |

Procedure / Commands:

pip install pnio-dcp

python3 - <<'PY'

import pnio_dcp, os

dcp = pnio_dcp.DCP(os.environ['IFACE'])

devs = dcp.identify_all()

# for d in devs: dcp.set_name_of_station(d.mac, 'attacker'); dcp.reset_to_factory(d.mac)

PY

Expected / Evidence: DCP set/reset succeeds unauthenticated → FAIL vs requirement (bounded to comms params → passivation). Evidence: dcp pcap, script log, PLC diagnostic. [PLACEHOLDER: Result]

TC-RQ011-03 — Fieldbus parameter write without device-side authorization

STATUS: Not Feasible (Environment/Device Limitation)

Why: Requires impersonating the controller and injecting a parameter record during the configuration phase (inline/param-phase injection). No bridge / single live unit. Retained as analytical (authorization delegated to environment).

Level 1 — Abstract Test Scenario

| Traceability | RQ-011 → CR 1.1 / 1.2 / 2.1 → Asset B, F |
| --- | --- |
| Target Threat | Tampering + Spoofing + EoP |
| SVV Category | SVV-4 |
| Objective | Show the device enforces no credential of its own on the fieldbus parameter-write path (only addressing + CRC). |
| Pass / Fail | Expected FAIL: only addressing + integrity values checked, no credential → authorization delegated. Distinct from the CRC forgeability of TC-RQ005-02. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Inline / param-phase injection required. |
| --- | --- |
| Tools | Scapy (pnio acyclic), NetfilterQueue (documented). |

Procedure / Commands:

# NOT EXECUTED. Requires controller impersonation / param-phase injection.

Expected / Evidence: Documented: authorization delegated to the environment (locked cabinet, SRIO-9402). No lab result. [ANALYTICAL FINDING]

TC-RQ011-04 — Firmware-update mode gate

STATUS: Adapted/Modified

Why: Attempting /firmware/install outside vs inside the Update state (rotary=999) is safe on a spare but risky on the live DUT → spare recommended. The rotary/power-cycle gating (SRIO-11534) makes this a clean positive control.

Level 1 — Abstract Test Scenario

| Traceability | RQ-011 → EDR 3.10 / CR 2.1 → Asset G, D |
| --- | --- |
| Target Threat | Elevation of Privilege |
| SVV Category | SVV-2 |
| Objective | Validate the update gate: rejected outside the Update state, accepted only within it. |
| Pass / Fail | PASS if rejected outside and accepted only in the Update state; FAIL if an update can start outside it. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Spare SRIO. |
| --- | --- |
| Tools | curl/IoT-Core upload, rotary switch. |

Procedure / Commands:

# rotary != 999 (not update state):

curl -s -X POST http://$SRIO/firmware/install -w "%{http_code}\n"   # expect reject

# set rotary=999, power-cycle, retry → expect acceptance of a validated container

Expected / Evidence: Update rejected outside the Update state, accepted only in it (SRIO-11534). Evidence: HTTP codes in both states. [PLACEHOLDER: Result]

TC-RQ011-05 — Physical-barrier silent defeat (⇄ TC-RQ003-03 / TC-RQ002-05)

STATUS: Not Feasible (Environment/Device Limitation)

Why: Requires seal defeat on a spare unit (see TC-RQ002-05/003-03). No spare / intact seal.

Level 1 — Abstract Test Scenario

| Traceability | RQ-011 → EDR 3.11 → Asset G, B |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-4 |
| Objective | Confirm the physical address/mode interface can be altered without visible seal damage and applies after restart. |
| Pass / Fail | Expected: barrier can be silently defeated (physical-authorization gap). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Spare SRIO required. |
| --- | --- |
| Tools | As TC-RQ003-03 / TC-RQ002-05 (documented). |

Procedure / Commands:

# NOT EXECUTED. Requires spare unit + seal defeat.

Expected / Evidence: Documented physical-authorization gap. No lab result. [ANALYTICAL FINDING]

14. RQ-012 — Enable a tracing log of intervention data for five years

IEC 62443-4-2 CR mapping: CR 2.8, 2.9, 2.10, 3.9, 6.1

TC-RQ012-01 — Circular-buffer overflow / capacity conflict

STATUS: Feasible

Why: Generate >100 loggable events (repeated passivation/self-detected events) + curl the log after each block. In-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-012 → CR 2.9 / CR 2.10 → Asset I |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-2 |
| Objective | Show the log is a fixed-capacity circular buffer that overwrites oldest entries (≥100, SRIO-10703/10663). |
| Pass / Fail | PASS if all events retained; expected FAIL: buffer caps at fixed size, overwrites oldest-first. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + TIA (event generation). |
| --- | --- |
| Tools | curl, event generator. |

Procedure / Commands:

curl -s http://$SRIO/devicestatus/errorlog/loglist > cm_rq012-01_log_00.json

# generate >100 identifiable events (e.g. repeated port over-current/passivation cycles),

# reading the log after each block:

for i in $(seq 1 5); do curl -s http://$SRIO/devicestatus/errorlog/loglist > cm_rq012-01_log_$i.json; sleep 5; done

Expected / Evidence: Buffer caps at fixed size (≥100), overwrites oldest-first. Evidence: sequential json. [PLACEHOLDER: Result]

TC-RQ012-02 — Cold-start deletion

STATUS: Feasible

Why: Populate log, power-cycle, re-read — in-lab (SRIO-6521).

Level 1 — Abstract Test Scenario

| Traceability | RQ-012 → CR 3.9 / CR 2.9 → Asset I |
| --- | --- |
| Target Threat | Repudiation (+Tampering) |
| SVV Category | SVV-2 |
| Objective | Show a cold start clears the tracing log (data does not persist across a power cycle). |
| Pass / Fail | PASS if entries persist across reset; expected FAIL (cold start): log cleared. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + power cycle. |
| --- | --- |
| Tools | curl. |

Procedure / Commands:

curl -s http://$SRIO/devicestatus/errorlog/loglist > cm_rq012-02_before.json

# cold-start (power cycle), then:

curl -s http://$SRIO/devicestatus/errorlog/loglist > cm_rq012-02_after.json

Expected / Evidence: Log cleared after cold start (SRIO-6521). Evidence: before/after json. [PLACEHOLDER: Result]

TC-RQ012-03 — Mandated-reboot retention impossibility

STATUS: Feasible

Why: Desk analysis + TC-RQ012-02 evidence: mandated ≤1-year reboot (SRIO-1812) is a cold start that clears the log → 5-year on-device retention structurally impossible.

Level 1 — Abstract Test Scenario

| Traceability | RQ-012 → CR 2.9 → Asset I |
| --- | --- |
| Target Threat | N/A (lifecycle analysis) |
| SVV Category | SVV-1 |
| Objective | Show the device’s own mandatory ≤1-year reboot forces ≥annual log erasure within the 5-year window. |
| Pass / Fail | Structural finding: 5-year on-device retention impossible, independent of any attack. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Desk analysis + TC-RQ012-02. |
| --- | --- |
| Tools | Documentation (SRIO-1812). |

Procedure / Commands:

# Written argument: SRIO-1812 (≤1y reboot) + SRIO-6521 (cold start clears log) →

# guaranteed ≥annual erasure within the 5-year window.

Expected / Evidence: Structural finding citing SRIO-1812 + TC-RQ012-02. [PLACEHOLDER: Result]

TC-RQ012-04 — Time-attribution of retained entries (⇄ TC-RQ003-04)

STATUS: Feasible

Why: Reuses the in-lab systick method under RQ-012.

Level 1 — Abstract Test Scenario

| Traceability | RQ-012 → CR 2.11 → Asset I |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-3 |
| Objective | Confirm no retained entry carries an absolute timestamp → surviving entries cannot be bounded to a 5-year window. |
| Pass / Fail | Expected: only relative uptime, no absolute timestamp → retained fragment undatable. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali. |
| --- | --- |
| Tools | curl. |

Procedure / Commands:

curl -s http://$SRIO/systemtime/systick; curl -s http://$SRIO/devicestatus/errorlog/loglist

Expected / Evidence: Only relative uptime, no absolute timestamp. Evidence: systick + log excerpts. [PLACEHOLDER: Result]

15. RQ-013 — Enable a tracing log of uploaded safety-software versions (5 years per upload)

IEC 62443-4-2 CR mapping: CR 2.8-2.11, CR 3.9, CR 6.1

TC-RQ013-01 — Current-version-only confirmation

STATUS: Feasible

Why: curl version fields + TIA I&M; search for history fields. In-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-013 → CR 2.9 → Asset I, D |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-1 |
| Objective | Confirm only the current version is exposed; no historical/previous-version or upload-history field. |
| Pass / Fail | PASS if a queryable per-upload history is exposed; expected FAIL: only current version, no history field. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali + TIA. |
| --- | --- |
| Tools | curl, TIA. |

Procedure / Commands:

curl -s http://$SRIO/firmware/version; echo

curl -s http://$SRIO/deviceinfo/swrevision | tee cm_rq013-01_version.txt

# TIA: read I&M0 SOFTWARE_REVISION / I&M5; search for any previous-version/history field

Expected / Evidence: Only current version reported; no history field. Evidence: version.txt, TIA I&M. [PLACEHOLDER: Result]

TC-RQ013-02 — Sequential-upload version-history test

STATUS: Adapted/Modified

Why: Two successive real FW uploads → requires a spare unit (repeated installs on the live DUT are unsafe). With a spare it is Feasible.

Level 1 — Abstract Test Scenario

| Traceability | RQ-013 → CR 2.8 / 2.9 / 3.9 → Asset I, D |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-2 |
| Objective | Show no device-resident field reports that a previous version was ever installed after a new upload. |
| Pass / Fail | Expected FAIL: only current version reported; no record of prior uploads persists. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | SPARE SRIO; two legitimate containers v_A, v_B. |
| --- | --- |
| Tools | curl/IoT-Core upload, TIA. |

Procedure / Commands:

curl -s http://$SRIO/firmware/version > cm_rq013-02_v0.txt

# upload v_A (spare), install, read; then upload v_B, install, read:

curl -s http://$SRIO/firmware/version > cm_rq013-02_vA.txt

curl -s http://$SRIO/firmware/version > cm_rq013-02_vB.txt

Expected / Evidence: Only current version reported; no prior-upload record. Evidence: version after each upload. [PLACEHOLDER: Result]

TC-RQ013-03 — Per-upload timestamp absence (⇄ TC-RQ003-04)

STATUS: Adapted/Modified

Why: The systick read is trivially feasible; anchoring it around a real upload needs a spare unit for the install.

Level 1 — Abstract Test Scenario

| Traceability | RQ-013 → CR 2.11 / CR 3.9 → Asset I |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-3 |
| Objective | Show a firmware upload carries no absolute per-upload timestamp (no RTC; install-forced cold start resets uptime). |
| Pass / Fail | Expected FAIL: uptime resets, no RTC → per-upload retention clock cannot be started. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | SPARE SRIO. |
| --- | --- |
| Tools | curl. |

Procedure / Commands:

curl -s http://$SRIO/systemtime/systick > cm_rq013-03_before.txt

# perform an upload (install restart), then:

curl -s http://$SRIO/systemtime/systick > cm_rq013-03_after.txt

Expected / Evidence: Uptime resets, no absolute time on the version change. Evidence: systick + version before/after. [PLACEHOLDER: Result]

TC-RQ013-04 — External-record (release-notes) verification

STATUS: Feasible

Why: Desk assessment of ifm Release Notes (SRIO-16242) against the criteria. No DUT.

Level 1 — Abstract Test Scenario

| Traceability | RQ-013 → CR 2.9 (compensating) → Asset I, D |
| --- | --- |
| Target Threat | Repudiation |
| SVV Category | SVV-1 |
| Objective | Assess whether external release docs provide per-serial, timestamped, tamper-evident, 5-year, per-upload records. |
| Pass / Fail | Expected: not device-resident, not per-serial, not tamper-evident, no guaranteed per-upload timestamped retention → cannot cure the on-device gap. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Desk analysis. |
| --- | --- |
| Tools | ifm Release Notes (SRIO-16242). |

Procedure / Commands:

# Assess release documentation against: device-resident? per-serial? tamper-evident?

# guaranteed 5-year per-upload retention? per-upload timestamp?

Expected / Evidence: External records cannot cure the on-device gap for a specific unit. Evidence: criteria table. [PLACEHOLDER: Result]

16. RQ-014 — Restrict access to tracing-log data to a competent authority request only

IEC 62443-4-2 CR mapping: CR 1.1, 1.2, 2.1, 3.9, 4.1, 6.1

TC-RQ014-01 — Unauthenticated tracing-log read

STATUS: Feasible

Why: Single unauthenticated curl from Kali returns the full log. Direct proof of the missing access boundary.

Level 1 — Abstract Test Scenario

| Traceability | RQ-014 → CR 1.1/1.2 / 2.1 / 3.9 → Asset I, H |
| --- | --- |
| Target Threat | Information Disclosure |
| SVV Category | SVV-4 |
| Objective | Show the tracing log is fully readable by an arbitrary unauthenticated node with no identity/authorization/purpose-limitation. |
| Pass / Fail | PASS if the read requires authentication + purpose-limited authorization; expected FAIL: full log returned unauthenticated. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali (no credentials). |
| --- | --- |
| Tools | curl, OWASP ZAP. |

Procedure / Commands:

curl -s http://$SRIO/devicestatus/errorlog/loglist | tee cm_rq014-01_log.json

# no auth challenge, no session, no purpose gate presented at any point

Expected / Evidence: Full log returned to an unauthenticated node → missing access-control gap. Evidence: log.json, ZAP session (no auth). [PLACEHOLDER: Result]

TC-RQ014-02 — Cleartext confidentiality of the tracing log (integrates CM-005)

STATUS: Feasible

Why: Kali self-captures its own log read on its own interface — no mirror needed — and sslscan confirms no TLS.

Level 1 — Abstract Test Scenario

| Traceability | RQ-014 → CR 4.1 / 4.3 (observation) → Asset I, H |
| --- | --- |
| Target Threat | Information Disclosure |
| SVV Category | SVV-3 |
| Objective | Show tracing-log content is transmitted without transport encryption and recoverable in cleartext. |
| Pass / Fail | Supporting observation (not the primary finding). Expected: cleartext recoverable, no TLS. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali self-capture. |
| --- | --- |
| Tools | tshark, sslscan (CM-005). |

Procedure / Commands:

sudo tshark -i "$IFACE" -f "host $SRIO and tcp port 80" -w cm_rq014-02_log.pcap &

curl -s http://$SRIO/devicestatus/errorlog/loglist >/dev/null; sleep 1; sudo pkill tshark

sslscan --show-certificate $SRIO:443 > cm_rq014-02_sslscan.txt 2>&1

Expected / Evidence: Cleartext log recoverable, no TLS. Evidence: pcap (Follow HTTP stream), sslscan.txt. [PLACEHOLDER: Result]

TC-RQ014-03 — Access-control mechanism probe

STATUS: Feasible

Why: ZAP + curl enumeration of the log endpoint for any auth/role/purpose gate. In-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-014 → CR 1.1 / 1.2 / 2.1 → Asset I, H |
| --- | --- |
| Target Threat | Information Disclosure |
| SVV Category | SVV-3 |
| Objective | Enumerate the interface for any authentication mechanism, role model, or purpose-gate on the log endpoint. |
| Pass / Fail | Expected: no auth mechanism, role model or purpose-gate → requirement unachievable (no identity model exists). |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali. |
| --- | --- |
| Tools | OWASP ZAP (CM-004), curl. |

Procedure / Commands:

curl -s -o /dev/null -w '%{http_code}\n' http://$SRIO/devicestatus/errorlog/loglist

# ZAP: enumerate log + adjacent endpoints for any auth/authorization control (threads=1-2)

Expected / Evidence: No authentication mechanism, role model or purpose-gate. Evidence: ZAP report, endpoint enumeration. [PLACEHOLDER: Result]

TC-RQ014-04 — Deletion-control probe

STATUS: Feasible

Why: curl DELETE + enumeration for any controlled-erase command. In-lab.

Level 1 — Abstract Test Scenario

| Traceability | RQ-014 → CR 3.9 → Asset I |
| --- | --- |
| Target Threat | Tampering |
| SVV Category | SVV-3 |
| Objective | Show there is no controlled-deletion mechanism for the tracing log (only the uncontrolled cold-start wipe). |
| Pass / Fail | Expected: no controlled-deletion command (only cold-start wipe) → absence of data-lifecycle control. |

Level 2 — Concrete Execution (Kali @192.168.0.80 → SRIO @192.168.0.2)

| HW / Network | Kali. |
| --- | --- |
| Tools | curl, ZAP. |

Procedure / Commands:

curl -s -X DELETE http://$SRIO/devicestatus/errorlog/loglist -w "%{http_code}\n" | tee cm_rq014-04_delete.txt

# enumerate for any delete/erase command; confirm none exists / not access-controlled

Expected / Evidence: No controlled-deletion command (only cold-start wipe). Evidence: delete.txt (HTTP codes). [PLACEHOLDER: Result]

Appendix A — Corrected boofuzz script (cm010_http_fuzz.py)

HTTP-aware, single-connection, throttled to the 2-connection cap, with a per-case liveness monitor so a real crash is distinguished from the device merely logging 0x1000 or from connection-pool starvation:

from boofuzz import *

import requests

SRIO = "192.168.0.2"

def liveness(target, logger, session, *a, **k):

try:

r = requests.get(f"http://{SRIO}/deviceinfo/swrevision", timeout=3)

if r.status_code >= 500:

raise BoofuzzTargetConnectionFailedError("5xx from IoT-Core")

except Exception as e:

raise BoofuzzTargetConnectionFailedError(str(e))

session = Session(

target=Target(connection=TCPSocketConnection(SRIO, 80)),

sleep_time=0.5,            # respect the 2-connection cap (SRIO-7909)

restart_sleep_time=5,

post_test_case_callbacks=[liveness],

)

s_initialize("HTTP-GET")

s_string("GET", name="method"); s_delim(" ")

s_string("/deviceinfo/swrevision", name="uri"); s_delim(" ")

s_string("HTTP/1.1"); s_static("\r\n")

s_string("Host", name="hdr"); s_delim(": "); s_string(SRIO); s_static("\r\n")

s_static("\r\n")

session.connect(s_get("HTTP-GET"))

session.fuzz()   # UI at http://127.0.0.1:26000

Appendix B — Domain-separation / liveness monitor (run during every active test)

Run this in a second terminal during nmap / nikto / siege / hping3 / boofuzz. It logs IoT-Core liveness alongside your manual PLC/PROFIsafe qualifier check, turning every active test into RQ-010 domain-separation evidence:

#!/bin/bash

: ${SRIO:=192.168.0.2}

while true; do

code=$(curl -s -o /dev/null -w "%{http_code}" --max-time 1 http://$SRIO/deviceinfo/swrevision)

echo "$(date +%H:%M:%S.%3N) IoT=$code  (watch PLC: PROFIsafe qualifier + DO read-back)"

sleep 0.5

done | tee domain_sep_monitor.log

Appendix C — Feasibility summary matrix

| Test case | Feasibility status |
| --- | --- |
| TC-RQ001-01 | Adapted/Modified |
| TC-RQ001-02 | Feasible |
| TC-RQ001-03 | Adapted/Modified |
| TC-RQ001-04 | Not Feasible |
| TC-RQ001-05 | Feasible |
| TC-RQ002-01 | Feasible |
| TC-RQ002-02 | Adapted/Modified |
| TC-RQ002-03 | Not Feasible |
| TC-RQ002-04 | Feasible |
| TC-RQ002-05 | Not Feasible |
| TC-RQ003-01 | Feasible |
| TC-RQ003-02 | Feasible |
| TC-RQ003-03 | Not Feasible |
| TC-RQ003-04 | Feasible |
| TC-RQ003-05 | Feasible |
| TC-RQ004-01 | Feasible |
| TC-RQ004-02 | Feasible |
| TC-RQ004-03 | Feasible |
| TC-RQ004-04 | Feasible |
| TC-RQ005-01 | Feasible |
| TC-RQ005-02 | Not Feasible |
| TC-RQ005-03 | Adapted/Modified |
| TC-RQ005-04 | Feasible |
| TC-RQ005-05 | Feasible |
| TC-RQ006-01 | Feasible |
| TC-RQ006-02 | Feasible |
| TC-RQ006-03 | Feasible |
| TC-RQ007-01 | Feasible |
| TC-RQ007-02 | Adapted/Modified |
| TC-RQ007-03 | Feasible |
| TC-RQ007-04 | Not Feasible |
| TC-RQ008-01 | Feasible |
| TC-RQ008-02 | Adapted/Modified |
| TC-RQ008-03 | Adapted/Modified |
| TC-RQ008-04 | Adapted/Modified |
| TC-RQ008-05 | Feasible |
| TC-RQ009-01 | Feasible |
| TC-RQ009-02 | Feasible |
| TC-RQ009-03 | Feasible |
| TC-RQ009-04 | Adapted/Modified |
| TC-RQ009-05 | Feasible |
| TC-RQ010-01 | Adapted/Modified |
| TC-RQ010-02 | Feasible |
| TC-RQ010-03 | Feasible |
| TC-RQ010-04 | Feasible |
| TC-RQ010-05 | Adapted/Modified |
| TC-RQ011-01 | Feasible |
| TC-RQ011-02 | Feasible |
| TC-RQ011-03 | Not Feasible |
| TC-RQ011-04 | Adapted/Modified |
| TC-RQ011-05 | Not Feasible |
| TC-RQ012-01 | Feasible |
| TC-RQ012-02 | Feasible |
| TC-RQ012-03 | Feasible |
| TC-RQ012-04 | Feasible |
| TC-RQ013-01 | Feasible |
| TC-RQ013-02 | Adapted/Modified |
| TC-RQ013-03 | Adapted/Modified |
| TC-RQ013-04 | Feasible |
| TC-RQ014-01 | Feasible |
| TC-RQ014-02 | Feasible |
| TC-RQ014-03 | Feasible |
| TC-RQ014-04 | Feasible |