WORKFLOW

Step 1: Critical Review & Theoretical Validation

- Requirement & Mapping: Review my mapping from the EU Machinery Regulation to the IEC 62443-4-2 Component Requirement (CR). Is it accurate and complete? Correct it if necessary.

- Assets & STRIDE: Evaluate my identified Target Assets and STRIDE threats. Are they plausible for this specific device and requirement? Add missing threats or correct misalignments.

- Security Functions & Gaps: Cross-reference my identified Security Functions and Gaps with the device documentation you were provided. Are the mitigations accurately described? Are the identified gaps realistic?

Step 2: Methodological Test Planning

- Based on the validated Security Functions and Gaps, derive concrete test scenarios to verify the mitigations or prove the gaps.

- Scientific Justification: Justify the chosen test methods using established technical guidelines (e.g., NIST SP 800-115 for network/port scanning, MITRE ATT&CK for ICS for active exploitation/sniffing, or OWASP for the IoT-Core web interface).

- IEC 62443-4-1 Categorization (SVV): Assign each test to the correct SVV category:

* SVV-1 (Requirements-based Testing): Verifying functional implementation in normal operation.

* SVV-2 (Threat Mitigation Testing): Verifying the effectiveness of countermeasures under attack/fault conditions.

* SVV-3 (Vulnerability Testing): Searching for known vulnerabilities or open attack surface (e.g., Nmap).

* SVV-4 (Penetration Testing): Active exploitation or protocol manipulation (e.g., Scapy, Fuzzing).

Step 3: Two-Level Test Catalog Generation

Generate the final test catalog entries for the planned tests. Each test must be structured in two levels:

- Level 1: Abstract Test Scenario (Product-neutral).

Must include: Test Case ID, Traceability (RQ -> CR -> Asset), Target Threat, SVV Category, Test Objective, Abstract Test Steps, and strict Pass/Fail Criteria.

- Level 2: Concrete Test Execution (Specific to my test environment).

Must include: Referenced Test Case ID, Hardware/Network Setup (Kali, Pi, Target IPs), Concrete Tools (Nmap, Wireshark, Scapy, etc.), Specific Commands/Scripts to be executed, and placeholders for the Result and Evidence (e.g., PCAP, XML logs).

REQUIREMENT 1

RQ-001 — Reprocessed (Scope-Corrected)

Correction applied: Rechecking RQ-001 against the scope-boundary discipline now established across RQ-002/004/005, I found one threat-without-a-CR inconsistency and one test that belongs to neighbouring requirements:

- DoS threat (over-reach). In my original RQ-001 I added Denial of Service to the STRIDE profile — but I never mapped a DoS CR (no CR 7.1) to back it, so the threat floated without a control. More importantly, generalized flooding/DoS resilience is the substance of RQ-010 ("withstand reasonably foreseeable malicious attempts"). RQ-001 is narrower: the connection and communication of another device does not lead to a hazardous situation — i.e., a rogue/additional device that connects and speaks (Spoofing + Tampering of the safety frames), not flooding-in-general. DoS is demoted to a cross-reference to RQ-010.

- DCP flood test (scope creep). My planned TS-5 (DCP set/reset flood) straddles RQ-011 (unauthenticated DCP = authorization — where I later made it the centrepiece, TC-RQ011-02) and RQ-010 (flooding). Removed from RQ-001 and cross-referenced.

What survives is the tight, correct RQ-001 core: a connecting device that communicates cannot drive the safety function into a hazardous state — protected by F_Dest_Add + 4-byte CRC + F_WD_Time watchdog, with the unchecked F_Source_Add / firewall-assumption as the residual gap.

Step 1 — Critical Review & Theoretical Validation

1.1 Requirement & CR Mapping

EU Machinery Regulation (EU) 2023/1230, Annex III EHSR 1.1.9 — the connection of another device, including any remote device that communicates with it, does not lead to a hazardous situation. This is an integrity + device-authenticity requirement about a connecting communication partner, and it is outcome-based ("does not lead to a hazardous situation") — so success = the connecting device's frames are rejected or resolve to the safe state.

| CR | Your input | Verdict | Correction |
| --- | --- | --- | --- |
| CR 3.1 – Communication Integrity | ✅ | Correct primary | Keep. Add RE(1) Communication Authentication — the*"connection of another device that communicates"* is fundamentally an authenticity question (is this the legitimate F-Host or a rogue partner?). |
| CR 1.2 – I&A of devices/processes | ✅ (I added) | Correct | Keep — the connecting device must be identified/authenticated.(Note the boundary: CR 1.2 authenticates the connecting device = RQ-001; CR 2.1 authorizes what it may do = RQ-011.) |
|  | — | Not mapped → RQ-010 | Generalized flooding-DoS resilience is RQ-010. No DoS CR belongs to RQ-001. |

Corrected mapping: RQ-001 → EHSR 1.1.9 →{CR 3.1 (+RE1) [integrity/authenticity of the connected device's data], CR 1.2 [I&A of the connecting device]}.(Flood/DoS resilience → RQ-010; unauthenticated DCP → RQ-011.) [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset A – Trusted Safety Function | ✅ | Correct (ultimate protected asset). |
| Asset F – Domain Separation | ✅ | Correct — the black channel (COM = non-safety) is why a connecting device on the non-safety path cannot directly corrupt the safety function. [Market_Acc...ty_Systems ¦ PDF] |
| Asset C – Transmitted Safety Frame | ✅ (I added) | Correct — the asset a connecting/rogue device actually attacks (forged/injected PROFIsafe frames). |

STRIDE — corrected to the connecting-device threats:

- ✅ Spoofing (primary). A rogue/additional device connects and impersonates the F-Host, or presents a forged F_Source_Add — the core "connection of another device that communicates" threat.

- ✅ Tampering (primary). That connected device injects/modifies safety frames.

- ✅ Replay (Tampering sub-case, keep). PROFIsafe's running consecutive number (folded into the CRC) is the mitigation.

- — Removed as a primary. A connected device that disrupts the black channel is handled by the watchdog → safe state (tested below as a fail-safe outcome), but generalized flooding-DoS resilience is RQ-010 — cross-referenced, not a standalone RQ-001 threat.

1.3 Security Functions & Gaps — Cross-Referenced

Your mitigations are confirmed:

| Mitigation | Documentation evidence | Confirmed |
| --- | --- | --- |
| PROFIsafe black channel (COM = QM non-safety; FSCP tunneled) | SysArch §3.3.1/4.1; SRIO-12921 | ✅ [Market_Acc...ty_Systems ¦ PDF] |
| Address Type 1: F_Dest_Add checked, F_Source_Add present but unchecked | SRIO-7830; SRIO-7794 ("In Address Type 1, the F-Source address will not be checked"); SRIO-10806/10807 | ✅ [209_SRIO_C...e_RemoteIO ¦ PDF] |
| 4-byte CRC, CRC-Seed24/32 per frame | SRIO-2998 | ✅ [209_SRIO_C...e_RemoteIO ¦ PDF] |
| F_WD_Time watchdog → passivation (outputs Low, qualifier bad); re-integration needs acknowledgment | SRIO-1857/1210; SRIO-853; channel-/device-granular passivation | ✅ [209_SRIO_C...e_RemoteIO ¦ PDF], [Market_Acc...ty_Systems ¦ PDF] |
| "Safe zone behind firewall" assumption | SRIO-9402 | ✅ (environmental) [100_SRIO_P...e_RemoteIO ¦ PDF] |

Gaps (connecting-device-scoped):

- G-1 (device-level, TESTABLE). Freedom-from-hazard against a connecting device's tampering and staleness is a device function: a forged/injected frame with a wrong F_Dest_Add or invalid CRC is rejected, and loss of the legitimate channel triggers watchdog passivation → safe state. This must be positively demonstrated. ✅ [209_SRIO_C...e_RemoteIO | PDF]

- G-2 (environmental, NOT device-testable — the RQ-001 core gap). Because F_Source_Add is unchecked under Address Type 1, a connecting device presenting a forged source but correct F_Dest_Add + a valid recomputed CRC cannot be rejected on a source basis. The CRC-Seed24/32 is a standardized, non-secret value in the public GSD, so it is not a cryptographic barrier. The only remaining barrier is the external firewall assumption (SRIO-9402) — an organizational/environmental control that device-level functional testing cannot close; it can only demonstrate the boundary (that a correctly-addressed, valid-CRC frame from a rogue device is accepted). [209_SRIO_C...e_RemoteIO | PDF], [100_SRIO_P...e_RemoteIO | PDF]

SL/conformance conclusion (connecting-device-scoped): RQ-001 is partially met. Against a connecting device that sends mis-addressed or corrupted frames, or that disrupts the channel, the device withstands the connection safely (F_Dest_Add + CRC + watchdog → reject/passivate → no hazardous output) — G-1 met. Against a connecting device that presents a forged source with valid addressing + CRC, authenticity cannot be enforced on the device (F_Source_Add unchecked); freedom-from-hazard rests on the firewall assumption — G-2, environmental.(Generalized flooding-DoS = RQ-010; unauthenticated DCP-reset = RQ-011.)

Step 2 — Methodological Test Planning (Scope-Corrected)

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Baseline PROFIsafe exchange with correct F_Dest_Add; verify qualifier = good, outputs follow control byte | Functional implementation of CR 3.1 in normal operation | Requirements-based verification | SVV-1 |
| TS-2 | Enumerate what a connecting device reaches on the DUT (services/ports on the non-safety path) vs. the approved matrix | Attack-surface a connected device sees → evidence for G-2 (firewall-assumption reliance) | NIST SP 800-115 §4.2/4.3; ifm CM-001 (Nmap) | SVV-3 |
| TS-3 | Rogue-device frame injection: (a) wrong F_Dest_Add, (b) bit-flipped CRC/payload, (c) correct F_Dest_Add + valid recomputed CRC, forged source | Effectiveness of CRC + F_Dest verification (a,b); demonstrates boundary of G-2 (c) | MITRE ATT&CK for ICS T0856 Spoof Reporting Message, T0836 Modify Parameter, T0855 Unauthorized Command Message; NIST SP 800-115 §5.2 | SVV-4 |
| TS-4 | A connecting/in-path device disrupts the black channel > F_WD_Time; observe passivation (outputs Low, qualifier bad) + acknowledged re-integration | Connection-induced disruption resolves to the safe state (fail-safe outcome of RQ-001) | MITRE ATT&CK for ICS T0814 context; PROFIsafe watchdog theory | SVV-2 |
|  | — | Moved → RQ-011 (unauthenticated DCP) / RQ-010 (flood) | — | — |

Scope note: TS-4 is framed as a connected device disrupting communication → safe state (the RQ-001 outcome), not as DoS-resilience-in-general; generalized flooding/fuzzing is cross-referenced to RQ-010 (TC-RQ010-02/03).

Environment binding: TS-3 side-inject on the Pi (eth0 = 192.168.0.100, EtherType 0x8892), captured on the laptop mirror; TS-4 uses the in-path 2-port bridge (temporary re-cabling per Zone-B §10), then reverts to side-inject. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog (Scope-Corrected)

Full entries for the baseline (TS-1), the rogue-device injection centrepiece (TS-3), the connection-disruption safe-state test (TS-4), and the reachability enumeration (TS-2). No DCP-flood entry (→ RQ-011/RQ-010).

TC-RQ001-01 — PROFIsafe Integrity in Normal Operation

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ001-01 |
| Traceability | RQ-001 (EHSR 1.1.9) → CR 3.1 → Asset C (+ Asset A) |
| Target Threat | Baseline (no attack) — reference for the connecting-device tests |
| SVV Category | SVV-1 (Requirements-based Testing) |
| Objective | Demonstrate that, with a correctly configured F_Dest_Add and valid CRC, the F-Device exchanges cyclic safety data, sets qualifier = good, and drives outputs per the control byte. |
| Abstract Steps | 1. Configure the F-Host with F_Dest_Add matching the DUT. 2. Establish PROFIsafe connection → OPERATE. 3. Toggle a safe input and a safe output. 4. Record process data, qualifiers, CRC over ≥ 60 s. |
| Pass/Fail | PASS iff qualifier = good throughout, outputs mirror the commanded control byte, and no passivation occurs nominally. FAIL on spurious passivation, qualifier=bad, or output/command mismatch. |

Level 2 — Concrete Execution

| Field | Content |  |  |
| --- | --- | --- | --- |
| Ref. | TC-RQ001-01 |  |  |
| HW/Network | SPS 192.168.0.1; SRIO DUT 192.168.0.2 (F_Dest_Add via rotary switch); Laptop mirror (Wireshark, no IP) |  |  |
| Tools | TIA Portal (F-Host), Wireshark (`pn_rt |  | pnio`) |
| Procedure | (1) Download safety program, set F_Dest_Add; (2) PLC→RUN; (3) capture on mirror; (4) force Q bit → observe DO; (5) stimulate F-DI → observe input in PLC |  |  |
| Result | ☐ PASS ☐ FAIL |  |  |
| Evidence | TC01_baseline.pcapng, PLC watch-table screenshot, timestamped |  |  |

TC-RQ001-02 — Rogue-Device Forged / Tampered Frame Injection (G-1 verification + G-2 boundary)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ001-02 |
| Traceability | RQ-001 → CR 3.1 (+RE1) / CR 1.2 → Asset C, Asset F |
| Target Threat | Spoofing + Tampering (connecting rogue device) |
| SVV Category | SVV-4 (Penetration Testing / protocol manipulation) |
| Objective | Prove the F-Device rejects frames from a connecting rogue device with (a) an incorrect F_Dest_Add and (b) an invalid CRC (→ safe outcome), and characterise the boundary by injecting (c) a frame with correct F_Dest_Add + a valid recomputed CRC and a forged source. |
| Abstract Steps | With cyclic traffic running, a rogue device injects crafted FSCP frames: (a) wrong F_Dest_Add; (b) correct F_Dest_Add, corrupted CRC/payload; (c) correct F_Dest_Add + recomputed valid CRC, forged source MAC. Observe qualifier, outputs, diagnostics. |
| Pass/Fail | PASS (a,b) iff the DUT ignores/rejects the frame and continues on legitimate traffic or passivates to safe state (outputs Low, qualifier bad) — no unsafe actuation.(c) = boundary characterisation: expected result = frame accepted (proving G-2 — authenticity depends on the black channel/firewall), recorded as residual risk covered by SRIO-9402, not scored as device FAIL. FAIL iff (a) or (b) causes an uncommanded/hazardous output. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ001-02 |
| HW/Network | Pi eth0 = 192.168.0.100 side-inject; DUT 192.168.0.2; SPS 192.168.0.1; laptop mirror |
| Tools | Scapy (Pi), Wireshark (laptop) |

#!/usr/bin/env python3

# TC-RQ001-02 Rogue-device PROFIsafe injection (Layer 2, EtherType 0x8892)

from scapy.all import Ether, Raw, sendp, rdpcap

IFACE = "eth0"

tmpl = rdpcap("ref.pcap")[0] # one legitimate cyclic frame from the mirror

payload = bytes(tmpl[Raw].load) # PN-RT + PROFIsafe (PD | status/control | 4-byte CRC)

def inject(name, data, dst=tmpl[Ether].dst, src="02:11:22:33:44:55"):

sendp(Ether(dst=dst, src=src, type=0x8892)/Raw(load=data),

iface=IFACE, count=200, inter=0.002)

print(f"[+] sent: {name}")

a = bytearray(payload); a[-6] ^= 0xFF # (a) WRONG F_Dest_Add

inject("a_wrong_dest", bytes(a))

b = bytearray(payload); b[2] ^= 0x01 # (b) corrupt PD, leave CRC -> mismatch

inject("b_bad_crc", bytes(b))

inject("c_valid_from_rogue", payload) # (c) BOUNDARY: valid frame, forged source -> G-2

| Result | (a) ☐PASS ☐FAIL (b) ☐PASS ☐FAIL (c) ☐ boundary confirmed → residual risk logged | | Evidence | TC02_inject.pcapng, Pi log, SRIO FS/P-LED photo, PLC diagnostic-buffer export |

Safety: active injection runs on the Pi in isolated Zone-B with the PLC as watchdog — never on the company laptop or production. [Security_T...rk_Kali_EN | Word]

TC-RQ001-03 — Connection-Disruption → Safe-State Verification (G-1, fail-safe outcome)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ001-03 |
| Traceability | RQ-001 → CR 3.1 → Asset C, Asset A |
| Target Threat | Connection-induced communication disruption (a connected/in-path device withholding the channel) |
| SVV Category | SVV-2 (Threat-Mitigation Testing) |
| Objective | Verify that when a connecting/in-path device disrupts the black channel longer than F_WD_Time, the device passivates to the defined safe state (outputs Low, qualifier bad) and that re-integration requires acknowledgment — i.e., the connection of a disrupting device does not lead to a hazardous situation. |
| Abstract Steps | 1. Reach OPERATE with a known F_WD_Time. 2. Interrupt the safe-comms path for t > F_WD_Time. 3. Measure time-to-safe-state; confirm outputs Low + qualifier bad. 4. Restore; confirm no auto-restart without acknowledgment. |
| Pass/Fail | PASS iff passivation occurs within the device acknowledgment/response time, outputs = Low, qualifier = bad, and re-integration is gated by acknowledgment. FAIL iff outputs remain energized past F_WD_Time, or the device auto-re-integrates without acknowledgment. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ001-03 |
| HW/Network | In-path bridge: Pi between SPS↔SRIO (temporary re-cabling per Zone-B §10). F_WD_Time e.g. 150 ms. DO wired to PLC input for readback. |
| Tools | Linux bridge utils (Pi); Wireshark (laptop) for Δt correlation |

# TC-RQ001-03 In-path connection-disruption (> F_WD_Time)

sudo ip link add br0 type bridge

sudo ip link set eth0 master br0 # towards SPS

sudo ip link set eth2 master br0 # towards SRIO

sudo ip link set br0 up

sudo ip link set br0 down; sleep 1; sudo ip link set br0 up # interrupt > F_WD_Time

| Result | ☐ PASS ☐ FAIL — measured t_passivation = ______ ms | | Evidence | TC03_wd.pcapng with Δt cursor, DO=Low trace, FS-LED photo, PLC passivation/acknowledge log |

Scope note: this verifies the fail-safe outcome for a connected device disrupting comms; generalized flooding/DoS resilience is tested under RQ-010 (TC-RQ010-02).

TC-RQ001-04 — Connecting-Device Reachability / Attack Surface (G-2 evidence)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ001-04 |
| Traceability | RQ-001 → CR 3.1 → Asset F |
| Target Threat | Spoofing/Tampering surface a connecting device sees (evidence for G-2) |
| SVV Category | SVV-3 (Vulnerability Testing) |
| Objective | Enumerate the services a connecting device can reach on the non-safety path and compare against the approved matrix, evidencing that RQ-001's residual safety on that path relies on the firewall assumption (SRIO-9402). |
| Abstract Steps | Full TCP + top-UDP scan + service/version detection against the DUT; diff vs. design; flag unexpected exposure. |
| Pass/Fail | PASS iff only design-approved services are exposed (e.g., IoT-Core HTTP; no HTTPS/MQTT/WebSocket). FAIL iff any un-approved service is reachable. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ001-04 |
| HW/Network | DUT IP set via DCP to 192.168.0.2; Pi eth0 = 192.168.0.100 |
| Tools | Nmap (Pi) — ifm CM-001; pnio_dcp for L2 identify |

# TC-RQ001-04 Reachability enumeration a connecting device would see

sudo nmap -sS -sV -p- -T3 -oA TC04_srio_tcp 192.168.0.2

sudo nmap -sU --top-ports 200 -oA TC04_srio_udp 192.168.0.2

python3 -c "import pnio_dcp; print(pnio_dcp.DCP('eth0').identify_all())"

| Expected | Open: HTTP :80 (IoT-Core), PROFINET/DCP L2. Absent: 443 (HTTPS), 1883 (MQTT), WebSocket (SRIO-7906/7907). | | Result | ☐ PASS ☐ FAIL — deviations: ______ | | Evidence | TC04_srio_tcp.xml/.nmap, TC04_srio_udp.xml, DCP identify log | [209_SRIO_C...e_RemoteIO | PDF]

Summary of Changes (vs. my original RQ-001)

- Removed the DoS threat over-reach — I had added Denial of Service to STRIDE without a backing CR; generalized flooding-DoS resilience is RQ-010. DoS is now a cross-reference, and the watchdog test is reframed as a fail-safe outcome of a connected device disrupting comms (the RQ-001 EHSR outcome), not DoS-resilience-in-general.

- Removed TS-5 (DCP set/reset flood) — it straddles RQ-011 (unauthenticated DCP, now TC-RQ011-02) and RQ-010 (flood). Cross-referenced out.

- Kept the RQ-001 core intact — CR 3.1(+RE1) + CR 1.2; Spoofing + Tampering (+Replay); Assets A, C, F; and the two decisive gaps: G-1 (device-level, testable: F_Dest+CRC+watchdog) and G-2 (environmental: F_Source_Add unchecked → firewall assumption, not device-testable). [209_SRIO_C...e_RemoteIO | PDF], [100_SRIO_P...e_RemoteIO | PDF]

- Reframed TC-RQ001-03 from "Denial of Service / staleness" to**"Connection-Disruption → Safe-State Verification,"** aligning it with RQ-001's outcome-based text and cross-referencing RQ-010 for flooding.

The corrected verdict is tightly bounded: RQ-001 = a connecting/communicating device cannot drive the safety function into a hazardous state — mis-addressed, corrupted, or disrupting connections resolve safely (G-1 met); a forged-source, valid-CRC connection is stopped only by the firewall assumption (G-2, environmental) — with flood/DoS resilience → RQ-010 and DCP/update authorization → RQ-011.

Would you like me to (a) re-check RQ-006 and RQ-010 for the same boundary realignment (RQ-006 reused RQ-004's identity-integrity material; RQ-010 should now cleanly receive the DoS/DCP-flood tests), or (b) proceed to consolidation of the corrected set into the thesis appendix?

REQUIREMENT 2

RQ-002 — Reprocessed (Scope-Corrected)

Correction applied: You are right, and thank you for the catch. In my original RQ-002 I imported the*"collect evidence of intervention"* clause and mapped CR 2.8 / CR 6.1 (auditable events / audit log), Asset I (Audit Data), and the Repudiation threat into this requirement. That was scope creep — evidence collection is the substance of RQ-003, which we subsequently processed on its own. RQ-002 is a pure protection-against-corruption requirement (integrity + tamper-resistance), not an accountability/audit one. Below is the corrected analysis with the audit material removed and returned to RQ-003.

Step 1 — Critical Review & Theoretical Validation

1.1 Requirement & CR Mapping

Your text is EU Machinery Regulation (EU) 2023/1230, Annex III EHSR 1.1.9 §1 ("protect … against corruption, whether accidental or intentional"). This is an integrity / tamper-resistance requirement across three surfaces: the transmitted frame, the stored parameters, and the physical address/mode interface.

| CR / EDR | Your input | Verdict | Correction / Addition |
| --- | --- | --- | --- |
| CR 3.1 – Communication Integrity | ✅ | Correct for the transmitted frame | Keep. Add RE(1) Communication Authentication — RE(1) is the intentional-corruption limb; the base CR only addresses integrity, not authenticity. |
| EDR 3.11 – Physical tamper resistance | ✅ | Correct for the physical switch interface | Keep — but scoped to the resistance limb (a barrier that resists/reveals corruption). The audit-record limb is not part of RQ-002. |
| CR 3.4 – Software & Information Integrity | ➕ Add | Correct addition | The iParCRC/F_ParCRC protect the stored safety configuration (data at rest), which CR 3.1 (in-transit) does not cover. This is a legitimate RQ-002 protection surface. |
|  | ❌ Removed | Scope creep → RQ-003 | These map to the evidence-collection obligation, which is RQ-003. Removed from RQ-002. |

Corrected mapping: RQ-002 → EHSR 1.1.9 §1 →{CR 3.1 (+RE1) [transmitted data], CR 3.4 [stored parameters], EDR 3.11 [physical tamper-resistance]}. [IEC62443_4_2_source | Txt]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset C – Transmitted Safety Frame | ✅ | Correct primary asset. |
| Asset G – Physical F-address/mode interface | ✅ | Correct — the rotary switch sets F_Dest_Add and, at value 999, triggers firmware-update mode (SRIO-2013), so this interface governs connection identity and access to safety-critical software. [100_SRIO_P...e_RemoteIO ¦ PDF], [Market_Acc...ty_Systems ¦ PDF] |
|  | ❌ Removed | Belongs to RQ-003 (evidence). Removed here. |

STRIDE — Tampering is correct and primary. Corrected profile:

- ✅ Tampering (primary) — corruption of frame / parameter set / physical switch (accidental or intentional).

- ➕ Spoofing-of-source (integrity limb, keep). For the transmitted frame, "intentional corruption" includes a forged-but-CRC-valid frame — an authenticity failure that CR 3.1 RE(1) would address. This stays, because it is about protecting integrity/authenticity of the data, not about audit.

- — Removed. Repudiation is the accountability threat that belongs to RQ-003.

1.3 Security Functions & Gaps — Cross-Referenced

Your mitigations are all confirmed in the documentation:

| Mitigation | Documentation evidence | Confirmed |
| --- | --- | --- |
| 4-byte CRC, CRC-Seed24/32 on the safety frame | SRIO-2998 (F_CRC_Length = 4-Byte-CRC; F_CRC_Seed = CRC-Seed24/32) | ✅ [209_SRIO_C...e_RemoteIO ¦ PDF] |
| iParCRC (UINT32) + F_ParCRC (UINT16); mismatch → EC3 rejection | F-Parameter table (byte 9 F_iParCRC UINT32, byte 13 F_ParCRC UINT16); SRIO-12627/8575/8574; CRC-Tool SRIO-9023 (T2, TÜV-Süd certified) | ✅ [209_SRIO_C...e_RemoteIO ¦ PDF], [Market_Acc...ty_Systems ¦ PDF], [218_SRIO_M...e_RemoteIO ¦ PDF] |
| Physical seals on switch housing | SRIO-2102 (plastic E12839 / metal E12542), SRIO-11001, SRIO-11000 | ✅ — but see gap: these exist for the moulding process and IP protection, not as a security tamper-resistance feature. [Market_Acc...ty_Systems ¦ PDF] |
| ➕ Rotary switch read only at init (runtime change ignored until reboot) | SRIO-8803, SRIO-10812 | ✅ Add — a genuine temporal protection you omitted: a physical change during operation cannot corrupt the running safety address. [Market_Acc...ty_Systems ¦ PDF] |

Gap validation — your gaps are correct; here they are, scoped strictly to protection (no audit):

- G-1 (transmitted data — accurate, keep). The CRC is a checksum, not a MAC. The CRC-Seed24/32 is a standardized, non-secret value in the public GSD, so an attacker who modifies the process data and recomputes a valid CRC is not detected. The device protects against accidental corruption but not intentional corruption — exactly the distinction RQ-002 draws. ✅ [209_SRIO_C...e_RemoteIO | PDF]

- G-2 (stored parameters — extend your gap here). The same limitation applies to iParCRC/F_ParCRC. F_ParCRC is only 16-bit (2¹⁶ space → no forgery resistance); the iParCRC is 32-bit but still non-cryptographic and recomputable from the public parameter layout. So the "accidental-only" protection gap covers the whole configuration path, not just process data. [209_SRIO_C...e_RemoteIO | PDF]

- G-3 (physical — accurate, scoped to resistance). The seals provide passive, visual tamper-resistance only. The device has no active tamper-detection mechanism that would trigger a protective response to physical corruption of the switch interface. The one genuine device-level protection is the temporal one (switch read only at init) — a runtime physical change is ignored, which protects the running safety function, but the stored/next-boot address can still be corrupted by physically altering the switch, and the seal is the only barrier.(Note: whether such tampering is recorded is out of scope here — that is RQ-003.) [Market_Acc...ty_Systems | PDF]

SL/conformance conclusion (protection-scoped): RQ-002 is partially met. The transmitted frame and stored parameters are protected against accidental corruption (CRC/iParCRC/F_ParCRC at SL 1) but not intentional corruption (no MAC/signature). The physical interface has passive tamper-resistance (seals) + a temporal protection (init-only read), but no active tamper-detection/response → EDR 3.11 resistance limb partially met. Net: protection against accidental corruption achieved; protection against intentional corruption not achieved.

Step 2 — Methodological Test Planning (Scope-Corrected)

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Present a configuration with a wrong iParCRC / F_ParCRC; verify rejection (EC3, stays in Parametrization) | Functional CR 3.4 in normal operation | Requirements-based verification | SVV-1 |
| TS-2 | Bit-flip the payload leaving CRC stale → CRC mismatch → passivation | Effectiveness of the CRC countermeasure under fault | NIST SP 800-115 §5.2; PROFIsafe CRC theory | SVV-2 |
| TS-3 | Recompute a VALID CRC over MODIFIED process data and inject | Proves G-1: intentional corruption undetected | MITRE ATT&CK for ICS T0836 Modify Parameter, T0856; NIST SP 800-115 §5.2 | SVV-4 |
| TS-4 | F_ParCRC forgery feasibility — analyse the 16-bit F_ParCRC / 32-bit iParCRC forgeability | Proves G-2: parameter CRCs are not authentication | NIST SP 800-115 §4.3; ifm CM-010 (boofuzz) for the search | SVV-3 |
| TS-5 | Physical tamper-resistance: remove seal, alter rotary switch, reboot; verify (a) runtime change is ignored (temporal protection) and (b) the next-boot address can be corrupted, with only the seal resisting | Proves G-3 (resistance-only physical protection) | OWASP ISTG ISTG-PHY; EDR 3.11 resistance limb | SVV-4 (PA-4) |

(Note: the check of whether a physical intervention is logged/recorded has been moved to RQ-003 / TC-RQ003-03 — it is not part of RQ-002.)

Environment binding: TS-2/TS-3/TS-4 side-inject on the Pi (eth0 = 192.168.0.100, EtherType 0x8892), captured on the laptop mirror; TS-5 requires invasive physical access (PA-4), isolated-lab, PLC-monitored. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog (Scope-Corrected)

Full entries for the anchor (TS-1), the decisive G-1 proof (TS-3), and the corrected physical-resistance test (TS-5). TS-2/TS-4 follow the same template on request.

TC-RQ002-01 — Parameter-CRC Rejection (iParCRC / F_ParCRC)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ002-01 |
| Traceability | RQ-002 (EHSR 1.1.9 §1) → CR 3.4 → Asset C (config at rest) |
| Target Threat | Tampering (parameter set) |
| SVV Category | SVV-1 (Requirements-based Testing) |
| Objective | Demonstrate that the F-Device validates the parameter set and rejects a configuration whose iParCRC/F_ParCRC does not match, remaining in the safe (Parametrization) state. |
| Abstract Steps | 1. Baseline. 2. Present a configuration with a deliberately incorrect iParCRC (then F_ParCRC). 3. Observe device state, diagnostics, qualifiers. |
| Pass/Fail | PASS iff mismatch triggers EC3, the device stays in Parametrization, qualifiers remain bad, and no OPERATE handshake completes. FAIL iff a mismatched-CRC configuration is accepted. |

Level 2 — Concrete Execution

| Field | Content |  |  |
| --- | --- | --- | --- |
| Ref. | TC-RQ002-01 |  |  |
| HW/Network | SPS 192.168.0.1, SRIO DUT 192.168.0.2; laptop mirror |  |  |
| Tools | TIA Portal (manipulate F-parameter CRC field), Wireshark (`pn_rt |  | pnio`) |
| Procedure | (1) Baseline OPERATE; (2) force F_iPar_CRC wrong, download; (3) repeat for F_Par_CRC; (4) read PLC diagnostic buffer for EC3 |  |  |
| Result | ☐ PASS ☐ FAIL — state: ______ |  |  |
| Evidence | TC01_iparcrc.pcapng, PLC diagnostic-buffer export, DUT FS-LED photo |  |  |

TC-RQ002-03 — Valid-CRC Injection over Modified Process Data (G-1 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ002-03 |
| Traceability | RQ-002 → CR 3.1(+RE1) → Asset C |
| Target Threat | Tampering (intentional) |
| SVV Category | SVV-4 (Penetration Testing / protocol manipulation) |
| Objective | Demonstrate the central residual risk of RQ-002: because the 4-byte CRC (CRC-Seed24/32) is a non-secret checksum, an attacker who modifies the process data and recomputes a matching CRC produces a frame the F-Device cannot distinguish from a legitimate one — protection against accidental but not intentional corruption. |
| Abstract Steps | 1. Capture a legitimate cyclic frame; extract F-Parameters (F_Dest_Add, seed). 2. Modify the payload. 3. Recompute the CRC over the modified payload. 4. Inject; observe qualifier/outputs. |
| Pass/Fail | Gap-characterisation test. Expected = frame ACCEPTED (qualifier good, outputs follow injected data) → evidences G-1; recorded as residual risk mitigated only by the environmental firewall assumption (SRIO-9402) and requiring a cryptographic-MAC upgrade. A device that rejected the valid-CRC frame indicates an undocumented check to investigate. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ002-03 |
| HW/Network | Pi eth0 = 192.168.0.100 side-inject; DUT 192.168.0.2; SPS 192.168.0.1; laptop mirror |
| Tools | Scapy + PROFIsafe CRC routine (CRC-Seed24/32, 4-byte), Wireshark |

#!/usr/bin/env python3

# TC-RQ002-03 Valid-CRC-over-modified-data (proves accidental != intentional protection)

from scapy.all import Ether, Raw, sendp, rdpcap

IFACE="eth0"; tmpl=rdpcap("ref.pcap")[0]; raw=bytearray(tmpl[Raw].load)

def profisafe_crc(data: bytes, seed: int) -> int:

poly=0x1F4ACFB13; crc=seed

for b in data:

crc ^= b<<24

for _ in range(8):

crc=((crc<<1)^poly) if (crc&0x80000000) else (crc<<1); crc&=0xFFFFFFFF

return crc

PD=slice(0,-4); SEED=0x00000000 # derive from GSD CRC-Seed24/32 + F-Parameters

raw[2]^=0x01 # modify safety process data

raw[-4:]=profisafe_crc(bytes(raw[PD]),SEED).to_bytes(4,"big") # recompute VALID CRC

sendp(Ether(dst=tmpl[Ether].dst, src="02:aa:bb:cc:dd:ee", type=0x8892)/Raw(bytes(raw)),

iface=IFACE, count=300, inter=0.002)

print("[+] injected modified frame WITH valid recomputed CRC")

| Result | ☐ Accepted (G-1 confirmed) ☐ Rejected (investigate) | | Evidence | TC03_validcrc.pcapng, Pi log, PLC I/O trace, CRC-recompute source archived |

Method note: validate the CRC routine reproduces a legitimate captured CRC bit-for-bit before modifying the payload, to make the acceptance result scientifically defensible. [NIST Techn...Assessment | PDF]

TC-RQ002-05 — Physical Tamper-Resistance of the F-Address Interface (G-3 proof — corrected scope)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ002-05 |
| Traceability | RQ-002 → EDR 3.11 (resistance limb) → Asset G |
| Target Threat | Tampering (physical, intentional) |
| SVV Category | SVV-4 (Penetration Testing — physical, PA-4) |
| Objective | Assess the protection the device provides against physical corruption of the F-address/mode switch: (a) confirm the temporal protection (a runtime switch change is ignored until reboot, so the running safety address cannot be corrupted live), and (b) determine whether the stored/next-boot address can be corrupted by physically altering the switch, with the seal being the only resisting barrier. This test evaluates tamper-resistance only; whether the intervention is recorded is assessed separately under RQ-003. |
| Abstract Steps | 1. Record baseline address and seal state. 2. With the device in OPERATE, remove the seal/sleeve and change the rotary switch; verify (per SRIO-10812) the running address is unchanged (temporal protection holds). 3. Cold-restart; confirm the new address now takes effect (next-boot corruption succeeded). 4. Assess the seal's resistance (did alteration require visible seal damage?). |
| Pass/Fail | Gap-characterisation test. Expected: (i) runtime change ignored (temporal protection = a genuine partial mitigation to credit); (ii) after reboot the altered address applies → the stored address is physically corruptible; (iii) the only resistance is the passive seal, with no active tamper-detection/response by the device → confirms G-3 (resistance-only physical protection), recorded as a partial EDR 3.11 result. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ002-05 |
| HW/Network | Physical access to DUT 192.168.0.2; management path Pi eth1 = 192.168.50.10 → IoT-Core (to read the applied address); browser 192.168.50.20 |
| Tools | Manual (seal removal, switch actuation); camera (seal state); curl to read /safecom/f_address |

# TC-RQ002-05 Physical tamper-resistance of the F-address switch (resistance only)

curl -s http://192.168.0.2/safecom/f_address -o TC05_addr_before.json # baseline

# --- OPERATE: remove seal, change rotary switch; verify running address UNCHANGED ---

curl -s http://192.168.0.2/safecom/f_address -o TC05_addr_runtime.json # expect: == before (SRIO-10812)

# --- COLD RESTART ---

curl -s http://192.168.0.2/safecom/f_address -o TC05_addr_afterboot.json # expect: == new switch value

diff TC05_addr_before.json TC05_addr_runtime.json | tee TC05_runtime_diff.txt # expect: no change

diff TC05_addr_runtime.json TC05_addr_afterboot.json | tee TC05_boot_diff.txt # expect: changed

| Expected | Runtime change ignored (temporal protection, SRIO-8803/10812); new address applies after reboot; only the passive seal resisted the physical alteration; no active tamper-detection. | | Result | ☐ Runtime change ignored (partial mitigation) ☐ Next-boot address corruptible ☐ Resistance = seal only (G-3 confirmed) | | Evidence | Seal before/after photos, TC05_addr_* reads, TC05_runtime_diff.txt, TC05_boot_diff.txt | [Market_Acc...ty_Systems | PDF]

Summary of Changes (vs. my original RQ-002)

- Removed the evidence-collection scope creep — dropped CR 2.8 / CR 6.1, Asset I (Audit Data), and the Repudiation threat from RQ-002. These belong to RQ-003 (where we correctly handled them). [IEC62443_4_2_source | Txt]

- Re-scoped EDR 3.11 to its tamper-resistance limb only (physical barrier that resists/reveals corruption), not the audit-record limb.

- Corrected TC-RQ002-05 — it now tests physical tamper-resistance (temporal protection + seal-only barrier), and the*"is the intervention logged?"* check is explicitly deferred to RQ-003 / TC-RQ003-03.

- Retained the substantive protection gaps (G-1 transmitted-frame CRC≠MAC; G-2 stored-parameter CRC≠MAC incl. 16-bit F_ParCRC; G-3 resistance-only physical protection) and the temporal-protection mitigation (switch read only at init). [209_SRIO_C...e_RemoteIO | PDF], [Market_Acc...ty_Systems | PDF]

The corrected verdict is cleaner: RQ-002 protects against accidental corruption (CRC/iParCRC/F_ParCRC + seals + init-only read) but not intentional corruption (no MAC/signature, resistance-only physical barrier) — with the evidence dimension now living entirely in RQ-003.

Would you like me to (a) also re-check RQ-004/005 for the same evidence-scope boundary (they reference audit in places), (b) complete TC-RQ002-02/04, or (c) proceed to consolidation of the corrected set?

REQUIREMENT 3

Step 1 — Critical Review & Theoretical Validation

RQ-003 is the accountability twin of RQ-002: where RQ-002 required protection against corruption, RQ-003 requires evidence collection of intervention. This is precisely the "collect evidence of intervention" clause I flagged as missing in your RQ-002 mapping — so it is good to see it broken out as its own requirement. Your analysis is honest and directionally correct, and your central conclusion (the requirement is essentially not met for physical interventions) is defensible. However, the CR set is materially incomplete, and you have overlooked several partial evidence mechanisms that the device does implement — a rigorous thesis must acknowledge and then dismantle them, rather than claim nothing exists.

1.1 Requirement & CR Mapping

Your text is EU Machinery Regulation (EU) 2023/1230, Annex III EHSR 1.1.9 §2 (the evidence sentence). This is fundamentally an audit/accountability requirement, and audit requirements in IEC 62443-4-2 never stand alone — CR 2.8 is the generation of records, but a record without time, capacity, overflow handling, and protection is not usable evidence.

| CR / EDR | Your input | Verdict | Correction / Addition |
| --- | --- | --- | --- |
| CR 2.8 – Auditable events | ✅ | Correct, but unmet for physical interventions | Keep. The device generates ErrorEvents for logical/self-detected faults, not for physical switch/seal manipulation. |
| CR 2.12 – Non-repudiation | ✅ | Correct target, but structurally impossible here | Keep with caveat: CR 2.12 base is about human users determined via identity. The physical F-address interface has no user authentication, so attribution ("who intervened") cannot be satisfied by design. |
| EDR 3.11 – Physical tamper resistance and detection | ✅ | Correct — the detection limb is the relevant one | Keep. Seals = resistance; detection limb is unmet (→ gap). |
| CR 2.11 – Timestamps | ❌ Missing | ➕ Add — this is the decisive omission | Evidence without reliable time is not evidence. The device has only a 4-byte uptime counter in seconds that is cleared/reset on cold start (SRIO-15218/15398) and a systick in ms — no absolute/synchronised real-time clock. An intervention therefore cannot be placed in absolute time. |
| CR 2.9 – Audit storage capacity | ❌ Missing | ➕ Add | The COM Error Log is a circular buffer of ≥100 entries (SRIO-10703/10663) — old evidence is overwritten. |
| CR 2.10 – Response to audit processing failure | ❌ Missing | ➕ Add | No defined safe behaviour on log overflow beyond circular overwrite. |
| CR 3.9 – Protection of audit information | ❌ Missing | ➕ Add | The evidence store is non-safety (QM) and cleared on cold start (SRIO-6521) — the evidence itself is neither integrity-protected nor persistent, so it fails as forensic evidence. |
| CR 6.1 – Audit-log accessibility | ❌ Missing | ➕ Add — and this one is (partially) MET | The Error Log is readable via IoT-Core (SRIO-10662/15221). Credit this as the one genuinely satisfied sub-capability. |
| EDR 2.13 / CR 2.13 – Use of physical diagnostic & test interfaces | ❌ Missing | ➕ Add | This is the correct home for your debug/test-port point (below), not CR 2.8. |

Corrected mapping: RQ-003 → EHSR 1.1.9 §2 →{CR 2.8, CR 2.9, CR 2.10, CR 2.11, CR 2.12, CR 3.9, CR 6.1, EDR 3.11 (detection limb), EDR 2.13}. [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset G – Physical F-address/OP-mode interface | ✅ | Correct — and doubly relevant: the rotary switch sets the F_Dest_Add and at value 999 enters firmware-update mode (SRIO-2013). [100_SRIO_P...e_RemoteIO ¦ PDF] |
| Asset I – Audit Data | ✅ | Correct — but note the candidate store (COM Error Log) is non-safety and volatile across cold start. [209_SRIO_C...e_RemoteIO ¦ PDF] |

STRIDE — Repudiation is correct and primary. Add one note:

- ➕ Tampering (on the audit asset itself). Because the log is unprotected and cold-start clears it, an attacker can erase evidence by forcing a power cycle — an anti-forensic Tampering vector against Asset I (this is why CR 3.9 must be in the map). MITRE ATT&CK for ICS frames this as T0872 Indicator Removal on Host.

1.3 Security Functions & Gaps — Cross-Referenced

Your stated mitigation ("seal + visual inspection only, no automatic tamper logging") is accurate for physical tamper, but you have understated what the device does collect. A defensible thesis must enumerate these partial mechanisms and then show why each is insufficient:

| Mechanism (some you missed) | Documentation evidence | What it evidences | Why it still fails RQ-003 |
| --- | --- | --- | --- |
| Physical seals (plastic E12839 / metal E12542) | SRIO-2102/11001/11000 | Visual resistance | No electronic record; exists for moulding/IP, not security. [Market_Acc...ty_Systems ¦ PDF] |
| Rotary switch read only at init | SRIO-8803, SRIO-10812 | Temporal protection (change ignored during OPERATE) | Partial resistance, generates no evidence. [Market_Acc...ty_Systems ¦ PDF] |
| ➕ I&M0 REVISION_COUNTER (increments on each parameter change) | SRIO-3002/3015 | Count of configuration interventions | Counter only — no who/when/what, no timestamp, wraps at 0xFFFF; useless for physical switch tamper. [209_SRIO_C...e_RemoteIO ¦ PDF] |
| ➕ I&M4 signature (F_iParCRC + F_Par_CRC) | SRIO-3011 | Detect that parameters changed (signature differs) | Runtime-generated, not stored remanently; no attribution/time. [209_SRIO_C...e_RemoteIO ¦ PDF] |
| ➕ COM Error Log (ErrorEvents, e.g. SCPU timeout) | SRIO-10703/10663/6521/10662 | Indirect trace of some logical interventions | Circular, non-safety, cleared on cold start; no physical-tamper events. [209_SRIO_C...e_RemoteIO ¦ PDF] |
| ➕ Debug/Test port fused off in production | SRIO-2580 (dev phase only), SRIO-1526, /fit (P) production_fuse | Removes an access path to safety software | Design exclusion → EDR 2.13, not an evidence entry — your point is correct. [100_SRIO_P...e_RemoteIO ¦ PDF], [209_SRIO_C...e_RemoteIO ¦ PDF] |

Gap validation — your gap is correct; I am partitioning and hardening it:

- G-1 (physical tamper — accurate, keep). No tamper switch/sensor → EDR 3.11 detection limb unmet. An intervention avoiding visible seal damage is unrecorded and unattributable. ✅

- G-2 (time attribution — you missed this; add it). Even the events the device does log carry no absolute timestamp — only uptime that resets on cold start. So RQ-003 evidence is not time-referenceable, and a cold start both clears and de-times the record. This is arguably a worse failure than the missing sensor, because it undermines even the partial logs. [209_SRIO_C...e_RemoteIO | PDF]

- G-3 (evidence protection & retention — add). The store is non-safety (QM), circular, cold-start-cleared → fails CR 2.9/3.9 and cannot be "retained for the mandated period." ✅ (extends your point)

- G-4 (debug/test port — accurate, keep). Fusing is a valid EDR 2.13 compensating exclusion, but RQ-003 strictly requires evidence of any intervention; a physical re-probe of the fused interface still yields no log entry. ✅

SL conclusion for the thesis: For the evidence-collection capability against physical interventions the device sits at SL 0 (no detection, no attribution, no protected/timed record). Against logical/configuration interventions it reaches only a weak SL 1-partial via REVISION_COUNTER / I&M4 / Error Log, undermined by the absent RTC and cold-start clearing. CR 6.1 (readability) is the sole fully-met sub-capability.

Step 2 — Methodological Test Planning

This requirement is largely about demonstrating the absence/insufficiency of a control, so the test design is evidence-of-negative — proving no usable record is produced.

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Enumerate what evidence does exist: read COM Error Log via IoT-Core; confirm I&M0 REVISION_COUNTER increments on a parameter change; read I&M4 signature | Functional baseline of the partial controls (CR 6.1 met; CR 2.8 partial) | Requirements-based verification; NIST SP 800-115 §3.2 (log review) | SVV-1 |
| TS-2 | Perform a legitimate authorised address/parameter change + reboot; check whether any retained, time-stamped evidence is generated | Effectiveness (or not) of the evidence function under normal use | Threat-mitigation verification | SVV-2 |
| TS-3 | Perform an illegitimate intervention avoiding visible seal damage (careful seal/sleeve removal, rotary change incl. → 999), reboot; verify no tamper record and that cold start clears the log | Proves G-1 & G-3: unattributable + anti-forensic clearing | MITRE ATT&CK for ICS T0872 Indicator Removal on Host; OWASP ISTG-PHY; EDR 3.11 detection limb | SVV-4 (physical, PA-4) |
| TS-4 | Time-attribution analysis: capture two events across a cold start; show uptime resets and no absolute time exists | Proves G-2: evidence is not time-referenceable (CR 2.11 unmet) | NIST SP 800-115 §3.2 (log review) + §7 (evidence handling) | SVV-3 |
| TS-5 | Attempt to invoke the fused diagnostic/FIT function in production (/fit/setfit, WO/P); confirm rejection and that the exclusion produces no log entry | Validates EDR 2.13 exclusion; confirms G-4 (no evidence) | NIST SP 800-115 §4.2; OWASP FSTM / ISTG-PHY | SVV-3 |

Environment binding: TS-1/TS-2/TS-4/TS-5 use the management path (Pi eth1 = 192.168.50.10 → IoT-Core; browser 192.168.50.20) and OT path as needed; TS-3 requires invasive physical access (PA-4) and is an isolated-lab, PLC-monitored activity only. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog

Full entries for the baseline (TS-1), the primary physical gap-proof (TS-3), and the time-attribution gap-proof (TS-4). TS-2/TS-5 follow the same template on request.

TC-RQ003-01 — Baseline: What Evidence the Device Actually Collects

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ003-01 |
| Traceability | RQ-003 (EHSR 1.1.9 §2) → CR 2.8 / CR 6.1 → Asset I |
| Target Threat | Repudiation (baseline of available evidence) |
| SVV Category | SVV-1 (Requirements-based Testing) |
| Objective | Establish which intervention-evidence mechanisms exist and are readable: the COM Error Log, the I&M0 REVISION_COUNTER, and the I&M4 parameter signature — confirming CR 6.1 (readability) is met while characterising the content's limitations. |
| Abstract Steps | 1. Read the audit/error log and the I&M records at a known-good baseline. 2. Perform one authorised parameter change. 3. Re-read; confirm REVISION_COUNTER incremented and I&M4 signature changed. 4. Inspect each record for the presence of identity and absolute timestamp. |
| Pass/Fail | PASS iff the log and I&M records are readable and the REVISION_COUNTER/I&M4 reflect the change (CR 6.1 + partial CR 2.8 confirmed). Documented shortfall iff no record carries a user identity or absolute timestamp (feeds CR 2.11/2.12 findings). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ003-01 |
| HW/Network | DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10; browser 192.168.50.20; F-Host on OT path |
| Tools | curl / IoT-Core visualizer; TIA Portal for the authorised change |
| Commands |  |

# TC-RQ003-01 Baseline evidence enumeration via IoT-Core

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC01_log_base.json

curl -s http://192.168.0.2/deviceinfo/ -o TC01_iandm_base.json # incl. I&M0 REVISION_COUNTER

curl -s http://192.168.0.2/safecom/f_address -o TC01_faddr_base.json

# --- perform one authorised iPar change in TIA, download, then: ---

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC01_log_after.json

curl -s http://192.168.0.2/deviceinfo/ -o TC01_iandm_after.json

diff TC01_iandm_base.json TC01_iandm_after.json | tee TC01_iandm_diff.txt # expect REVISION_COUNTER +1

| Result | ☐ Readable (CR 6.1 met) ☐ REVISION_COUNTER incremented ☐ No identity/timestamp present | | Evidence | TC01_*.json, TC01_iandm_diff.txt, IoT-Core screenshots |

TC-RQ003-03 — Illegitimate Physical Intervention & Anti-Forensic Clearing (G-1 / G-3 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ003-03 |
| Traceability | RQ-003 → EDR 3.11 (detection) / CR 2.8 / CR 2.12 / CR 3.9 → Asset G, Asset I |
| Target Threat | Repudiation (+ anti-forensic Tampering on audit data) |
| SVV Category | SVV-4 (Penetration Testing — physical, PA-4) |
| Objective | Demonstrate that (a) a physical intervention avoiding visible seal damage produces no automatic tamper record and no attribution, and (b) a subsequent cold start clears any incidental log content — proving the device cannot collect retained, protected evidence of intervention. |
| Abstract Steps | 1. Capture baseline log/I&M/address. 2. Remove seal/sleeve carefully; change the rotary switch (incl. → 999 to enter update mode); note any live status change. 3. Cold-restart to apply the change. 4. Re-read the log and check for any tamper/intervention entry; check whether the log survived the cold start. 5. Assess attribution (identity) and timing (absolute timestamp) of anything found. |
| Pass/Fail | Gap-characterisation test. Expected: no tamper-specific entry is generated; any incidental entries are lost on cold start; no user identity and no absolute timestamp are present. This confirms G-1 (no detection/attribution) and G-3 (unprotected, non-retained evidence) → FAIL against RQ-003 is recorded. A device that did produce a persistent, attributed, time-stamped tamper record would indicate an undocumented capability to investigate. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ003-03 |
| HW/Network | Physical access to DUT 192.168.0.2; mgmt path Pi eth1 = 192.168.50.10; browser 192.168.50.20 |
| Tools | Manual (seal removal, switch actuation); camera; curl to IoT-Core |
| Commands |  |

# TC-RQ003-03 Physical tamper -> evidence check across cold start

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC03_log_before.json

curl -s http://192.168.0.2/safecom/f_address -o TC03_addr_before.json

curl -s http://192.168.0.2/devicestatus/operatingstate -o TC03_state_before.json

# --- remove seal, change rotary switch (e.g. addr change; then 999), COLD RESTART ---

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC03_log_after.json

curl -s http://192.168.0.2/safecom/f_address -o TC03_addr_after.json

diff TC03_log_before.json TC03_log_after.json | tee TC03_logdiff.txt # expect: empty/cleared, no tamper event

| Expected | No tamper entry; log empty after cold start (circular, non-safety QM, cleared on cold start); address changed with no attribution/time. | | Result | ☐ No evidence collected (G-1/G-3 confirmed) ☐ Evidence found (investigate) | | Evidence | Seal before/after photos, TC03_log_before/after.json, TC03_logdiff.txt, update-mode LED photo, address diff | [209_SRIO_C...e_RemoteIO | PDF], [100_SRIO_P...e_RemoteIO | PDF]

TC-RQ003-04 — Time-Attribution Failure of Collected Evidence (G-2 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ003-04 |
| Traceability | RQ-003 → CR 2.11 / CR 3.9 → Asset I |
| Target Threat | Repudiation (loss of temporal attribution) |
| SVV Category | SVV-3 (Vulnerability Testing — log/time review) |
| Objective | Demonstrate that even the evidence the device does record cannot be placed in absolute time: the device exposes only an uptime/systick reference that resets on cold start, with no synchronised real-time clock — so interventions separated by a power cycle cannot be temporally ordered or dated. |
| Abstract Steps | 1. Generate a loggable event; read systick/uptime and the log entry. 2. Cold-restart. 3. Generate a second event; read systick/uptime again. 4. Compare: show the time base reset and that no absolute timestamp is attached to either entry. |
| Pass/Fail | Gap-characterisation test. Expected: uptime/systick resets to ~0 after cold start; no absolute date/time is present in any record → CR 2.11 unmet, evidence is not time-referenceable → FAIL against RQ-003. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ003-04 |
| HW/Network | DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10 |
| Tools | curl to IoT-Core (/systemtime/systick, error log); NIST SP 800-115 §3.2 log-review method |
| Commands |  |

# TC-RQ003-04 Demonstrate absence of absolute time / reset on cold start

curl -s http://192.168.0.2/systemtime/systick -o TC04_tick_1.json

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC04_log_1.json

# --- trigger a loggable event (e.g. brief SCPU/comms interruption), then COLD RESTART ---

curl -s http://192.168.0.2/systemtime/systick -o TC04_tick_2.json # expect reset toward 0

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC04_log_2.json

| Expected | tick_2 << tick_1 (counter reset); log entries carry only relative uptime, no absolute timestamp (SRIO-15218/15398). | | Result | ☐ Time base resets, no absolute timestamp (G-2 confirmed) | | Evidence | TC04_tick_1/2.json, TC04_log_1/2.json, log-review note per NIST SP 800-115 §3.2 | [209_SRIO_C...e_RemoteIO | PDF]

Summary of My Corrections to Your Input

- Mapping under-scoped: an evidence requirement needs the full audit family → add CR 2.9, CR 2.10, CR 2.11 (decisive), CR 3.9, CR 6.1, plus EDR 2.13 for the debug/test port. Note CR 6.1 is the one sub-capability that is met (log is IoT-readable). [209_SRIO_C...e_RemoteIO | PDF], [IEC62443_4_2_source | Txt]

- You understated existing controls: the device does collect partial evidence — I&M0 REVISION_COUNTER, I&M4 signature, and the COM Error Log. A rigorous thesis must name these and then show why each fails (no identity, no time, cold-start cleared). [209_SRIO_C...e_RemoteIO | PDF]

- The killer gap you missed is time (G-2): no RTC; uptime resets on cold start → collected evidence is not time-attributable and is erasable by a power cycle (anti-forensic Tampering, MITRE T0872). [209_SRIO_C...e_RemoteIO | PDF]

- STRIDE: Repudiation is right; add the Tampering-on-audit-data vector (cold-start clearing) to justify CR 3.9.

- Debug port: your reading is correct — fusing is an EDR 2.13 design exclusion, not evidence; the residual (a re-probe generates no log entry) stands. [100_SRIO_P...e_RemoteIO | PDF]

Would you like me to (a) complete TC-RQ003-02 (SVV-2, legitimate-intervention evidence) and TC-RQ003-05 (SVV-3, fused diagnostic-interface exclusion), (b) compile RQ-001 → RQ-003 into a single formatted Word/PDF thesis appendix with a consolidated traceability matrix, or (c) proceed to RQ-004?

REQUIREMENT 4

RQ-004 — Reprocessed (Scope-Corrected)

Correction applied: Rechecking RQ-004 against the same scope-boundary discipline, I found one genuine scope error and one debatable over-reach in my original analysis:

- Scope error (the RQ-002-analogous one): I cited the I&M0 REVISION_COUNTER as a "data identification" mechanism. That is wrong for the same reason the audit material was wrong in RQ-002 — REVISION_COUNTER increments on modification; it is modification-evidence, which belongs to RQ-009, not identification. It counts that the config changed, not what the config is. Removed from RQ-004 and returned to RQ-009.

- Over-reach: I mapped CR 3.4 (identification integrity) + a Spoofing threat + a standalone MITM test (TC-RQ004-04). On reflection, protecting the identification data against falsification is the substance of RQ-005 ("protect identified software and data against corruption"). RQ-004 is a pure identify requirement. I have demoted this to a cross-reference to RQ-005 rather than a primary RQ-004 mapping/test.

What survives is the clean, two-limb requirement the device text and your own submission framed: completeness of identification (CR 7.8) + confidentiality of the identification data (CR 4.1).

Step 1 — Critical Review & Theoretical Validation

1.1 Requirement & CR Mapping

EU Machinery Regulation (EU) 2023/1230 identification obligation — identify the software and data critical for EHSR compliance. Pure inventory/identification requirement.

| CR | Your input | Verdict | Correction |
| --- | --- | --- | --- |
| CR 7.8 – Component inventory | ✅ | Correct primary | Keep. Modern CR 7.8 = SBOM-grade inventory → drives the decisive gap G-2. |
| CR 4.1 – Confidentiality of information | ✅ | Correct | Keep. Add CR 4.3 — no HTTPS (SRIO-7907), so no transport confidentiality for the identification data. |
|  | — | Demoted → RQ-005 | Protecting the identification/asset against falsification is RQ-005's substance. Cross-referenced, not mapped here. |
| IEC 62443-4-1 SM-9 / SM-10 | ➕ note | Process anchor | The SBOM/third-party-component obligation anchors in the 4-1 secure-development requirements. |

Corrected mapping: RQ-004 → EHSR identification obligation →{CR 7.8 [inventory/SBOM — primary], CR 4.1 (+CR 4.3) [confidentiality]}, with 4-1 SM-9/SM-10 as the SBOM process anchor.(Identification-integrity/spoofing → see RQ-005.) [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset B – Config (data) | ✅ | Correct — and partially identified via the I&M4 fingerprint (see §1.3), which corrects your "not addressed." |
| Asset D – Safety Firmware | ✅ | Correct primary. |
|  | — | Dropped as primary. The domain-crossing was only relevant to the identity-integrity concern now cross-referenced to RQ-005. Retain only as a one-line note if desired. |

STRIDE — corrected:

- ✅ Information Disclosure (primary). The observable behaviour of RQ-004 is emitting identification data; the always-present threat is reconnaissance (exact HW/FW/stack versions → CVE mapping). This is the correct primary, and matches your own secondary gap elevated to its proper place.

- — Cross-referenced to RQ-005 (protection of identified data). Not a primary RQ-004 threat.

1.3 Security Functions & Gaps — Cross-Referenced

Identification surface (software and data), corrected to exclude REVISION_COUNTER:

| Mechanism | Documentation evidence | Identifies |
| --- | --- | --- |
| /deviceinfo/hwversion, hwrevision, swrevision, swinfo/{cpubootloader, scpu, scpubootloader}version | SRIO-2946 | Software / HW (Asset D) [209_SRIO_C...e_RemoteIO ¦ PDF] |
| I&M0 (MANUFACTURER_ID, ORDER_ID, SERIAL, HW/SW revision) | SRIO-3002/3015 | Device + firmware identity (Asset D) [209_SRIO_C...e_RemoteIO ¦ PDF] |
| I&M5 annotation ("host_fw_1.0.3.215") | SRIO-3023 | Host firmware (Asset D) [209_SRIO_C...e_RemoteIO ¦ PDF] |
| /firmware/version, /firmware/type | SRIO-2953 | Update-container firmware (Asset D) [209_SRIO_C...e_RemoteIO ¦ PDF] |
| /fieldbussetup/fieldbusfirmware ("3.3.0.12 (PROFIsafe IO Device)") | SRIO-2948 | PROFIsafe stack (Asset D) [209_SRIO_C...e_RemoteIO ¦ PDF] |
| I&M4 signature = F_iParCRC + F_Par_CRC | SRIO-3011 | The safety parameter set (Asset B) — data identification, as a fingerprint [209_SRIO_C...e_RemoteIO ¦ PDF] |
|  | SRIO-3002 | REMOVED — modification count, belongs to RQ-009 |

Gaps (corrected, in-scope only):

- G-1 (data identification — corrected, REVISION_COUNTER removed). Your "config/data is not addressed" is incorrect, but the correct evidence is only the I&M4 F_iParCRC/F_Par_CRC signature — a fingerprint that identifies which configuration is loaded. It is not the REVISION_COUNTER (that counts changes = RQ-009). So the accurate gap is: the data is identified only by an opaque CRC fingerprint, not by a content-level, human-/machine-readable classification of which data elements are EHSR-critical (safe data mapping, iPar, F-Par). RQ-004 is therefore partially met for data — via fingerprint, not enumeration. [209_SRIO_C...e_RemoteIO | PDF]

- G-2 (SBOM — decisive, in scope). CR 7.8 expects identification of the software genuinely critical for EHSR compliance, including third-party components: µC/OS-II (Micrium) (SRIO-1260), the SIL3 PROFIsafe stack (SRIO-1929), the STM32F7 self-test library (SRIO-10766), the netX90 PROFINET stack (SRIO-1768). The device exposes only top-level version strings and no SBOM → these cannot be inventoried or CVE-correlated. Strongest, most citable gap (OWASP ISVS V1.1.1; BSI TR-03183; 4-1 SM-9/SM-10). [209_SRIO_C...e_RemoteIO | PDF], [ifmworld-m...epoint.com], [ICS-Securi...endium_pdf | PDF]

- G-3 (confidentiality — in scope, your secondary gap). All identification served over plaintext HTTP (SRIO-7903), HTTPS/WebSocket/MQTT unsupported (SRIO-7907/7906/7905) → version reconnaissance on the Ethernet interface. CR 4.1/4.3 not met. [209_SRIO_C...e_RemoteIO | PDF]

- — Moved to RQ-005 (protection of identified software/data). Cross-reference only.

SL/conformance conclusion (identification-scoped): RQ-004 is partially met. Software identification is provided and readable (CR 7.8 partial); data is identified by CRC fingerprint (I&M4) but not content-classified (G-1); no SBOM-grade third-party inventory (G-2, decisive); no transport confidentiality for the identification data (G-3). The integrity/authenticity of the identification is assessed under RQ-005, not here.

Step 2 — Methodological Test Planning (Scope-Corrected)

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Enumerate the identification surface (/deviceinfo/*, I&M0, I&M5, /firmware/version, /fieldbussetup/fieldbusfirmware, I&M4 for data); build the actual inventory and diff vs. expected EHSR-critical SW+data | Functional CR 7.8; confirms SW + data (I&M4 fingerprint) identification; exposes SBOM gap | Requirements-based verification; OWASP FSTM Stage 1 | SVV-1 |
| TS-2 | SBOM-completeness / version→CVE analysis: attempt to derive third-party components (µC/OS-II, PROFIsafe stack, STM STL, netX90); show no SBOM → no CVE correlation | Proves G-2 | OWASP FSTM "SBOM Generation" + ISVS V1.1.1; ifm CM-008 | SVV-3 |
| TS-3 | Passive plaintext capture of the identification traffic; confirm cleartext versions + no TLS | Proves G-3 (confidentiality) | NIST SP 800-115 §3.5 (sniffing); OWASP ISTG-DES-INFO; ifm CM-005 | SVV-3 |
| TS-4 | Data-identification fingerprint check: read I&M4 for config A; load a different config B; read I&M4; confirm the signature uniquely characterises the loaded config (identification limb) | Confirms the partial data-identification (corrects G-1) — as identification, not modification | Requirements-based verification | SVV-1 |
|  | — | Moved to RQ-005 (protection). Cross-referenced. | — | — |

Environment binding: TS-1/TS-2/TS-4 over the management path (Pi eth1 = 192.168.50.10 → IoT-Core; browser 192.168.50.20); TS-3 passive on the laptop mirror. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog (Scope-Corrected)

Full entries for the inventory baseline (TS-1) and the decisive SBOM proof (TS-2). TS-3 executes per the plaintext-recon method; TS-4 is a short identification-fingerprint confirmation. The MITM identity-falsification case is removed from RQ-004 and belongs to RQ-005.

TC-RQ004-01 — Identification-Surface Inventory (SW + Data)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ004-01 |
| Traceability | RQ-004 (EHSR identification obligation) → CR 7.8 → Asset D (SW) + Asset B (data via I&M4) |
| Target Threat | Information Disclosure / incomplete identification |
| SVV Category | SVV-1 (Requirements-based Testing) |
| Objective | Enumerate the complete identification set the device exposes; confirm it identifies safety software (firmware/bootloader/stack versions) and safety data (parameter-set fingerprint via I&M4); diff against the expected EHSR-critical SW+data set to reveal the SBOM completeness gap. |
| Abstract Steps | 1. Retrieve all /deviceinfo/* version fields. 2. Read I&M0, I&M4 (signature), I&M5. 3. Read /firmware/version, /fieldbussetup/fieldbusfirmware. 4. Compile the inventory; compare against the expected set (incl. third-party components). |
| Pass/Fail | PASS (partial-conformance baseline) iff the device returns top-level SW identification and a data-identification fingerprint (I&M4). Documented shortfall iff no SBOM of third-party components is present and no explicit safety-critical-data classification is exposed. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ004-01 |
| HW/Network | DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10; browser 192.168.50.20 |
| Tools | curl / IoT-Core visualizer; PROFINET acyclic read for I&M0/4/5 |

# TC-RQ004-01 Full identification-surface enumeration (REVISION_COUNTER excluded)

for ep in deviceinfo/hwversion deviceinfo/hwrevision deviceinfo/swrevision </span>

deviceinfo/swinfo/cpubootloaderversion deviceinfo/swinfo/scpuversion </span>

deviceinfo/swinfo/scpubootloaderversion firmware/version firmware/type </span>

fieldbussetup/fieldbusfirmware ; do

echo "== $ep =="; curl -s "http://192.168.0.2/$ep";

done | tee TC01_swid.txt

# I&M0 (identity), I&M4 (parameter-set FINGERPRINT = F_iParCRC + F_Par_CRC), I&M5 (host fw) via acyclic PN read

# NOTE: REVISION_COUNTER is NOT read here — it is modification evidence (RQ-009), not identification.

| Expected | SW versions returned; I&M4 = F_iParCRC+F_Par_CRC (data fingerprint); no SBOM of µC/OS-II, PROFIsafe stack, STM STL, netX90. | | Result | ☐ SW identified ☐ Data fingerprint (I&M4) present ☐ No SBOM (G-2) | | Evidence | TC01_swid.txt, I&M0/4/5 read logs, inventory-vs-expected diff table | [209_SRIO_C...e_RemoteIO | PDF]

TC-RQ004-02 — SBOM Completeness & Third-Party CVE-Correlation Gap (G-2 proof — decisive)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ004-02 |
| Traceability | RQ-004 → CR 7.8 (SBOM limb) / 4-1 SM-9-SM-10 → Asset D |
| Target Threat | Information Disclosure (incomplete inventory → un-managed vulnerability) |
| SVV Category | SVV-3 (Vulnerability Testing) |
| Objective | Demonstrate that the exposed inventory is top-level only and provides no SBOM of the safety-critical third-party components, so their versions cannot be enumerated or CVE-correlated — CR 7.8 met at product granularity but not at component granularity. |
| Abstract Steps | 1. From TC-RQ004-01, list all device-exposed version strings. 2. From documentation, list the known constituent third-party components. 3. Attempt to derive each component's version from any exposed interface. 4. For derivable versions, query a vulnerability database (off-lab); record the rest as un-inventoried. |
| Pass/Fail | Expected = FAIL: third-party components are not individually exposed and no SBOM exists → confirms G-2; documented as a CR 7.8 completeness shortfall requiring an SBOM (BSI TR-03183 / ISVS V1.1.1). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ004-02 |
| HW/Network | DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10; CVE lookup performed off-lab (no lab↔Internet coupling) |
| Tools | curl (inventory); manual coverage table; NVD lookup off-lab; optional GVM/OpenVAS (ifm CM-008) on the HTTP surface only |

# TC-RQ004-02 SBOM-completeness assessment (device side = read-only enumeration)

curl -s http://192.168.0.2/fieldbussetup/fieldbusfirmware -o TC0402_stack.json # PROFIsafe stack ver

# Coverage table: exposed-version vs known constituent (NOT exposed by device):

# uC/OS-II (Micrium) [SRIO-1260] -> NOT exposed

# PROFIsafe stack (SIL3) [SRIO-1929] -> partially via fieldbusfirmware

# STM32F7 Self-Test Library [SRIO-10766] -> NOT exposed

# netX90 PROFINET stack [SRIO-1768] -> NOT exposed

# derivable versions -> off-lab NVD query; rest -> "un-inventoried"

| Expected | Only the PROFIsafe stack version partially derivable; RTOS/STL/netX90 versions not exposed → no SBOM, no third-party CVE correlation. | | Result | ☐ SBOM absent (G-2 confirmed) — un-inventoried: ______ | | Evidence | TC0402_stack.json, exposed-vs-constituent coverage table, off-lab NVD notes | [209_SRIO_C...e_RemoteIO | PDF], [ifmworld-m...epoint.com], [ICS-Securi...endium_pdf | PDF]

TC-RQ004-04 — Data-Identification Fingerprint Uniqueness (G-1 correction, identification-scoped)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ004-04 |
| Traceability | RQ-004 → CR 7.8 (data limb) → Asset B |
| Target Threat | Incomplete identification (data) |
| SVV Category | SVV-1 (Requirements-based Testing) |
| Objective | Confirm that the safety data is identified: the I&M4 signature (F_iParCRC + F_Par_CRC) uniquely characterises the loaded safety configuration, so that different configurations yield different fingerprints — establishing that data is identified (as a fingerprint), correcting the "data not addressed" premise. This is an identification test (does the fingerprint identify the config?), not a modification-evidence test. |
| Abstract Steps | 1. Load configuration A; read I&M4. 2. Load a materially different configuration B; read I&M4. 3. Confirm the two fingerprints differ and each uniquely maps to its configuration. |
| Pass/Fail | PASS iff distinct configurations produce distinct I&M4 fingerprints (data identified via fingerprint). Documented limitation: the fingerprint identifies that a specific config is loaded but does not enumerate/classify which data elements are EHSR-critical (G-1 residual). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ004-04 |
| HW/Network | SPS 192.168.0.1, DUT 192.168.0.2; Pi mgmt path |
| Tools | TIA Portal (load configs A/B); PN acyclic read for I&M4 |
| Procedure | Read I&M4 (config A) → load config B → read I&M4; compare F_iParCRC/F_Par_CRC values |
| Expected | I&M4(A) ≠ I&M4(B); each fingerprint uniquely characterises its config (SRIO-3011). [209_SRIO_C...e_RemoteIO ¦ PDF] |
| Result | ☐ Fingerprint unique per config (data identified) ☐ Content-level classification absent (G-1 residual) |
| Evidence | I&M4 reads for A and B, config-vs-fingerprint mapping table |

Summary of Changes (vs. my original RQ-004)

- Removed REVISION_COUNTER as a "data identification" mechanism — it is a modification counter = RQ-009 (modification evidence), not identification. This is the direct analogue of the RQ-002 correction. [209_SRIO_C...e_RemoteIO | PDF]

- Data identification now rests solely on the I&M4 fingerprint (F_iParCRC + F_Par_CRC), reframed as identification-by-fingerprint (not content-level classification) — G-1 corrected without importing modification-evidence. [209_SRIO_C...e_RemoteIO | PDF]

- Demoted CR 3.4 / Spoofing / MITM identity-falsification — protecting the identification against corruption is RQ-005. Removed the primary CR 3.4 mapping, the Spoofing primary threat, Asset F as primary, and TC-RQ004-04 (MITM); these are now cross-referenced to RQ-005.

- STRIDE simplified to Information Disclosure (primary) — the correct always-present threat for an identification requirement; tampering/spoofing of identity cross-referenced out.

- Retained the two genuine RQ-004 limbs and their strongest findings: G-2 (SBOM completeness — CR 7.8) and G-3 (confidentiality — CR 4.1/4.3), both in scope. [209_SRIO_C...e_RemoteIO | PDF], [ifmworld-m...epoint.com]

The corrected verdict is tighter and non-overlapping: RQ-004 = identify (completeness, incl. SBOM gap) + keep that identification confidential — with integrity/protection of the identification cleanly deferred to RQ-005, and change-tracking deferred to RQ-009.

Would you like me to (a) apply the same scope-recheck to RQ-006 (it reused the identity-integrity/MITM material and should be realigned the same way), (b) re-check RQ-005 to confirm it now correctly receives the identification-integrity concern, or (c) proceed to consolidation?

REQUIREMENT 5

Step 1 — Critical Review & Theoretical Validation

RQ-005 — Reprocessed (Scope-Corrected)

Correction applied: Rechecking RQ-005 against the same scope-boundary discipline, I found the mirror-image error to RQ-004. RQ-004 correctly hands the identification-integrity concern to RQ-005; but in doing so I must also make sure RQ-005 does not itself reach into RQ-011's territory (authorization / access control). On review, my original RQ-005 did exactly that:

- I added an Elevation-of-Privilege threat and analysed the physical update gate (rotary switch = 999 + Update state) as an RQ-005 mitigation, and wrote TC-RQ005-05 (Firmware-Update Access-Control Gate). That is authorization — "who is allowed to update" — which is the substance of RQ-011 ("prevent modifications… where such modifications could lead to a hazardous situation"). It has been removed from RQ-005 and returned to RQ-011.

- Asset G (operating-mode/switch) was likewise imported as the access-control trigger → demoted to a cross-reference (its security-relevant analysis is authorization = RQ-011).

RQ-005 is a pure integrity / authenticity requirement: is the identified software and data protected against corruption (accidental or intentional)? — not who is authorized to change it. The corrected analysis keeps the integrity core (CRC-for-data, secure-boot/update-signature-for-firmware) and cleanly excludes the authorization material.

Step 1 — Critical Review & Theoretical Validation

1.1 Requirement & CR Mapping

EU Machinery Regulation (EU) 2023/1230, Annex III EHSR 1.1.9 §1, applied to the identified assets from RQ-004. Pure integrity / authenticity requirement across two limbs: Part B (data/config at rest) and Part D (firmware image / boot).

| CR / EDR | Your input | Verdict | Correction |
| --- | --- | --- | --- |
| CR 3.4 – Software & Information Integrity | ✅ | Correct — Part B | Keep. |
| EDR 3.14 – Integrity of boot process | ✅ | Correct — Part D | Keep — unmet (bootloaders report versions but no boot-image signature verification is documented). |
| EDR 3.10 – Update integrity/authenticity verification | ➕ Add | Decisive | The intentional-corruption vector for firmware is the update path: the device installs an unsigned BLOB (SRIO-2953/7915). EDR 3.10's signature-verification limb is the RQ-005 concern.(EDR 3.10's authorization limb — who may update — is RQ-011.) |
| EDR 3.12 / 3.13 – Manufacturer / operator trust anchors | ➕ Add | Enablers | Their absence is why EDR 3.14/3.10 cannot be met — no on-device root of trust. |
| CR 3.3 – Security functionality verification | ➕ Add | Partial credit | STM32F7 self-test library + startup tests give runtime integrity of executing code/RAM. |
|  | — | Removed → RQ-011 | "Who may perform the update" is access control = RQ-011, not integrity. |

Corrected mapping: RQ-005 → EHSR 1.1.9 §1 →{CR 3.4 [data], EDR 3.14 [boot], EDR 3.10 [update signature verification — decisive], EDR 3.12/3.13 [trust anchors], CR 3.3 [runtime self-test]}.(Update authorization / access gate → RQ-011.) [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset B – Runtime integrity (config/data) | ✅ | Correct. |
| Asset D – Image integrity (firmware) | ✅ | Correct primary. |
| Asset F – Domain separation | ✅ (I added) | Keep — the update BLOB and version reporting traverse the non-safety COM; a compromised COM is the delivery path for a corrupted image (integrity concern). |
|  | — | Demoted → RQ-011. The switch is the authorization gate; its security analysis is access control, not integrity. Cross-reference only. |

STRIDE — corrected to integrity/authenticity only:

- ✅ Tampering (primary) — recompute-CRC over modified parameters (Part B); modified firmware image (Part D).

- ✅ Spoofing-of-image-source (authenticity limb, keep). An unsigned update means the device cannot verify the authenticity of the firmware image — a rogue/MITM image is indistinguishable from the legitimate one. This is an authenticity/integrity failure (does the image come from a trusted source and is it unmodified?), correctly in RQ-005. MITRE ATT&CK for ICS T0857 System Firmware.

- — Removed. "Installing firmware requires which privilege / passing which gate" is access control = RQ-011.(The impact of a corrupted image — total safety compromise — is noted in the RQ-005 gap severity, but the threat category of privilege/authorization is out of scope here.)

1.3 Security Functions & Gaps — Cross-Referenced

Part B mitigation accurately described; Part D corrected (integrity mechanisms only; the switch-gate moved to RQ-011):

| Mechanism | Documentation evidence | Protects (integrity) | Sufficiency |
| --- | --- | --- | --- |
| iParCRC (UINT32) + F_ParCRC (UINT16), verified each parametrization → EC3 | SRIO-12627/8575/8574 | Asset B (data) | Accidental only (G-1) [209_SRIO_C...e_RemoteIO ¦ PDF] |
| iParCRC via externally certified T2 CRC-Tool (TÜV Süd) | SRIO-9023 | Asset B (process) | Process control; no runtime forgery resistance [218_SRIO_M...e_RemoteIO ¦ PDF] |
| Dual SCPU (CPU1+CPU2) cross-check | SRIO-12921 | Asset D (runtime) | Detects divergent corruption; blind to identical modification of both (G-3) [Market_Acc...ty_Systems ¦ PDF] |
| STM32F7 self-test library + startup tests | SRIO-10766, SRIO-1417 | Asset D (runtime code/RAM) | Detects random faults, not intentional signed modification [209_SRIO_C...e_RemoteIO ¦ PDF], [100_SRIO_P...e_RemoteIO ¦ PDF] |
| FW update behind firewall (environmental) | SRIO-1196, SRIO-9402 | Asset D (environment) | Environmental, not device-level integrity [218_SRIO_M...e_RemoteIO ¦ PDF] |
| Firmware update container = unsigned BLOB | SRIO-2953 / SRIO-7915 | Asset D | No signature field → the core gap (G-2) [209_SRIO_C...e_RemoteIO ¦ PDF] |
|  | SRIO-2013/11534 | — | Moved → RQ-011 (authorization). Not an integrity protection. |

Gaps (integrity-scoped):

- G-1 (Part B — accurate, keep). iParCRC/F_ParCRC are checksums, not MACs; the seed/layout is public and F_ParCRC is only 16-bit → an attacker who recomputes matching CRCs over modified parameters is not detected. Protection against accidental but not intentional corruption. Same class as RQ-002.(prEN 50742 §7.4.3.4.1 SRSL3 — present as your benchmark; I cannot independently verify that draft clause.) [209_SRIO_C...e_RemoteIO | PDF]

- G-2 (Part D — decisive, integrity-scoped). The firmware update installs an unsigned BLOB with no signature verification (EDR 3.10) and no secure boot (EDR 3.14), because there is no on-device trust anchor (EDR 3.12/3.13). Intentional firmware corruption is therefore not prevented by any device-level cryptographic integrity/authenticity mechanism — the only remaining barriers are the environmental firewall assumption and (as authorization, per RQ-011) the physical gate. Fully citable (OWASP ISTG-FW[UPDT]-CRYPT-001/004; FSTM firmware-integrity). [209_SRIO_C...e_RemoteIO | PDF], [OWASP IoT...ting Guide | PDF], [ifmworld-m...epoint.com]

- G-3 (runtime blind spot — keep). Dual-SCPU cross-check + STL detect divergent/random faults but are blind to a coherent modification applied identically to both channels — exactly what a signed-image mechanism would catch. This is why self-tests ≠ authenticity. [Market_Acc...ty_Systems | PDF]

Cross-reference note (not re-imported): the authenticity of reported identification data (unsigned HTTP version strings, MITM-falsifiable) is weak — but that is a reporting-integrity issue tied to RQ-004's confidentiality/identification limb, not a corruption of the software itself. Noted, not tested here.

SL/conformance conclusion (integrity-scoped): RQ-005 is partially met. Part B reaches integrity at SL 1 (accidental), not the intentional-corruption level (no MAC). Part D has runtime/redundancy protection (dual-SCPU + STL, detecting random/divergent faults) but no cryptographic authenticity: EDR 3.10, EDR 3.14, EDR 3.12/3.13 unmet → intentional firmware corruption is not prevented at the device level. Net: protection against accidental corruption achieved; protection against intentional corruption not achieved — with the authorization of updates assessed separately under RQ-011.

Step 2 — Methodological Test Planning (Scope-Corrected)

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Verify iParCRC/F_ParCRC rejection of a mismatched configuration (references TC-RQ002-01) | Functional CR 3.4 in normal operation | Requirements-based verification | SVV-1 |
| TS-2 | Recompute valid iParCRC/F_ParCRC over modified parameters; inject → accepted | Proves G-1 (Part B: intentional corruption undetected) | MITRE ATT&CK for ICS T0836 Modify Parameter; NIST SP 800-115 §5.2 | SVV-4 |
| TS-3 | Upload a modified/unsigned firmware BLOB; confirm no signature/integrity verification before install | Proves G-2 (Part D: no update authenticity — decisive) | OWASP ISTG-FW[UPDT]-CRYPT-001/004; OWASP FSTM Stage 7; MITRE T0857 | SVV-4 |
| TS-4 | Obtain the update image/BLOB; static analysis for signature/header + binary-hardening review | Characterises absence of signing / secure-boot artefacts | OWASP FSTM Stage 3–5; ifm CM-008 | SVV-3 |
|  | — | Moved → RQ-011 (authorization) | — | — |

(Note: entering Update state — switch 999 + reboot — is a precondition for TS-3, executed operationally; whether that gate correctly restricts who may update is tested under RQ-011, not here. TS-3 tests only whether the image is verified once accepted.)

Environment binding: TS-2 side-inject on Pi eth0; TS-3/TS-4 over the management path. TS-3 modifies safety firmware → isolated-lab, PLC-monitored, spare-DUT only; re-flash with a validated image afterward. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog (Scope-Corrected)

Full entries for the Part-B forgery proof (TS-2) and the decisive Part-D unsigned-firmware proof (TS-3). TS-1 references TC-RQ002-01; TS-4 follows the FSTM template on request. TC-RQ005-05 (access-control gate) removed → RQ-011.

TC-RQ005-02 — Valid-CRC Forgery over Modified Parameters (Part B / G-1 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ005-02 |
| Traceability | RQ-005 → CR 3.4 → Asset B |
| Target Threat | Tampering (intentional) |
| SVV Category | SVV-4 (Penetration Testing / protocol manipulation) |
| Objective | Demonstrate that because iParCRC/F_ParCRC are non-secret checksums, an attacker who modifies the safety parameter set and recomputes matching CRCs produces a configuration the SCPU accepts — proving protection against accidental but not intentional corruption. |
| Abstract Steps | 1. Capture/derive a legitimate parameter record + F-Parameter block. 2. Modify a safety-relevant iPar. 3. Recompute iParCRC (UINT32) and F_ParCRC (UINT16). 4. Present the forged configuration; observe whether the SCPU accepts it (reaches OPERATE) or raises EC3. |
| Pass/Fail | Gap-characterisation test. Expected = ACCEPTED (no EC3) → evidences G-1; residual risk mitigated only by the environmental firewall assumption, requiring a cryptographic-signature upgrade for full conformance. A device that rejected the recomputed-CRC set indicates an undocumented check to investigate. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ005-02 |
| HW/Network | SPS 192.168.0.1, DUT 192.168.0.2; Pi eth0 = 192.168.0.100 (inject); laptop mirror |
| Tools | Scapy + iParCRC/F_ParCRC implementation (certified CRC-Tool spec); Wireshark |

#!/usr/bin/env python3

# TC-RQ005-02 Forge iParCRC/F_ParCRC over modified safety parameters

# Precondition: CRC routine verified to reproduce a captured legitimate record bit-for-bit.

def ipar_crc32(params: bytes, seed: int) -> int: ... # per certified CRC-Tool polynomial

def fpar_crc16(fparams: bytes) -> int: ... # 16-bit F-Parameter CRC

params = bytearray(open("ref_ipar.bin","rb").read())

params[7] = 0x02 # e.g. Filter F-DI 1: 5ms -> 10ms (safety-relevant)

new_ipar = ipar_crc32(bytes(params), SEED).to_bytes(4,"big")

# write new_ipar into F-Parameter byte 9..12, recompute F_ParCRC, inject acyclic write (param phase)

print("[+] forged parameter set with valid recomputed iParCRC/F_ParCRC prepared")

| Result | ☐ Accepted (G-1 confirmed) ☐ EC3 raised (investigate) | | Evidence | TC05-02_forge.pcapng, CRC-routine validation log, PLC state/diagnostic export |

TC-RQ005-03 — Unsigned / Modified Firmware Installation (Part D / G-2 proof — decisive)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ005-03 |
| Traceability | RQ-005 → EDR 3.10 / EDR 3.14 (/ EDR 3.12-3.13) → Asset D, Asset F |
| Target Threat | Tampering + Spoofing-of-image-source (authenticity) |
| SVV Category | SVV-4 (Penetration Testing) |
| Objective | Demonstrate that the firmware-update mechanism installs an update without cryptographic authenticity/integrity verification: a modified/unsigned firmware BLOB is accepted for installation — proving the intentional-corruption protection for the safety firmware is absent (only an environmental firewall assumption — and, as authorization, the RQ-011 physical gate — stand between an attacker and a malicious image). |
| Abstract Steps | 1. Place the (spare) DUT in Update state*(operational precondition; the gate's authorization is tested under RQ-011)*. 2. Take a legitimate update container and modify a byte (or strip/forge any signature/header). 3. Upload via the IoT-Core /firmware/container BLOB service and invoke /firmware/install. 4. Observe whether the device verifies the image before install (rejects) or proceeds. |
| Pass/Fail | PASS against the requirement iff the modified/unsigned image is rejected by a signature/integrity check before install. Expected = ACCEPTED for install → confirms EDR 3.10/3.14 unmet (G-2); recorded as a FAIL against RQ-005 for the firmware asset. Safety: spare DUT only; re-flash a validated image afterward. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ005-03 |
| HW/Network | Spare SRIO DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10; browser 192.168.50.20 |
| Tools | curl to IoT-Core firmware profile; hex editor / binwalk (OWASP FSTM) to inspect & modify container; Wireshark |

# TC-RQ005-03 Attempt install of a modified/unsigned firmware container (spare DUT only)

# 0) Device in Update state (operational precondition; authorization of the gate = RQ-011)

# 1) Inspect the legitimate container for any signature/header

binwalk -E fw_container.bin ; binwalk fw_container.bin | tee TC0503_hdr.txt

# 2) Modify one byte (or blank a putative signature region)

cp fw_container.bin fw_mod.bin ; printf '\x00' | dd of=fw_mod.bin bs=1 seek=4096 conv=notrunc

# 3) Upload via IoT-Core BLOB service (chunked) and trigger install

curl -s http://192.168.0.2/firmware/container/start_stream_set

# ...chunked stream_set of fw_mod.bin per container/chunksize...

curl -s -X POST http://192.168.0.2/firmware/install # observe accept vs. signature-reject

| Expected | Container is an unsigned BLOB (SRIO-2953); no signature-verification step → modified image accepted for install (device reboots to apply). | | Result | ☐ Modified image accepted (G-2 confirmed → FAIL vs RQ-005) ☐ Rejected (investigate) | | Evidence | TC0503_hdr.txt, upload/install PCAP, LED sequence photo, post-install version read; re-flash log with validated image | [209_SRIO_C...e_RemoteIO | PDF]

Justification: maps to OWASP ISTG ISTG-FW[UPDT]-CRYPT-001 (insufficient update signature) and**-CRYPT-004** (insufficient signature verification); OWASP FSTM Stage 7 firmware-integrity. [OWASP IoT...ting Guide | PDF], [ifmworld-m...epoint.com]

Summary of Changes (vs. my original RQ-005)

- Removed the authorization/access-control scope creep — dropped the Elevation-of-Privilege threat, the**"physical update gate (switch 999 + Update state)"** as an RQ-005 mitigation, Asset G as primary, and TC-RQ005-05 (Firmware-Update Access-Control Gate). These are access control = RQ-011 and have been returned there.(Mirror of the RQ-004 correction: RQ-004 hands identification-integrity to RQ-005; RQ-005 hands update-authorization to RQ-011.)

- Kept the integrity core intact — CR 3.4 (Part B), EDR 3.14 (boot), EDR 3.10 signature-verification (Part D, decisive), EDR 3.12/3.13 (trust anchors), CR 3.3 (runtime self-test). [209_SRIO_C...e_RemoteIO | PDF]

- Reframed Spoofing as authenticity of the firmware image (integrity — the device cannot verify the image's source/integrity), which is legitimately RQ-005, rather than an access-control bypass.

- Retained the substantive gaps — G-1 (Part B CRC≠MAC, incl. 16-bit F_ParCRC), G-2 (Part D unsigned BLOB / no secure boot / no trust anchor — decisive), G-3 (runtime cross-check blind to identical modification). [209_SRIO_C...e_RemoteIO | PDF], [Market_Acc...ty_Systems | PDF]

- Standard-citation caution retained — prEN 50742 §7.4.3.4.1 (SRSL3) presented as your benchmark, not verified fact.

Corrected verdict, cleanly bounded: RQ-005 = protect the firmware image and config against corruption (accidental met via CRC/redundancy; intentional not met — no cryptographic authenticity) — with who may update deferred to RQ-011 and identify the software/data handled by RQ-004.

Would you like me to (a) re-check RQ-006 for the same identity-integrity/MITM realignment (it reused that material), (b) re-check RQ-011 to confirm it now correctly receives the update-authorization gate + EoP, or (c) proceed to consolidation of the corrected set?

REQUIREMENT 6

RQ-006 — Reprocessed (Scope-Corrected)

Correction applied: RQ-006 reused RQ-004's identity-integrity material, so it inherits the same over-reach I just corrected in RQ-004:

- Identity-integrity / MITM over-reach. My original RQ-006 mapped CR 3.4 (inventory integrity), added a Spoofing/Tampering-of-inventory primary threat, brought in Asset F, and reused TC-RQ004-04 (MITM falsification) as its G-3 proof. Protecting the inventory against falsification is the substance of RQ-005 ("protect identified software and data against corruption"). RQ-006 is a pure identify requirement. All of this is demoted to a cross-reference to RQ-005.

- Scope clarity (a strength to keep). RQ-006 is software-only (it drops RQ-004's "data" limb), so — unlike RQ-004 — CR 7.8 alone genuinely suffices as primary, and there is no I&M4 data-fingerprint concern here. This distinction should be stated explicitly.

What survives is the tight, correct RQ-006 core: identify the installed software necessary for safe operation (completeness, incl. the SBOM gap) + keep that identification confidential.

Step 1 — Critical Review & Theoretical Validation

1.1 Requirement & CR Mapping

EU Machinery Regulation identification obligation, scoped to installed software necessary for safe operation — the software inventory requirement in its purest form.

| CR | Your input | Verdict | Correction |
| --- | --- | --- | --- |
| CR 7.8 – Component inventory | ✅ | Correct — and, unlike RQ-004, sufficient as sole primary | Because RQ-006 excludes "data," CR 7.8 alone carries the requirement. Modern CR 7.8 = SBOM-grade → drives G-1. |
| CR 4.1 – Confidentiality of information | ✅ (in your gap) | Correct | Add to the mapping (your confidentiality point had no CR anchor). Add CR 4.3 — no HTTPS (SRIO-7907). |
|  | — | Demoted → RQ-005 | Protecting the inventory against falsification is RQ-005. Cross-referenced, not mapped here. |
| IEC 62443-4-1 SM-9 / SM-10 | ➕ note | Process anchor | SBOM/third-party-component obligation anchors here. |

Corrected mapping: RQ-006 → EHSR identification obligation (software) →{CR 7.8 [inventory/SBOM — sole primary], CR 4.1 (+CR 4.3) [confidentiality]}, with 4-1 SM-9/SM-10 as the SBOM process anchor.(Integrity/authenticity of the inventory → RQ-005.) [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset D – host FW, SCPU FW, bootloaders | ✅ | Correct and complete for the identified software. |
|  | — | Dropped as primary. The domain-crossing mattered only to the identity-integrity concern now at RQ-005. Optional one-line note (SCPU versions relayed by the non-safety COM), not a primary asset here. |

STRIDE — corrected:

- ✅ Information Disclosure (primary). The observable behaviour of RQ-006 is emitting an installed-software list; the always-present threat is version reconnaissance → CVE mapping. This is your own "secondary" correctly elevated to primary.

- — Cross-referenced to RQ-005 (protection of identified software). Not a primary RQ-006 threat.

1.3 Security Functions & Gaps — Cross-Referenced

Installed-software identification surface (software only — no data fingerprint, correct for RQ-006):

| Mechanism | Documentation evidence | Identifies |
| --- | --- | --- |
| /deviceinfo/swrevision, /swinfo/{cpubootloader, scpu, scpubootloader}version | SRIO-2946 | Host FW + SCPU FW + both bootloaders (Asset D) — your list ✅ [209_SRIO_C...e_RemoteIO ¦ PDF] |
| I&M0 SOFTWARE_REVISION | SRIO-3002/3015 | Firmware revision (Asset D) — your list ✅ [209_SRIO_C...e_RemoteIO ¦ PDF] |
| I&M5 annotation ("host_fw_1.0.3.215") | SRIO-3023 | Host FW string — your list ✅ [209_SRIO_C...e_RemoteIO ¦ PDF] |
| /firmware/version, /firmware/type | SRIO-2953 | Update-container firmware (Asset D) — add [209_SRIO_C...e_RemoteIO ¦ PDF] |
| /fieldbussetup/fieldbusfirmware ("3.3.0.12 (PROFIsafe IO Device)") | SRIO-2948 | PROFIsafe stack — the single most safety-relevant installed software — add [209_SRIO_C...e_RemoteIO ¦ PDF] |

Gaps (identification-scoped):

- G-1 (SBOM completeness — decisive, in scope). The device exposes top-level version strings only and provides no consolidated SBOM. The installed software necessary for safe operation demonstrably includes un-enumerated third-party components: µC/OS-II (Micrium) (SRIO-1260), the SIL3 PROFIsafe stack (SRIO-1929), the STM32F7 self-test library (SRIO-10766), the netX90 PROFINET stack (SRIO-1768). Without an SBOM these cannot be inventoried or CVE-correlated → CR 7.8 partially met. Strongest, requirement-specific gap (OWASP ISVS V1.1.1; BSI TR-03183; 4-1 SM-9/SM-10). [209_SRIO_C...e_RemoteIO | PDF], [ifmworld-m...epoint.com], [ICS-Securi...endium_pdf | PDF]

- G-2 (confidentiality — in scope, your secondary gap). All inventory served over plaintext HTTP (SRIO-7903), HTTPS/WebSocket/MQTT unsupported (SRIO-7907/7906/7905) → on-segment version reconnaissance. CR 4.1/4.3 not met. [209_SRIO_C...e_RemoteIO | PDF]

- — Moved to RQ-005 (protection of identified software). Cross-reference only.

SL/conformance conclusion (identification-scoped): RQ-006 is partially met. Top-level installed-software identification exists and is readable (CR 7.8 accessibility satisfied); shortfalls are no SBOM-grade third-party inventory (G-1, decisive) and no transport confidentiality (G-2). The integrity/authenticity of the inventory is assessed under RQ-005, not here. This mirrors RQ-004's verdict (a defensible consistency to foreground).

Step 2 — Methodological Test Planning (Scope-Corrected)

RQ-006 overlaps RQ-004; I reuse and reference rather than duplicate. Only the SBOM-completeness test is genuinely RQ-006-specific.

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Enumerate the full installed-software surface (/deviceinfo/swinfo/*, I&M0, I&M5, /firmware/version, /fieldbussetup/fieldbusfirmware); build the installed-software inventory | Functional CR 7.8; confirms host/SCPU/bootloader/PROFIsafe-stack identification | Requirements-based verification; OWASP FSTM Stage 1 | SVV-1 |
| TS-2 | SBOM-completeness / version→CVE analysis: attempt to derive third-party components (µC/OS-II, PROFIsafe stack, STM STL, netX90); show no SBOM → no CVE correlation | Proves G-1 (decisive, requirement-specific) | OWASP FSTM "SBOM Generation" + ISVS V1.1.1; ifm CM-008 | SVV-3 |
| TS-3 | (reuse TC-RQ004-02) passive plaintext capture of the inventory read + confirm no TLS | Proves G-2 (confidentiality) | NIST SP 800-115 §3.5; OWASP ISTG-DES-INFO; ifm CM-005 | SVV-3 |
|  | — | Moved → RQ-005 (TC-RQ005-… identity/image integrity). Cross-referenced. | — | — |

Environment binding: TS-1/TS-2 over the management path (Pi eth1 = 192.168.50.10 → IoT-Core; browser 192.168.50.20); TS-3 passive on the laptop mirror. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog (Scope-Corrected)

Full entry for the requirement-specific baseline (TS-1) and the decisive SBOM-completeness proof (TS-2). TS-3 executes per the already-issued TC-RQ004-02. No MITM/inventory-integrity entry (→ RQ-005).

TC-RQ006-01 — Installed-Software Inventory Enumeration

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ006-01 |
| Traceability | RQ-006 (EHSR identification obligation, software) → CR 7.8 → Asset D |
| Target Threat | Information Disclosure / incomplete inventory |
| SVV Category | SVV-1 (Requirements-based Testing) |
| Objective | Enumerate the complete set of installed-software identifiers and confirm coverage of the software necessary for safe operation: host firmware, both bootloaders, SCPU firmware, and the PROFIsafe stack. Compile the actual inventory as the CR 7.8 baseline. |
| Abstract Steps | 1. Read all /deviceinfo/swinfo/* and /deviceinfo/swrevision. 2. Read I&M0 SOFTWARE_REVISION and I&M5 annotation. 3. Read /firmware/version and /fieldbussetup/fieldbusfirmware. 4. Assemble the inventory; map each entry to the safety function it supports. |
| Pass/Fail | PASS (partial-conformance baseline) iff host FW, both bootloaders, SCPU FW, and the PROFIsafe-stack version are all retrievable and internally consistent (I&M0/I&M5 agree with /deviceinfo/*). Documented shortfall where any safety-relevant installed component is not individually identified (feeds SBOM gap G-1). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ006-01 |
| HW/Network | DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10; browser 192.168.50.20 |
| Tools | curl / IoT-Core visualizer; PROFINET acyclic read for I&M0/I&M5 |

# TC-RQ006-01 Installed-software inventory enumeration (software-only; no data fingerprint)

for ep in deviceinfo/swrevision </span>

deviceinfo/swinfo/cpubootloaderversion </span>

deviceinfo/swinfo/scpuversion </span>

deviceinfo/swinfo/scpubootloaderversion </span>

firmware/version firmware/type </span>

fieldbussetup/fieldbusfirmware ; do

echo "== $ep =="; curl -s "http://192.168.0.2/$ep";

done | tee TC0601_swinv.txt

# I&M0 SOFTWARE_REVISION + I&M5 annotation via acyclic PN read -> append to inventory

| Expected | Host FW, CPU bootloader, SCPU FW, SCPU bootloader, PROFIsafe stack ("3.3.0.12"), I&M0/I&M5 all returned and mutually consistent. | | Result | ☐ All safety-relevant SW identified ☐ Consistency (deviceinfo ↔ I&M) confirmed | | Evidence | TC0601_swinv.txt, I&M0/I&M5 read log, inventory→safety-function mapping table | [209_SRIO_C...e_RemoteIO | PDF]

TC-RQ006-02 — SBOM Completeness & Third-Party CVE-Correlation Gap (G-1 proof — decisive)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ006-02 |
| Traceability | RQ-006 → CR 7.8 (SBOM limb) / 4-1 SM-9-SM-10 → Asset D |
| Target Threat | Information Disclosure (incomplete inventory → un-managed vulnerability) |
| SVV Category | SVV-3 (Vulnerability Testing) |
| Objective | Demonstrate that the exposed inventory is top-level only and provides no SBOM of the safety-critical third-party components (RTOS, PROFIsafe stack, STM self-test library, netX90 stack), so their versions cannot be enumerated or CVE-correlated — CR 7.8 met at product granularity but not at component granularity. |
| Abstract Steps | 1. From TC-RQ006-01, list all device-exposed version strings. 2. From documentation, list the known constituent third-party components. 3. Attempt to derive each component's version from any exposed interface. 4. For derivable versions, query a vulnerability database (off-lab); record the rest as un-inventoried. |
| Pass/Fail | Expected = FAIL: third-party components are not individually exposed and no SBOM exists → confirms G-1; documented as a CR 7.8 completeness shortfall requiring an SBOM (BSI TR-03183 / ISVS V1.1.1). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ006-02 |
| HW/Network | DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10; CVE lookup performed off-lab (no lab↔Internet coupling) |
| Tools | curl (inventory); manual coverage table; NVD lookup off-lab; optional GVM/OpenVAS (ifm CM-008) on the HTTP surface only |

# TC-RQ006-02 SBOM-completeness assessment (device side = read-only enumeration)

curl -s http://192.168.0.2/fieldbussetup/fieldbusfirmware -o TC0602_stack.json # PROFIsafe stack ver

# Coverage table: exposed-version vs known constituent (NOT exposed by device):

# uC/OS-II (Micrium) [SRIO-1260] -> NOT exposed

# PROFIsafe stack (SIL3) [SRIO-1929] -> partially via fieldbusfirmware

# STM32F7 Self-Test Library [SRIO-10766] -> NOT exposed

# netX90 PROFINET stack [SRIO-1768] -> NOT exposed

# derivable versions -> off-lab NVD query; rest -> "un-inventoried"

| Expected | Only the PROFIsafe stack version partially derivable; RTOS/STL/netX90 versions not exposed → no SBOM, no third-party CVE correlation. | | Result | ☐ SBOM absent (G-1 confirmed) — un-inventoried: ______ | | Evidence | TC0602_stack.json, exposed-vs-constituent coverage table, off-lab NVD notes | [209_SRIO_C...e_RemoteIO | PDF], [ifmworld-m...epoint.com], [ICS-Securi...endium_pdf | PDF]

Lab-safety note: DUT-side action is read-only enumeration; any CVE lookup runs on a separate Internet-connected host to preserve Zone-B isolation. [ifmworld-m...epoint.com]

Summary of Changes (vs. my original RQ-006)

- Removed the identity-integrity / MITM over-reach — dropped the CR 3.4 mapping, the Spoofing/Tampering-of-inventory primary threat, Asset F as primary, the G-3 (inventory integrity) gap, and the TC-RQ004-04 (MITM) reuse. All cross-referenced to RQ-005 (protection of identified software/data).(Consistent with the RQ-004 correction: RQ-004/006 identify; RQ-005 protects.)

- Anchored your confidentiality point to CRs — added CR 4.1 (+CR 4.3) to the mapping so G-2 is traceable (previously a gap with no CR). [209_SRIO_C...e_RemoteIO | PDF]

- STRIDE simplified to Information Disclosure (primary) — the correct always-present threat for an identification requirement.

- Kept the RQ-006 strengths — the software-only scope (CR 7.8 sole primary; no data-fingerprint), the completed mitigation surface (/firmware/version, /fieldbussetup/fieldbusfirmware = PROFIsafe stack), and the two genuine gaps: G-1 (SBOM — decisive) and G-2 (confidentiality). [209_SRIO_C...e_RemoteIO | PDF], [ifmworld-m...epoint.com]

Corrected verdict, tightly bounded: RQ-006 = identify installed safety software (completeness, incl. SBOM gap) + keep that identification confidential — with integrity/protection of the inventory deferred to RQ-005, and change-tracking deferred to RQ-009.

Consolidation status

The scope-boundary recheck is now complete across the requirements that shared material:

- RQ-001 → DoS/DCP-flood deferred to RQ-010/RQ-011; connecting-device integrity/authenticity core retained.

- RQ-002 → evidence-collection deferred to RQ-003; tamper-resistance core retained.

- RQ-004 → REVISION_COUNTER deferred to RQ-009; identity-integrity/MITM deferred to RQ-005; identification + confidentiality core retained.

- RQ-005 → update-authorization/gate deferred to RQ-011; firmware/config integrity + authenticity core retained.

- RQ-006 → identity-integrity/MITM deferred to RQ-005; software-identification + confidentiality core retained.

The clean requirement boundaries are now: identify (004/006) → protect (002/005) → authorize (011) → collect evidence (003/008/009) → retain/restrict (012/013/014) → withstand (001/010).

Would you like me to (a) re-check RQ-010 to confirm it correctly receives the DoS/DCP-flood tests now deferred from RQ-001 (and that its firmware-EoP cross-references RQ-005/RQ-011 cleanly), or (b) proceed to compile the fully corrected RQ-001 → RQ-014 set into the consolidated thesis appendix (Word/PDF) with the master traceability matrix and the boundary map above?

REQUIREMENT 7

RQ-007 — Reprocessed (Scope-Corrected)

Correction applied: Rechecking RQ-007 against the boundary discipline, I found a framing/scope error rather than an imported-asset error, plus two tests that belong to RQ-010:

- Threat mis-framing (the key correction). My original set the primary threat as**"Denial of Service (attack)."** But re-reading the requirement —"shall be able to provide … at all times" — the dominant, decisive failing is not a third-party attack; it is the device's own state machine (Init, install-reboot, SCPU-flash, FatalError, and the documented EMC "not-supervised" concession). That is an inherent availability/reliability limitation, not a malicious DoS. The primary should therefore be inherent unavailability (design limitation), with a self-inflicted DoS (the 2-connection cap) as the one attack-adjacent, RQ-007-specific vector.

- Generalized-DoS scope creep → RQ-010. My TS-4 (sustained load/flood) and the attack-induction of TS-5 (COM-fault DoS) are generalized "withstand malicious attempts" = RQ-010. Removed from RQ-007; the availability-architecture half of the COM-fault point is retained as a redundancy/reliability check.

- Boundary statement (kept explicit). RQ-007 is availability + accessibility of the identification info only — completeness → RQ-006, confidentiality → RQ-004/006, generalized DoS resilience → RQ-010. (My original correctly did not import SBOM or confidentiality — that stays out.)

Step 1 — Critical Review & Theoretical Validation

1.1 Requirement & CR Mapping

EU Machinery Regulation identification obligation with an availability + accessibility qualifier ("at all times," "easily accessible form"). Two limbs: availability (FR 7) and accessibility (readable form).

| CR | Your input | Verdict | Correction |
| --- | --- | --- | --- |
| CR 7.8 – Component inventory | ✅ | Correct (the what is identified) | Keep — carries over from RQ-006. |
| CR 7.1 – DoS protection | ✅ | Re-scope | Keep, but scope to the self-inflicted 2-connection vector; generalized malicious DoS → RQ-010. |
| CR 7.2 – Resource management | ✅ | Correct | Keep — the connection cap is a resource-management control (and its own weakness). |
| CR 6.1 – Audit/data accessibility | ➕ Add | Correct | The**"easily accessible form"** limb maps here — and is partially MET via the Visualizer. Anchoring it makes the verdict precise. |
| CR 7.7 – Least functionality | ➕ note (supporting) | Keep as supporting | "Incoming-only, no outgoing/subscription" (SRIO-7908) is surface reduction. |
|  | — | → RQ-010 | "Withstand reasonably foreseeable malicious attempts" (flooding, fuzzing) is RQ-010. |

Corrected mapping: RQ-007 → EHSR identification obligation (availability/accessibility) →{CR 7.8 [inventory], CR 7.1 [self-DoS — 2-connection], CR 7.2 [resource mgmt], CR 6.1 [accessible form — partially met]}, with CR 7.7 supporting.(Generalized flood/DoS → RQ-010.) [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset H – IoT-Core availability | ✅ | Correct primary. |
| Asset D – identification data ("at all times") | ✅ | Correct — note split sourcing: host FW/CPU-bootloader = COM-sourced (EEPROM); scpuversion/scpubootloaderversion = SCPU-sourced via SysCom (SRIO-2946). During an SCPU flash the SCPU-sourced identifiers are unavailable even if HTTP answers. [209_SRIO_C...e_RemoteIO ¦ PDF] |
| Asset F – Domain separation (availability dependency) | ✅ (I added) | Keep, but re-scoped to reliability/redundancy — SCPU-sourced identification depends on SysCom (SRIO-21357); a SysCom timeout removes safe-side identification while the safety function continues (availability inversion). This is a reliability dependency of RQ-007, not an attack surface (attack-induction → RQ-010). |

STRIDE — corrected:

- ✅ Inherent unavailability / loss of availability (primary — design/reliability). The dominant failing is the device's own operating states, not an attacker.(STRIDE labels this under Denial-of-Service broadly, but the cause is design, not malice — state this explicitly.)

- ✅ Self-inflicted DoS (secondary — the one attack-adjacent, RQ-007-specific vector). The 2-connection cap (SRIO-7909) is a CR 7.1 mechanism that doubles as a CR 7.1 weakness — two held connections block all further identification reads, with no attacker bandwidth needed (Slowloris-style, no documented session timeout).

- — → RQ-010.

1.3 Security Functions & Gaps — Cross-Referenced

Mitigations confirmed; accessibility control added:

| Mechanism | Documentation evidence | Effect |
| --- | --- | --- |
| Max 2 concurrent HTTP connections | SRIO-7909 | Resource-exhaustion cap (CR 7.1/7.2) — and the self-DoS vector [209_SRIO_C...e_RemoteIO ¦ PDF] |
| Incoming-only, no outgoing/subscription | SRIO-7908 | Surface reduction (CR 7.7) [209_SRIO_C...e_RemoteIO ¦ PDF] |
| IoT-Core Visualizer (browser, by IP) | SRIO-7914/7913 | The**"easily accessible form"** — accessibility limb largely MET [209_SRIO_C...e_RemoteIO ¦ PDF] |
| HTTP served by non-safety COM (QM) only, no redundancy | SRIO-2100; SysArch COM = QM | Single point of failure for availability [209_SRIO_C...e_RemoteIO ¦ PDF], [Market_Acc...ty_Systems ¦ PDF] |

Gaps (availability/accessibility-scoped):

- G-1 (availability "at all times" — the core, design-not-attack). The info is not available at all times for five documented reasons — none of which is a malicious attack:

  - (a) install reboot — /firmware/install "initiates a reboot to install" (SRIO-2953);

  - (b) Init state at every power-on before IoT-Core is up (SRIO-1417);

  - (c) SCPU-flash window — SCPU-sourced identifiers unavailable (SRIO-15759);

  - (d) FatalError — safety parts held in safe state (SRIO-15757);

  - (e) EMC disturbance — the manufacturer's own Criterion A/B concedes**"IoT-Core Communication: Not supervised … After the tests: IoT-Core access possible"** (SRIO-11573/11572). This is the killer citation — a documented admission that identification is not guaranteed during disturbance, directly contradicting "at all times." [209_SRIO_C...e_RemoteIO | PDF], [100_SRIO_P...e_RemoteIO | PDF]

- G-2 (2-connection self-DoS — accurate, keep — the one attack-adjacent vector). Two held/idle connections exhaust the pool (SRIO-7909) and block all further legitimate identification reads; incoming-only, no documented timeout. ✅ [209_SRIO_C...e_RemoteIO | PDF]

- G-3 (no redundancy / QM single point of failure — keep, reliability-scoped). A COM fault or SysCom timeout removes identification availability entirely while the safety function persists → identification is structurally less available than the safety function it describes.(Reliability dependency; attack-induction of the fault → RQ-010.) [209_SRIO_C...e_RemoteIO | PDF]

- G-4 (accessibility — partial credit, not a gap). The Visualizer (SRIO-7914) satisfies "easily accessible form" — state this for a balanced verdict. [209_SRIO_C...e_RemoteIO | PDF]

SL/conformance conclusion: The accessibility limb ("easily accessible form") is met via the Visualizer; the availability limb ("at all times") is not met — for five documented, non-malicious operational reasons (G-1, incl. the manufacturer's EMC "not-supervised" concession), compounded by a self-inflicted 2-connection DoS (G-2) and no redundancy on a QM component (G-3). Net: RQ-007 partially met; availability is the failing limb, and the failing is primarily a design/reliability limitation, not a third-party attack (generalized malicious DoS is assessed under RQ-010).

Step 2 — Methodological Test Planning (Scope-Corrected)

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Retrieve identification via IoT-Core + Visualizer in OPERATE; confirm human-readable, browser-accessible | Functional CR 7.8; confirms "easily accessible" limb (G-4 credit) | Requirements-based verification | SVV-1 |
| TS-2 | Availability-across-states: continuously poll the identification endpoint while driving Init (power-cycle), Update (999 + install reboot), and FatalError; log outage windows and the SCPU-flash window | Proves G-1 (not available "at all times" — design, not attack) | NIST SP 800-115 §3.2 (availability/log review) | SVV-2 |
| TS-3 | Connection-hold self-DoS: open and hold 2 HTTP connections; attempt a 3rd legitimate identification read | Proves G-2 (self-inflicted DoS; CR 7.1 mechanism = CR 7.1 weakness) | MITRE ATT&CK for ICS T0814 (self-inflicted variant); NIST SP 800-115 §5.2; ifm CM-009 | SVV-4 |
| TS-4 | Redundancy/reliability check: with a COM/SysCom interruption present, confirm identification is unavailable while the safety function persists (availability inversion) | Proves G-3 (no redundancy) — reliability, not attack | NIST SP 800-115 §3.2; OWASP ISTG-DES | SVV-3 |
|  | — | → RQ-010 (TC-RQ010-02/03/04) | — | — |

Environment binding: all over the management path (Pi eth1 = 192.168.50.10 → IoT-Core; browser 192.168.50.20); the connection-hold tool runs on the Pi, never the laptop. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog (Scope-Corrected)

Full entries for the accessibility baseline (TS-1), the availability-across-states proof (TS-2 → G-1, the core), and the 2-connection self-DoS proof (TS-3 → G-2). TS-4 follows the established template. No generalized-flood entry (→ RQ-010).

TC-RQ007-01 — Identification Availability & Accessibility Baseline

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ007-01 |
| Traceability | RQ-007 (EHSR identification, availability/accessibility) → CR 7.8 / CR 6.1 → Asset H, Asset D |
| Target Threat | Loss of availability (baseline reference) |
| SVV Category | SVV-1 (Requirements-based Testing) |
| Objective | Confirm that, in normal OPERATE, the safety-relevant installed-software identification is retrievable and presented in an easily accessible form (browser Visualizer + human-readable version strings), establishing the availability baseline. |
| Abstract Steps | 1. In OPERATE, open the IoT-Core Visualizer by device IP. 2. Retrieve all safety-relevant SW identifiers. 3. Confirm values are human-readable and the Visualizer renders without special tooling. |
| Pass/Fail | PASS iff the Visualizer loads and all safety-relevant SW identifiers are shown in human-readable form (accessibility limb met). FAIL iff any identifier is unreadable or the Visualizer is inaccessible in OPERATE. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ007-01 |
| HW/Network | DUT 192.168.0.2; browser 192.168.50.20; Pi mgmt eth1 = 192.168.50.10 |
| Tools | Web browser (Visualizer), curl |
| Procedure | http://192.168.0.2 (Visualizer) + curl -s http://192.168.0.2/deviceinfo/swinfo/scpuversion etc. |
| Result | ☐ Visualizer accessible ☐ SW identifiers human-readable |
| Evidence | Visualizer screenshot, TC0701_ids.txt |

TC-RQ007-02 — Identification Availability Across Operating States (G-1 proof — core)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ007-02 |
| Traceability | RQ-007 → CR 7.8 / (availability, FR 7) → Asset H, Asset D |
| Target Threat | Inherent unavailability (design/reliability, not attack) |
| SVV Category | SVV-2 (Threat-Mitigation Testing) |
| Objective | Demonstrate that identification data is not available "at all times" by continuously polling the identification endpoint while the device transitions through Init (power-cycle), Update (999 + install reboot), and FatalError, and by identifying the SCPU-flash window in which SCPU-sourced identifiers vanish. |
| Abstract Steps | 1. Start a timestamped polling loop against a COM-sourced and an SCPU-sourced identifier. 2. Power-cycle (Init window). 3. Enter Update (999 + reboot; trigger /firmware/install reboot on a spare DUT). 4. Force FatalError. 5. Record every interval where the request fails or the SCPU-sourced value is absent. |
| Pass/Fail | Gap-characterisation test. Expected: measurable outage windows during Init, install-reboot, and FatalError, and unavailability of SCPU-sourced identifiers during the SCPU flash → confirms G-1 and a FAIL against "at all times." A device returning valid identification continuously through all states would refute G-1 (investigate). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ007-02 |
| HW/Network | Spare DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10 |
| Tools | curl polling loop with timestamps; power switch; rotary switch |

# TC-RQ007-02 Availability sampling across state transitions (design-inherent unavailability)

LOG=TC0702_avail.csv; echo "ts,endpoint,http_code" > $LOG

poll() { while true; do

for ep in deviceinfo/swrevision deviceinfo/swinfo/scpuversion; do

code=$(curl -s -o /dev/null -m 1 -w '%{http_code}' http://192.168.0.2/$ep)

echo "$(date +%s.%N),$ep,$code" >> $LOG

done; sleep 0.2; done; }

poll & # start sampling

# --- operator actions, timestamped in a lab notebook ---

# (1) power-cycle DUT -> Init outage window

# (2) set rotary=999, reboot -> Update state

# (3) trigger /firmware/install -> install reboot outage

# (4) induce FatalError (EC1) -> safe-state window

kill %1

# analyse: intervals where http_code != 200, and where scpuversion is empty/timeout

| Expected | Outage rows (non-200/timeout) during Init, install-reboot, FatalError; scpuversion empty during SCPU flash (SRIO-15759, SRIO-2953); cross-reference EMC "not-supervised" concession (SRIO-11573/11572). | | Result | ☐ Outage windows recorded (G-1 confirmed) — total unavailable time: ______ | | Evidence | TC0702_avail.csv, annotated timeline, LED-state photos per transition | [209_SRIO_C...e_RemoteIO | PDF], [100_SRIO_P...e_RemoteIO | PDF]

TC-RQ007-03 — Connection-Hold Self-DoS on the Identification API (G-2 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ007-03 |
| Traceability | RQ-007 → CR 7.1 / CR 7.2 → Asset H |
| Target Threat | Self-inflicted DoS (resource-cap design flaw) |
| SVV Category | SVV-4 (Penetration Testing) |
| Objective | Demonstrate that the 2-connection limit (SRIO-7909), intended as DoS protection, is itself a DoS vector: two held connections exhaust the pool and block all further legitimate access to the identification API, defeating "at all times" without any bandwidth-based flooding (which is RQ-010). |
| Abstract Steps | 1. In OPERATE, open two HTTP connections to the IoT-Core and hold them (no completion / slow read). 2. From a third client, attempt a legitimate identification read. 3. Measure whether the third request is refused/timed out. 4. Release one held connection; confirm access is restored. |
| Pass/Fail | PASS against the requirement iff a legitimate third read still succeeds (service resilient to two held connections). Expected = FAIL: the third request is blocked while two connections are held → confirms G-2 (self-inflicted DoS availability failure). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ007-03 |
| HW/Network | DUT 192.168.0.2; Pi eth1 = 192.168.50.10 (attacker + legitimate client) |
| Tools | Python (raw sockets, hold connections), curl (third-client probe) |

#!/usr/bin/env python3

# TC-RQ007-03 Hold 2 HTTP connections; then probe with a 3rd legitimate read

import socket, subprocess, time

DUT, PORT = "192.168.0.2", 80

def hold(): # open + partial request, never finish (Slowloris-style)

s = socket.create_connection((DUT, PORT), timeout=5)

s.sendall(b"GET /deviceinfo/swrevision HTTP/1.1\r\nHost: dut\r\n") # header withheld

return s

held = [hold(), hold()] # occupy both permitted connections (SRIO-7909)

time.sleep(1)

print("[*] 2 connections held; probing with a 3rd legitimate request...")

r = subprocess.run(["curl","-s","-m","3","-o","/dev/null","-w","%{http_code}",

f"http://{DUT}/deviceinfo/swinfo/scpuversion"], capture_output=True, text=True)

print("[+] 3rd-client HTTP code:", r.stdout or "TIMEOUT") # expect: timeout/refused -> G-2

for s in held: s.close()

| Expected | Third-client read times out / is refused while both connections are held; access returns after release (SRIO-7909). | | Result | ☐ 3rd read blocked (G-2 confirmed) ☐ Access restored on release | | Evidence | Script console log, curl timing, optional mirror PCAP of the three sessions | [209_SRIO_C...e_RemoteIO | PDF]

Scope note: this is the self-inflicted, RQ-007-specific design DoS (two legitimate connections suffice). Generalized malicious flooding of the device is RQ-010 (TC-RQ010-02). Even this benign socket-hold runs on the Pi, isolated Zone-B, PLC-monitored. [Security_T...rk_Kali_EN | Word]

Summary of Changes (vs. my original RQ-007)

- Re-framed the primary threat from**"DoS attack"** to**"inherent unavailability (design/reliability limitation)."** The dominant failing is the device's own operating states (Init, install-reboot, SCPU-flash, FatalError) and the manufacturer's EMC "not-supervised" concession — none of which is a third-party attack. STRIDE still labels this under DoS broadly, but the cause is design, not malice, and that framing better matches the "at all times" text. [209_SRIO_C...e_RemoteIO | PDF], [100_SRIO_P...e_RemoteIO | PDF]

- Deferred generalized DoS/flood to RQ-010 — removed TS-4 (sustained load/flood) and the attack-induction of the COM-fault test; TS-4 is now a redundancy/reliability check (G-3), not an attack.

- Kept the RQ-007-specific self-DoS (2-connection cap, G-2) — it is a design property of this identification service, not generalized flooding, so it stays.

- Anchored the accessibility limb to CR 6.1 and credited the Visualizer (G-4, largely met) for a balanced verdict.

- Held the boundary: completeness → RQ-006, confidentiality → RQ-004/006, generalized DoS resilience → RQ-010; here = availability + accessibility only.

Corrected verdict, tightly bounded: RQ-007 = provide the identification "at all times" (availability — fails via ≥5 inherent operational windows + a self-inflicted 2-connection DoS + no redundancy) in an "easily accessible form" (accessibility — met via the Visualizer) — with generalized malicious DoS deferred to RQ-010.

Would you like me to (a) re-check RQ-010 next, to confirm it now correctly receives the deferred generalized-DoS/flood tests from RQ-001 and RQ-007 (and that its firmware-EoP cross-references RQ-005/RQ-011 cleanly), or (b) proceed to compile the fully corrected RQ-001 → RQ-014 set into the consolidated thesis appendix?

REQUIREMENT 8

RQ-008 — Reprocessed (Scope-Corrected)

Correction applied: Rechecking RQ-008 against the boundary discipline, I found the same config-vs-software leak that I corrected in RQ-004, this time crossing into RQ-009:

- Config-modification markers imported (scope creep → RQ-009). My original RQ-008 listed the I&M0 REVISION_COUNTER and the I&M4 signature as evidence mechanisms. Both are configuration-modification markers — REVISION_COUNTER increments on parameter change; I&M4 fingerprints the parameter set. Modification of software or its configuration is RQ-009. RQ-008 is scoped to intervention in the installed software (firmware). You yourself flagged this in your submission ("REVISION_COUNTER … primarily relevant to configuration modification, RQ-009") — I am now acting on that flag and removing both from RQ-008, keeping only software/firmware-intervention evidence.

- G-2 framing tightened. The "legitimate-vs-illegitimate indistinguishability" point is valid for RQ-008, but its cause (no firmware-signature verification) is RQ-005's finding. I re-frame G-2 as an evidence-classification failure and cross-reference RQ-005 for the underlying protection gap — without re-testing the protection here.

- G-3 (self-erasing install) retained but bounded. The install-reboot clearing is RQ-008's most decisive finding (the software-intervention's own record is destroyed at collection time); the 5-year retention dimension of that same mechanism is RQ-012 — cross-referenced.

The audit-family CR mapping (CR 2.8/2.9/2.10/2.11/2.12/3.9/6.1 + EDR 2.13) is correct and stays — consistent with RQ-003.

Step 1 — Critical Review & Theoretical Validation

1.1 Requirement & CR Mapping

EU Machinery Regulation (EU) 2023/1230, Annex III EHSR 1.1.9 §2, scoped to installed software — an audit/accountability requirement. CR 2.8 alone cannot carry it (evidence needs time, capacity, overflow handling, protection).

| CR | Your input | Verdict | Note |
| --- | --- | --- | --- |
| CR 2.8 – Auditable events | ✅ | Correct, partial | Logs that a software event occurred, not whether authorised. |
| CR 2.12 – Non-repudiation | ✅ | Correct, structurally constrained | No user auth on the FW-update/IoT path (SRIO-8299) → no actor. |
| CR 2.11 – Timestamps | ➕ (I added) | Decisive | No RTC; uptime resets on cold start (SRIO-15218/15398). |
| CR 2.9 / 2.10 – Capacity / overflow response | ➕ (I added) | Keep | Circular ≥100 buffer (SRIO-10703/10663). |
| CR 3.9 – Protection of audit information | ➕ (I added) | Killer | Log non-safety (QM), cleared on cold start (SRIO-6521); the install reboot (SRIO-2953) + Exit-Update power reset (SRIO-9916) destroy the record. |
| CR 6.1 – Audit-log accessibility | ➕ (I added) | MET | Log IoT-readable (SRIO-10662). |
| EDR 2.13 – Physical diagnostic/test interfaces | ➕ (I added) | Keep | Correct home for the debug/test-port point (fusing = exclusion, not evidence). |
|  | — | Removed → RQ-009 | Config-modification markers belong to RQ-009. |

Corrected mapping: RQ-008 → EHSR 1.1.9 §2 (software) →{CR 2.8, CR 2.9, CR 2.10, CR 2.11, CR 2.12, CR 3.9, CR 6.1, EDR 2.13}. [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset D – subject of intervention (firmware) | ✅ | Correct primary. |
| Asset G – software-intervention trigger | ✅ | Correct — FW update gated by rotary 999 + reboot (SRIO-2013/11534). [100_SRIO_P...e_RemoteIO ¦ PDF], [Market_Acc...ty_Systems ¦ PDF] |
| Asset I – evidence | ✅ | Correct — candidate store = COM Error Log (non-safety, volatile). |
| Asset F – Domain separation | ✅ (I added) | Keep — log on non-safety COM; update commands cross SysCom (UPDATE_START/COMPLETED/FAILED, SRIO-10785). [Market_Acc...ty_Systems ¦ PDF] |

STRIDE: ✅ Repudiation (primary) + ➕ Tampering-on-audit (cold-start/reboot clears the log; MITRE ATT&CK for ICS T0872 Indicator Removal on Host — justifies CR 3.9). No change needed.

1.3 Security Functions & Gaps — Cross-Referenced

Software-intervention evidence surface — corrected to exclude config markers:

| Mechanism | Documentation evidence | Evidences | Failure |
| --- | --- | --- | --- |
| COM Error Log (FW-update actions, self-detected errors, SysCom timeout) | SRIO-10665/10703/10663/6521 | That a software event occurred | Circular, non-safety, cold-start-cleared [209_SRIO_C...e_RemoteIO ¦ PDF] |
| SysCom update command/response events | SRIO-10785 (UPDATE_START/COMPLETED/FAILED) | Update start/success/failure | Transient control messages, not persistently logged with time/actor [Market_Acc...ty_Systems ¦ PDF] |
| /firmware/version + I&M0 SOFTWARE_REVISION delta | SRIO-2953/3015 | That firmware version changed | Indirect state-delta; no when/who/authorised [209_SRIO_C...e_RemoteIO ¦ PDF] |
| Update-state LED sequence (US green/FS on fail) | SRIO-11359/15512/15511 | Local, transient visual indication | Not recorded [Market_Acc...ty_Systems ¦ PDF] |
| Debug/test port fused in production | SRIO-2580/1526 | Removes a modification path | EDR 2.13 exclusion, no log entry [100_SRIO_P...e_RemoteIO ¦ PDF], [209_SRIO_C...e_RemoteIO ¦ PDF] |
|  | SRIO-3002 | REMOVED → RQ-009 (config modification) |  |
|  | SRIO-3011 | REMOVED → RQ-009/RQ-004 (config fingerprint) |  |

Gaps (software-intervention-scoped):

- G-1 (attribution — accurate, keep). Events recorded without actor identity; no user auth on the FW-update path → CR 2.12 unmet. An illegitimate software intervention avoiding an official path is unrecorded and unattributable. [209_SRIO_C...e_RemoteIO | PDF]

- G-2 (legitimate-vs-illegitimate indistinguishable — re-framed as evidence-classification). RQ-008 demands evidence of both legitimate AND illegitimate intervention. The evidence footprint of a genuine update and a modified/unsigned image is identical ("update completed" + version change). Cause: the device performs no firmware-signature verification — but that protection gap is RQ-005 (TC-RQ005-03); here the finding is that the evidence cannot classify the two, so the "legitimate OR illegitimate" obligation is unmet. [209_SRIO_C...e_RemoteIO | PDF]

- G-3 (self-erasing install — decisive, evidence-scoped). /firmware/install"initiates a reboot to install" (SRIO-2953) and Exit-Update requires a power reset (SRIO-9916); a cold start clears the log (SRIO-6521) and resets uptime (SRIO-15398). The act of installing software destroys the log evidence of that installation — the update mechanism is inherently self-erasing.(The 5-year retention dimension of this same mechanism → RQ-012.) [209_SRIO_C...e_RemoteIO | PDF]

- G-4 (debug/test port — accurate, keep). Fusing = EDR 2.13 exclusion; a re-probe still generates no log entry. [100_SRIO_P...e_RemoteIO | PDF]

SL/conformance conclusion (software-intervention-scoped): RQ-008 is not met for genuine intervention evidence. The log is readable (CR 6.1 met) but provides no actor attribution (CR 2.12), no absolute/persistent time (CR 2.11), no protection/retention (CR 3.9 — actively self-erased by the install reboot, G-3), and no legitimate/illegitimate classification (CR 2.8 intent, G-2).(Config-modification evidence → RQ-009; firmware-protection cause of G-2 → RQ-005; 5-year retention → RQ-012.)

Step 2 — Methodological Test Planning (Scope-Corrected)

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Enumerate the software-event evidence that exists: Error Log, /firmware/version, I&M0 SOFTWARE_REVISION at baseline (no REVISION_COUNTER/I&M4 — those are RQ-009) | Functional baseline (CR 6.1 met; CR 2.8 partial) | Requirements-based verification; NIST SP 800-115 §3.2 | SVV-1 |
| TS-2 | Perform a legitimate authorised FW update; capture whether a retained, time-stamped, attributed record survives the install reboot | Proves G-3 (self-erasing install) + G-1 | NIST SP 800-115 §3.2 / §7 (evidence handling) | SVV-2 |
| TS-3 | Compare evidence footprints of a legitimate update vs the modified/unsigned image installed in TC-RQ005-03; check whether the log distinguishes them | Proves G-2 (evidence cannot classify) — install action referenced from RQ-005 | MITRE ATT&CK for ICS T0872; OWASP ISTG-FW[UPDT] | SVV-4 |
| TS-4 | Time-attribution across the install cold start: uptime resets, no absolute timestamp on either state | Proves G-1/CR 2.11 | NIST SP 800-115 §3.2 (reuse TC-RQ003-04 method) | SVV-3 |
| TS-5 | Invoke the fused diagnostic/FIT path (/fit/setfit, WO/P) in production; confirm exclusion produces no log entry | Validates EDR 2.13; confirms G-4 | NIST SP 800-115 §4.2; OWASP FSTM / ISTG-PHY | SVV-3 |

Environment binding: management path (Pi eth1 = 192.168.50.10 → IoT-Core); TS-3 reuses the spare-DUT, isolated-lab, PLC-monitored constraints of TC-RQ005-03; re-flash a validated image afterward. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog (Scope-Corrected)

Full entries for the baseline (TS-1), the self-erasing legitimate install (TS-2 → G-3), and the evidence-classification proof (TS-3 → G-2). REVISION_COUNTER/I&M4 excluded from all (→ RQ-009). TS-4/TS-5 per established templates.

TC-RQ008-01 — Software-Event Evidence Baseline

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ008-01 |
| Traceability | RQ-008 (EHSR 1.1.9 §2, software) → CR 2.8 / CR 6.1 → Asset I, Asset D |
| Target Threat | Repudiation (baseline of available software-event evidence) |
| SVV Category | SVV-1 (Requirements-based Testing) |
| Objective | Establish which software-intervention evidence the device produces and whether it is readable: the COM Error Log, /firmware/version, and I&M0 SOFTWARE_REVISION — and inspect each for actor identity and absolute timestamp.(Config markers REVISION_COUNTER/I&M4 are out of scope → RQ-009.) |
| Abstract Steps | 1. Read the error log and firmware/SOFTWARE_REVISION records at a known-good baseline. 2. Trigger a benign self-detected software event (e.g., brief SysCom interruption). 3. Re-read; confirm an Error-Log entry appears. 4. Inspect the entry for identity and absolute timestamp. |
| Pass/Fail | PASS iff the log and software records are readable and the induced event is captured (CR 6.1 + partial CR 2.8). Documented shortfall iff no entry carries actor identity or absolute timestamp. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ008-01 |
| HW/Network | DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10; browser 192.168.50.20 |
| Tools | curl / IoT-Core visualizer; PN acyclic read for I&M0 SOFTWARE_REVISION |

# TC-RQ008-01 Software-event evidence baseline (config markers EXCLUDED -> RQ-009)

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC0801_log_base.json

curl -s http://192.168.0.2/firmware/version -o TC0801_fwver_base.json

# I&M0 SOFTWARE_REVISION via acyclic PN read (NOT REVISION_COUNTER / NOT I&M4 -> RQ-009)

# --- induce a benign self-detected software event (e.g., brief SCPU/SysCom interruption) ---

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC0801_log_after.json

diff TC0801_log_base.json TC0801_log_after.json | tee TC0801_logdiff.txt # expect: new ErrorEvent

| Result | ☐ Log readable (CR 6.1 met) ☐ Event captured ☐ No identity/timestamp present | | Evidence | TC0801_*.json, TC0801_logdiff.txt, IoT-Core screenshots |

TC-RQ008-02 — Self-Erasing Firmware Install (G-3 proof — decisive)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ008-02 |
| Traceability | RQ-008 → CR 3.9 / CR 2.11 → Asset I, Asset D |
| Target Threat | Repudiation + anti-forensic Tampering (self-inflicted) |
| SVV Category | SVV-2 (Threat-Mitigation Testing) |
| Objective | Demonstrate that a legitimate firmware install destroys its own evidence: the /firmware/install reboot (and mandatory Exit-Update power reset) triggers a cold start that clears the Error Log and resets uptime, so no retained, time-stamped record of the software intervention survives. |
| Abstract Steps | 1. Baseline the Error Log and uptime. 2. Perform an authorised firmware update (999 + reboot → upload → /firmware/install). 3. After the install reboot, re-read the Error Log and uptime. 4. Determine whether any persistent, time-stamped record of the install remains. |
| Pass/Fail | Gap-characterisation test. Expected: post-install log empty/cleared and uptime reset toward zero → the install has erased its own evidence, confirming G-3 and a FAIL against RQ-008. A device retaining a persistent, time-stamped install record across the reboot would refute G-3 (investigate). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ008-02 |
| HW/Network | Spare DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10 |
| Tools | curl (log/uptime + update services); rotary switch |

# TC-RQ008-02 Does a legitimate install retain evidence across its own reboot?

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC0802_log_pre.json

curl -s http://192.168.0.2/systemtime/systick -o TC0802_tick_pre.json

# --- authorised update: rotary=999 + reboot; upload a valid container; then: ---

curl -s -X POST http://192.168.0.2/firmware/install # "initiates a reboot to install"

# --- device reboots (cold start); after Exit-Update power reset and re-init: ---

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC0802_log_post.json

curl -s http://192.168.0.2/systemtime/systick -o TC0802_tick_post.json

diff TC0802_log_pre.json TC0802_log_post.json | tee TC0802_logdiff.txt # expect: log cleared

| Expected | Post-install log empty (cleared on cold start, SRIO-6521); tick_post << tick_pre (uptime reset, SRIO-15398); no persistent install record. | | Result | ☐ Evidence self-erased (G-3 confirmed) ☐ Record retained (investigate) | | Evidence | TC0802_log_pre/post.json, TC0802_tick_pre/post.json, TC0802_logdiff.txt, /firmware/version before/after | [209_SRIO_C...e_RemoteIO | PDF]

TC-RQ008-03 — Legitimate vs Illegitimate: Evidence Cannot Classify (G-2 proof — evidence-scoped)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ008-03 |
| Traceability | RQ-008 → CR 2.8 (classification intent) → Asset D, Asset I, Asset F |
| Target Threat | Repudiation (illegitimate intervention recorded as legitimate) |
| SVV Category | SVV-4 (Penetration Testing) |
| Objective | Demonstrate that the evidence record cannot distinguish a legitimate update from an illegitimate one: installing a modified/unsigned image**(the install action is performed per TC-RQ005-03)** produces the same log/version footprint as a genuine update — so the "legitimate OR illegitimate" evidence obligation is unmet. The cause (no signature verification) is RQ-005's protection gap; RQ-008 tests only the evidence footprint. |
| Abstract Steps | 1. Record the evidence delta of a legitimate update (from TC-RQ008-02). 2. On the spare DUT, install the modified/unsigned image per TC-RQ005-03. 3. Record its evidence delta (log entries, version change, any classification flag). 4. Compare: determine whether any evidence field marks the second install as illegitimate/unauthorised. |
| Pass/Fail | Gap-characterisation test. Expected: the illegitimate install produces an indistinguishable evidence footprint (same "update completed" semantics + version change, no authenticity flag) → confirms G-2 and a FAIL against RQ-008's legitimate/illegitimate requirement. Any evidence field flagging the unsigned image as illegitimate would refute G-2 (document as a positive finding). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ008-03 |
| HW/Network | Spare DUT 192.168.0.2 (per TC-RQ005-03 constraints); Pi mgmt eth1 = 192.168.50.10 |
| Tools | curl (update + log); binwalk/hex editor (modify container, per TC-RQ005-03); Wireshark |

# TC-RQ008-03 Compare evidence footprint: legitimate vs modified/unsigned install

# (A) legitimate install evidence delta = from TC-RQ008-02 (TC0802_logdiff.txt)

# (B) illegitimate install (modified container from TC-RQ005-03):

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC0803_log_pre.json

# ...999 + reboot; upload fw_mod.bin (from TC-RQ005-03); POST /firmware/install; reboot...

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC0803_log_post.json

curl -s http://192.168.0.2/firmware/version -o TC0803_fwver_post.json

diff TC0803_log_pre.json TC0803_log_post.json | tee TC0803_logdiff.txt

# Compare (A) vs (B): any field distinguishing authorised from unauthorised?

| Expected | (B) footprint ≈ (A): update proceeds/attempts identically, no authenticity/authorisation flag; unsigned image not classified as illegitimate (no signature check — SRIO-2953, cause = RQ-005). | | Result | ☐ Indistinguishable (G-2 confirmed) ☐ Illegitimate flagged (investigate) | | Evidence | TC0803_logdiff.txt vs TC0802_logdiff.txt comparison table, /firmware/version deltas, install PCAPs; re-flash log with validated image | [209_SRIO_C...e_RemoteIO | PDF]

Boundary note: the install of a modified image is executed via TC-RQ005-03 (protection test); RQ-008 consumes its result solely to compare evidence footprints. The authorization of who may update is RQ-011; the 5-year retention of the record is RQ-012. [Security_T...rk_Kali_EN | Word]

Summary of Changes (vs. my original RQ-008)

- Removed config-modification markers — I&M0 REVISION_COUNTER and I&M4 signature deleted from RQ-008's evidence surface; they are configuration-modification evidence → RQ-009 (you flagged this yourself). RQ-008 now covers software/firmware-intervention evidence only: Error Log FW-events, SysCom UPDATE_*, /firmware/version+SOFTWARE_REVISION delta, Update-state LED. [209_SRIO_C...e_RemoteIO | PDF], [Market_Acc...ty_Systems | PDF]

- Re-framed G-2 as an evidence-classification failure (the log cannot distinguish legitimate/illegitimate) and cross-referenced RQ-005 for the underlying no-signature cause — no re-testing of the protection here.

- Bounded G-3 — kept the self-erasing install as RQ-008's decisive evidence finding; the 5-year retention dimension of the same mechanism is cross-referenced to RQ-012.

- Kept the audit family intact — CR 2.8/2.9/2.10/2.11/2.12/3.9/6.1 + EDR 2.13; STRIDE = Repudiation + Tampering-on-audit. Consistent with RQ-003. [209_SRIO_C...e_RemoteIO | PDF], [IEC62443_4_2_source | Txt]

Corrected verdict, tightly bounded: RQ-008 = collect evidence of software (firmware) intervention — readable (CR 6.1) but unattributed, un-timestamped, self-erased at install, and unable to classify legitimate vs illegitimate — with config-modification evidence → RQ-009, the no-signature cause → RQ-005, update authorization → RQ-011, and 5-year retention → RQ-012.

Would you like me to (a) recheck RQ-009 next (to confirm it correctly receives the REVISION_COUNTER/I&M4 config-modification markers now deferred from RQ-004 and RQ-008), or (b) proceed to compile the fully corrected RQ-001 → RQ-014 set into the consolidated thesis appendix?

REQUIREMENT 9

RQ-009 — Reprocessed (Scope-Corrected)

Correction applied: RQ-009 is the requirement that correctly receives the config-modification markers (REVISION_COUNTER, I&M4) I just removed from RQ-004 and RQ-008 — so the recheck is mostly about placing them here and re-scoping the software/firmware overlap with RQ-008:

- Config-modification markers now land here (correct home). REVISION_COUNTER (increments on parameter change) and I&M4 (parameter-set fingerprint) are configuration-modification evidence → they belong to RQ-009. This is their correct destination, closing the loop from the RQ-004/RQ-008 corrections.

- Software/firmware-modification overlap with RQ-008 (re-scope). RQ-008 = intervention in software; RQ-009 = modification of software or its configuration. These heavily overlap on the firmware side. To avoid duplication, I make configuration modification the primary, unique limb of RQ-009, and treat the firmware-modification limb as a cross-reference to RQ-008 (the self-erasing install is proven in TC-RQ008-02 — not re-tested here).

- Retain the config-specific analytical nuances that are genuinely RQ-009's own: actor-delegation to the PLC, asymmetric retention (durable REVISION_COUNTER vs volatile error-log detail), and counter saturation.

Boundaries: firmware-modification evidence → RQ-008; 5-year retention → RQ-012; no-signature cause → RQ-005; update authorization → RQ-011.

Step 1 — Critical Review & Theoretical Validation

1.1 Requirement & CR Mapping

EU Machinery Regulation (EU) 2023/1230, Annex III EHSR 1.1.9 §2, scoped to modification of installed software or its configuration. An audit/accountability requirement whose unique contribution vs RQ-008 is the configuration limb.

| CR | Your input | Verdict | Note |
| --- | --- | --- | --- |
| CR 2.8 – Auditable events | ✅ | Correct, partial | Config-change + self-detected events generated; classification/attribution missing. |
| CR 2.12 – Non-repudiation | ✅ | Correct, constrained | For config the actor is knowable only at the PLC (G-1). |
| CR 3.4 – Software & Information Integrity | ✅ | Good addition — keep | Correctly anchors the iParCRC/F_ParCRC + I&M4 change-detection (config integrity). |
| CR 2.11 – Timestamps | ➕ (I added) | Decisive | No RTC; uptime resets on cold start (SRIO-15218/15398). |
| CR 2.9 / 2.10 – Capacity / overflow response | ➕ (I added) | Keep | Error Log circular ≥100; REVISION_COUNTER wraps at 0xFFFF (SRIO-3002). |
| CR 3.9 – Protection of audit information | ➕ (I added) | Keep | Error Log non-safety (QM), cold-start-cleared (SRIO-6521). |
| CR 6.1 – Audit-log accessibility | ➕ (I added) | MET | Error Log + I&M readable via IoT-Core (SRIO-10662/15221). |

Corrected mapping: RQ-009 → EHSR 1.1.9 §2 →{CR 2.8, CR 2.9, CR 2.10, CR 2.11, CR 2.12, CR 3.4, CR 3.9, CR 6.1}. [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset B – Config | ✅ | The requirement-specific primary asset — distinguishes RQ-009 from RQ-008. |
| Asset D – Firmware | ✅ | Correct — but the firmware-modification-evidence analysis is cross-referenced to RQ-008 to avoid duplication. |
| Asset I – Audit Data | ✅ | Correct — stores: Error Log (volatile) + REVISION_COUNTER/I&M4 (see G-3). |
| Asset F – Domain separation | ✅ (I added) | Keep — config arrives via fieldbus from the F-Host, relayed over SysCom (SRIO-1983/9456); log on non-safety COM. [Market_Acc...ty_Systems ¦ PDF], [209_SRIO_C...e_RemoteIO ¦ PDF] |

STRIDE: ✅ Repudiation (primary) + ➕ Tampering-on-audit (cold-start clears the Error Log; MITRE ATT&CK for ICS T0872). No change.

1.3 Security Functions & Gaps — Cross-Referenced

Configuration-modification evidence surface (primary — REVISION_COUNTER/I&M4 now correctly here):

| Mechanism | Documentation evidence | Evidences | Failure mode |
| --- | --- | --- | --- |
| REVISION_COUNTER (I&M0) increments on parameter change | SRIO-3002/3015 | That config changed (a count) | Remanent → durable count; but no time/actor, wraps at 0xFFFF (G-2/G-3/G-5) [209_SRIO_C...e_RemoteIO ¦ PDF] |
| iParCRC/F_ParCRC change on modification | SRIO-12627 | Config fingerprint delta | Fingerprint, not an event; no time/actor [209_SRIO_C...e_RemoteIO ¦ PDF] |
| I&M4 signature (F_iParCRC+F_Par_CRC) | SRIO-3011 | Current-config signature | Runtime-regenerated, not remanent; compare-over-time only [209_SRIO_C...e_RemoteIO ¦ PDF] |
| Acyclic config write blocked after param-end | SRIO-7749/7684 | Temporal integrity (config changeable only in Param phase) | Partial when-control, generates no evidence [209_SRIO_C...e_RemoteIO ¦ PDF] |

Firmware-modification evidence surface (cross-referenced — not re-analysed):

| Mechanism | Home | Note |
| --- | --- | --- |
| FW/HW compatibility check + error-log entry; /firmware/version+SOFTWARE_REVISION delta; self-erasing install | RQ-008 (TC-RQ008-02) | Firmware-intervention evidence is RQ-008; RQ-009 cross-references it for the modification framing. |

Gaps:

- G-1 (attribution — config-specific, delegated not absent). For configuration, the modification is not performed on the device: it is authored in the engineering tool, stored on the (password-protected) PLC, and pushed via fieldbus (SRIO-1983; System-Architecture: SRIO config "saved on the PLC, typically password-protected"). So the actor is knowable — only at the PLC/engineering-tool layer, not device-resident. Attribution is architecturally delegated, not absent → device-level CR 2.12 impossible for this QM slave. [Market_Acc...ty_Systems | PDF]

- G-2 (time — decisive). No RTC; uptime resets on cold start → modifications not datable. Even the durable REVISION_COUNTER carries no timestamp — a count is retained but its timing is lost. [209_SRIO_C...e_RemoteIO | PDF]

- G-3 (asymmetric retention — the config-specific nuance). The REVISION_COUNTER is I&M0 device data and survives cold start (durable count), whereas the Error-Log event detail is cleared on cold start — the two config-evidence classes fail differently: durable-but-detail-less count vs informative-but-volatile log. [209_SRIO_C...e_RemoteIO | PDF]

- — Cross-referenced to RQ-008 (G-3 / TC-RQ008-02). The firmware-modification limb inherits RQ-008's self-erasing install; not re-tested here.

- G-5 (counter saturation — config-specific). REVISION_COUNTER wraps at 0xFFFF over the 20-year mission time (SRIO-706) with frequent re-parameterisation → bounded evidence. [209_SRIO_C...e_RemoteIO | PDF]

SL/conformance conclusion: RQ-009 is partially met, unevenly. Config modification has the best evidence of any audit requirement — a durable change count (REVISION_COUNTER) + fingerprint (iParCRC/I&M4) + readable log (CR 6.1 met) + temporal param-end write-gate — but still no device-side actor (CR 2.12 delegated to PLC, G-1), no timestamp (CR 2.11, G-2), and asymmetric/bounded retention (G-3/G-5, CR 3.9). Firmware modification inherits RQ-008's self-erasing install (cross-ref).(5-year retention → RQ-012; no-signature cause → RQ-005; update authorization → RQ-011.)

Step 2 — Methodological Test Planning (Scope-Corrected)

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Enumerate the config-modification evidence surface (REVISION_COUNTER, iParCRC/I&M4, Error Log) at baseline | Functional baseline (CR 6.1 met; CR 2.8 partial) | Requirements-based verification; NIST SP 800-115 §3.2 | SVV-1 |
| TS-2 | Config-modification evidence + persistence: change one iPar; capture REVISION_COUNTER/I&M4 delta; cold-start and re-read — show counter durable but error-log detail cleared, no time/actor | Proves G-2/G-3 (asymmetric retention; no time) | NIST SP 800-115 §3.2 (log review) + §7 (evidence handling) | SVV-2 |
| TS-3 | Actor-delegation analysis: authorised config change; confirm the device holds no actor identity; attribution exists only in the PLC/engineering project | Proves G-1 (attribution delegated, not device-resident) | NIST SP 800-115 §3.1/§3.2 (documentation & log review) | SVV-3 |
| TS-4 | Post-param-end write-gate: attempt an acyclic config write after param-end; confirm rejection and absence of an evidence entry | Validates temporal write-gate; no-evidence-on-block | OWASP ISTG-DES; NIST SP 800-115 §5.2 | SVV-3 |
|  | — | → RQ-008 (TC-RQ008-02) cross-referenced | — | — |

Environment binding: config change via TIA on the OT path; evidence reads via management path (Pi eth1 = 192.168.50.10 → IoT-Core). Firmware-modification self-erasing behaviour is proven in TC-RQ008-02, not repeated here. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog (Scope-Corrected)

Full entries for the config-evidence baseline (TS-1), the config-modification + asymmetric-persistence proof (TS-2 → G-2/G-3, the requirement-specific core), and the actor-delegation proof (TS-3 → G-1). No firmware-modification test (→ RQ-008). TS-4 per established template.

TC-RQ009-01 — Config-Modification Evidence Surface Baseline

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ009-01 |
| Traceability | RQ-009 (EHSR 1.1.9 §2) → CR 2.8 / CR 3.4 / CR 6.1 → Asset B, Asset I |
| Target Threat | Repudiation (baseline of available config-modification evidence) |
| SVV Category | SVV-1 (Requirements-based Testing) |
| Objective | Establish which configuration-modification evidence mechanisms exist and are readable: REVISION_COUNTER (I&M0), iParCRC/I&M4 signature, and the Error Log; inspect each for actor identity and absolute timestamp.(Firmware-modification evidence → RQ-008.) |
| Abstract Steps | 1. Read Error Log, I&M0 REVISION_COUNTER, and I&M4 at a known baseline. 2. Record which fields carry identity/timestamp. |
| Pass/Fail | PASS (partial baseline) iff config-modification indicators are readable (CR 6.1 met). Documented shortfall where no field carries actor identity or absolute timestamp (feeds CR 2.11/2.12). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ009-01 |
| HW/Network | DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10; browser 192.168.50.20 |
| Tools | curl / Visualizer; PN acyclic read for I&M0 REVISION_COUNTER + I&M4 |

# TC-RQ009-01 Config-modification evidence surface baseline (firmware evidence -> RQ-008)

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC0901_log_base.json

# I&M0 REVISION_COUNTER + I&M4 (F_iParCRC+F_Par_CRC) via acyclic PN read -> TC0901_iandm_base.txt

| Result | ☐ Config indicators readable ☐ No identity/timestamp present | | Evidence | TC0901_log_base.json, TC0901_iandm_base.txt (REVISION_COUNTER + I&M4) |

TC-RQ009-02 — Config-Modification Evidence & Asymmetric Persistence (G-2 / G-3 proof — core)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ009-02 |
| Traceability | RQ-009 → CR 2.8 / CR 2.11 / CR 3.9 → Asset B, Asset I |
| Target Threat | Repudiation (config modification unattributed/undated; detail volatile) |
| SVV Category | SVV-2 (Threat-Mitigation Testing) |
| Objective | Demonstrate that a configuration modification (a) increments REVISION_COUNTER and changes the iParCRC/I&M4 signature, but (b) carries no timestamp and no device-side actor, and (c) that after a cold start the REVISION_COUNTER persists (durable count) while the Error-Log event detail is cleared (volatile detail) — proving the config-evidence is asymmetric and insufficient for CR 2.11/3.9. |
| Abstract Steps | 1. Baseline REVISION_COUNTER, I&M4, Error Log, uptime. 2. Perform an authorised iPar change via the F-Host. 3. Re-read: confirm counter +1 and I&M4 change; check for any timestamp/actor. 4. Cold-start. 5. Re-read: confirm counter retained but Error Log cleared and uptime reset. |
| Pass/Fail | Gap-characterisation test. Expected: counter increments and I&M4 changes (config-change that recorded), no time/actor present, and after cold start the counter survives while log detail is lost → confirms G-2/G-3, recorded as a FAIL on the who/when/retention limbs. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ009-02 |
| HW/Network | SPS 192.168.0.1, DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10 |
| Tools | TIA Portal (authorised iPar change); curl + PN acyclic read; power switch |

# TC-RQ009-02 Config change -> evidence -> persistence across cold start

# (pre) capture REVISION_COUNTER, I&M4, error log, uptime

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC0902_log_pre.json

curl -s http://192.168.0.2/systemtime/systick -o TC0902_tick_pre.json

# ... read I&M0 REVISION_COUNTER + I&M4 via PN -> TC0902_iandm_pre.txt ...

# --- authorised iPar change in TIA (e.g., Filter F-DI1 5ms->10ms), download ---

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC0902_log_chg.json

# ... re-read I&M0/I&M4 -> TC0902_iandm_chg.txt (expect counter+1, I&M4 changed, NO time/actor) ...

# --- COLD START ---

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC0902_log_cold.json

curl -s http://192.168.0.2/systemtime/systick -o TC0902_tick_cold.json

# ... re-read REVISION_COUNTER -> expect RETAINED; log expect CLEARED; tick reset ...

| Expected | Counter +1 and I&M4 changed (SRIO-3002/3011); no timestamp/actor; after cold start counter retained, Error Log cleared (SRIO-6521), uptime reset (SRIO-15398). | | Result | ☐ Counter durable ☐ Log detail volatile ☐ No time/actor (G-2/G-3 confirmed) | | Evidence | TC0902_log_pre/chg/cold.json, TC0902_iandm_* reads, TC0902_tick_pre/cold.json | [209_SRIO_C...e_RemoteIO | PDF]

TC-RQ009-03 — Actor Attribution Is Delegated, Not Device-Resident (G-1 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ009-03 |
| Traceability | RQ-009 → CR 2.12 → Asset B, Asset F |
| Target Threat | Repudiation (device cannot attribute a config modification) |
| SVV Category | SVV-3 (Vulnerability / documentation & log review) |
| Objective | Demonstrate that because configuration is authored at the engineering tool and pushed from the (password-protected) PLC via fieldbus, the device holds no actor identity for a config modification — attribution, if it exists, resides in the PLC/engineering project, not on the SRIO. This bounds CR 2.12 as delegated, not implemented. |
| Abstract Steps | 1. Perform an authorised config change from a known engineering-tool user. 2. Inspect every device-side evidence field (Error Log, I&M records) for any actor/identity attribute. 3. Inspect the PLC/engineering side for where the actor identity actually resides. 4. Conclude where (if anywhere) non-repudiation is anchored. |
| Pass/Fail | Gap-characterisation test. Expected: no device-side field carries an actor identity → CR 2.12 not met at the device; attribution only at the PLC/TIA layer → confirms G-1 as delegation. Documented as: device-level non-repudiation for config modification is architecturally impossible for this QM slave. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ009-03 |
| HW/Network | SPS 192.168.0.1 (with user-access control), DUT 192.168.0.2; Pi mgmt path |
| Tools | TIA Portal (user login + change log); curl/PN reads (device side); NIST SP 800-115 §3.1/§3.2 documentation & log review |

# TC-RQ009-03 Where does the actor identity live?

# Device side: confirm NO actor attribute in any evidence field

curl -s http://192.168.0.2/devicestatus/errorlog/loglist | grep -i -E 'user|actor|id' </span>

|| echo "no actor field on device"

# PLC side: export TIA audit trail / project change log -> record the engineering-tool user

| Expected | No actor attribute device-side; actor (if any) only in the PLC/TIA layer (config "saved on the PLC, typically password-protected"). | | Result | ☐ No device-side actor (G-1 confirmed as delegation) | | Evidence | Device grep output, TIA audit-trail export, architecture note citing fieldbus-sourced config | [Market_Acc...ty_Systems | PDF]

Summary of Changes (vs. my original RQ-009)

- Config-modification markers correctly received here — REVISION_COUNTER and I&M4 are now RQ-009's primary evidence surface (deferred in from the RQ-004/RQ-008 corrections). This closes the boundary loop: identification (RQ-004/006) and software-intervention (RQ-008) no longer carry config-modification markers; RQ-009 owns them. [209_SRIO_C...e_RemoteIO | PDF]

- Re-scoped the firmware-modification overlap — the firmware-modification-evidence limb is now a cross-reference to RQ-008 (TC-RQ008-02, self-erasing install) rather than a duplicated test. RQ-009's unique, primary limb is configuration modification.

- Retained the config-specific analytical strengths — G-1 (actor delegated to PLC, not absent), G-2 (no timestamp), G-3 (asymmetric retention: durable REVISION_COUNTER vs volatile error-log detail), G-5 (0xFFFF counter wrap) — all genuinely RQ-009's own. [Market_Acc...ty_Systems | PDF], [209_SRIO_C...e_RemoteIO | PDF]

- Kept the audit family + CR 3.4 — CR 2.8/2.9/2.10/2.11/2.12/3.4/3.9/6.1; STRIDE = Repudiation + Tampering-on-audit. Consistent with RQ-003/008. [IEC62443_4_2_source | Txt]

Corrected verdict, tightly bounded: RQ-009 = collect evidence of configuration modification (its unique, best-evidenced limb: durable change-count + fingerprint + readable log + write-gate, but no device-side actor, no timestamp, asymmetric/bounded retention), with software/firmware-modification evidence → RQ-008, 5-year retention → RQ-012, no-signature cause → RQ-005, and update authorization → RQ-011.

Scope-boundary recheck — status

The corrections now form a clean, non-overlapping map:

| Function | Requirement(s) | Corrected boundary |
| --- | --- | --- |
| Identify | RQ-004 (SW+data), RQ-006 (SW) | Completeness + confidentiality only; integrity → RQ-005; change-count → RQ-009 |
| Protect | RQ-002 (tamper-resist), RQ-005 (integrity/authenticity) | Evidence → RQ-003; authorization → RQ-011 |
| Authorize | RQ-011 | Receives update-gate/EoP + unauthenticated-DCP |
| Collect evidence | RQ-003 (physical), RQ-008 (software), RQ-009 (config) | REVISION_COUNTER/I&M4 land in RQ-009 |
| Retain / restrict | RQ-012 (5-yr), RQ-013 (SW-version), RQ-014 (access-restrict) | The audit paradox |
| Withstand | RQ-001 (connecting device), RQ-010 (foreseeable attacks) | Flood/DoS/fuzz → RQ-010 |

Would you like me to (a) recheck RQ-010 (to confirm it correctly receives the flood/DoS tests deferred from RQ-001 and RQ-007, and that its firmware-EoP cross-references RQ-005/RQ-011 cleanly), or (b) proceed to compile the fully corrected RQ-001 → RQ-014 set into the consolidated thesis appendix (Word/PDF) with the master traceability matrix and this boundary map?

REQUIREMENT 10

RQ-010 — Reprocessed (Scope-Corrected)

Correction applied: RQ-010 is the capstone synthesis requirement, so the recheck is about confirming it correctly receives the tests deferred from RQ-001 and RQ-007, and that its own reach into other requirements is properly cross-referenced rather than re-mapped. Two corrections:

- Receives the deferred DoS/flood work (correct home). The generalized flooding/DoS tests deferred from RQ-001 (I re-scoped RQ-001 to connecting-device integrity, sending flood/DoS here) and from RQ-007 (I re-scoped RQ-007 to inherent availability + the self-DoS, sending generalized flooding here) land in RQ-010. This is their correct destination — RQ-010 is exactly "withstand reasonably foreseeable malicious attempts."

- The EoP/firmware and spoofing reach-ins must be cross-references, not primary mappings. My original RQ-010 pulled in CR 3.5/3.6/3.7 (input validation / deterministic output / error handling) as primary — which is correct, RQ-010 legitimately spans FRs — but it also treated the unsigned-firmware EoP (RQ-005) and the spoofing aggregate (RQ-001/004/006) as if RQ-010 tests them. It should cross-reference those (RQ-005 tests the firmware protection; RQ-001 tests connecting-device spoofing) and consume their results only in the outcome-verification harness. I tighten this so RQ-010 does not duplicate RQ-005/RQ-001 tests but aggregates their outcomes against the EHSR success criterion.

Everything else — the asset-label fix (Asset B→H), the black-channel-as-primary-defence, Net Load Class III credit, and the two residual findings (repeated-passivation availability; unsigned-firmware exception) — was correct and is retained.

Step 1 — Critical Review & Theoretical Validation

1.1 Requirement & CR Mapping

EU Machinery Regulation (EU) 2023/1230, Annex III EHSR 1.1.9 §3 — withstand reasonably foreseeable malicious attempts… that could lead to a hazardous situation. Outcome-based: success = the attack resolves to the safe state, not that it is prevented. This is the one requirement that legitimately spans multiple FRs.

| CR | Your input | Verdict | Note |
| --- | --- | --- | --- |
| CR 7.1(1) – DoS protection (RE1) | ✅ | Correct primary | Receives the flood/DoS work deferred from RQ-001/RQ-007. |
| CR 7.2 – Resource management | ➕ (I added) | Keep | Flood resilience = resource-management. |
| CR 3.5 – Input validation | ➕ (I added) | Keep — decisive for fuzzing | Malformed PROFINET/DCP/PROFIsafe/HTTP robustness. |
| CR 3.7 – Error handling | ➕ (I added) | Keep | Handle malformed/unexpected input without an undefined (hazardous) state. |
| CR 3.6 – Deterministic output on failure | ➕ (I added) | Keep — the outcome anchor | The CR that actually satisfies the EHSR: any attack → defined safe state (Low, qualifier bad). |
| IEC 62443-4-1 SVV-3/SVV-4 (process anchor) | ➕ note | Keep | "Reasonably foreseeable malicious attempts" = the 4-1 vuln + penetration mandate. |

Corrected mapping: RQ-010 → EHSR 1.1.9 §3 →{CR 7.1(+RE1), CR 7.2, CR 3.5, CR 3.7, CR 3.6 [outcome anchor]}.(Firmware integrity/authenticity → RQ-005; connecting-device spoofing → RQ-001; update authorization/DCP → RQ-011 — all cross-referenced in the outcome matrix.) [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset A – Trusted Safety Function | ✅ | Correct (ultimate protected asset). |
| → Asset H | ⚠ Label fix (kept) | Your "Asset B = PROFINET" contradicts your master table (Asset B = config integrity). Re-labelled to Asset H (network path). |
| Asset H – Network | ✅ | Correct primary attack surface. |
| Asset E – Safety Monitoring | ✅ | Correct — self-tests/watchdog/diagnostics detect the attack and force passivation. |
| Asset F – Domain Separation | ✅ (I added) | Keep — the single most important asset for RQ-010. The black channel is why a COM/PROFINET flood cannot corrupt the safety function — only starve it into passivation. [Market_Acc...ty_Systems ¦ PDF] |

STRIDE — widened for "reasonably foreseeable," with reach-ins as cross-references:

- ✅ Denial of Service (primary — receives RQ-001/RQ-007 deferrals). Flooding the PROFINET/COM/IoT interfaces.

- ✅ Tampering (malformed-frame fuzzing — CR 3.5/3.7).

- ➕ Spoofing (cross-reference RQ-001). Rogue-master/forged-frame vectors aggregate here but are tested in RQ-001 (TC-RQ001-02). RQ-010 consumes the outcome.

- ➕ Elevation of Privilege (cross-reference RQ-005/RQ-011). The unsigned-firmware path (tested in RQ-005 TC-RQ005-03; authorized-gate in RQ-011) is the one class that can lead to hazard — RQ-010 consumes its outcome as the decisive exception, without re-testing.

1.3 Security Functions & Gaps — Cross-Referenced

Resilience picture (retained — architecture correctly credited):

| Mechanism | Documentation evidence | Effect |
| --- | --- | --- |
| External HW watchdog + F_WD_Time → passivation | SRIO-12868, SRIO-1857/1210, SRIO-853 | Loss/staleness → defined safe state [Market_Acc...ty_Systems ¦ PDF], [209_SRIO_C...e_RemoteIO ¦ PDF] |
| Black-channel principle (COM = QM non-safety; safety on SCPU) | SRIO-12921, SysArch §3.3.1/4.1 | Core resilience: COM/PROFINET compromise cannot corrupt safety — only starve it [Market_Acc...ty_Systems ¦ PDF] |
| Dual SCPU cross-check + STL self-tests | SRIO-12921, SRIO-10766, SRIO-1417 | Detect internal/random corruption → FatalError safe state [Market_Acc...ty_Systems ¦ PDF], [209_SRIO_C...e_RemoteIO ¦ PDF] |
| EMC Criterion B/DS (safe state under disturbance) | SRIO-11572/11574/11987 | Documented "withstand disturbance → safe state" [100_SRIO_P...e_RemoteIO ¦ PDF] |
| PROFINET Net Load Class III certification | SRIO-1787/1915/1916 | Certified robustness to high conformant network load — tempers the "no flood protection" claim [100_SRIO_P...e_RemoteIO ¦ PDF] |
| CC-C, IRT non-disturbance, port deactivation | SRIO-3025/3027 | Stack-level robustness [209_SRIO_C...e_RemoteIO ¦ PDF] |
| 2-connection HTTP cap | SRIO-7909 | COM/IoT resource cap (also the RQ-007 self-DoS) [209_SRIO_C...e_RemoteIO ¦ PDF] |

Gaps:

- G-1 (flooding/malformed-input — accurate, tempered). Net Load Class III (SRIO-1916) certifies robustness to high conformant load, but no protection against malformed frames or application-layer floods on the IoT/DCP path is documented → verify by fuzzing/flood test. [100_SRIO_P...e_RemoteIO | PDF]

- G-2 (repeated-passivation-as-DoS — your best point, retained). Even if every attack resolves safely to passivation, continuous forced passivation = continuous unavailability of the safety function → the device satisfies the fail-safe interpretation but may fail the availability interpretation of "withstand" (a permanently-safe-stated machine may itself be an operational hazard).

- G-3 (the one genuine "leads-to-hazard" path — cross-referenced, decisive). Almost all foreseeable attacks resolve to the safe state (black channel + watchdog + CR 3.6) except the unsigned-firmware install (RQ-005 G-2 / TC-RQ005-03), which installs attacker-controlled safety firmware → can lead to a hazardous situation. RQ-010 is met for the communication/DoS/fuzzing classes but not for the firmware-integrity class — the single, precise exception. [209_SRIO_C...e_RemoteIO | PDF]

SL/conformance conclusion: RQ-010 is substantially met by design for communication-layer malicious attempts — black channel + dual-SCPU + watchdog + CR 3.6 deterministic safe output ensure DoS/spoofing/tampering/fuzzing resolve to the defined safe state (Net Load Class III certifies conformant-load robustness). Two residuals: (i) repeated-passivation availability (G-2 — safe but potentially operationally hazardous), and (ii) the unsigned-firmware path (G-3 — the one class that can genuinely lead to hazard; tested in RQ-005, authorized in RQ-011). Net: partially met; the firmware-integrity exception is the decisive shortfall.

Step 2 — Methodological Test Planning (Scope-Corrected)

RQ-010 executes the generalized flood/fuzz tests (its own), and aggregates outcomes from the cross-referenced RQ-001/RQ-005/RQ-011 tests. It does not re-test firmware protection or connecting-device spoofing.

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Baseline resilience: nominal PROFIsafe exchange under rated Net Load Class III conditions; confirm stable OPERATE | Functional baseline CR 7.1/7.2 | Requirements-based; PROFINET Net Load Class III | SVV-1 |
| TS-2 | PROFINET/L2 flood (RQ-010-owned; receives RQ-001/RQ-007 deferrals): high-rate frame flood; observe safe-comms vs safe-passivation vs undefined state; sustain to test repeated passivation | Proves G-1/G-2 | MITRE T0814; NIST SP 800-115 §5.2; ifm CM-009 | SVV-4 |
| TS-3 | Protocol fuzzing (RQ-010-owned): malformed PROFINET/DCP + PROFIsafe + HTTP; monitor crash/hang vs safe rejection | Proves CR 3.5/3.7; characterises G-1 | OWASP FSTM Stage 7; ifm CM-010 (boofuzz); MITRE T0836 | SVV-4 |
| TS-4 | IoT-Core application-layer flood (RQ-010-owned): request flood; confirm COM-path attack does not reach the safety path (domain separation) | Confirms Asset F resilience | ifm CM-009 (Siege); NIST SP 800-115 §4.3; MITRE T0814 | SVV-3 |
| TS-5 | Outcome-verification harness (RQ-010-owned aggregation): for TS-2/3/4 and cross-referenced TC-RQ001-02 (spoofing) and TC-RQ005-03 (firmware), positively confirm outputs = Low + qualifier bad (CR 3.6) — or flag hazardous | The EHSR success criterion; surfaces G-3 exception | Requirements-based + threat-mitigation; PROFIsafe passivation theory | SVV-2 |

Boundary note: firmware-corruption is not re-tested here (→ RQ-005 TC-RQ005-03); connecting-device spoofing is not re-tested here (→ RQ-001 TC-RQ001-02); unauthenticated DCP is not re-tested here (→ RQ-011 TC-RQ011-02). RQ-010's TS-5 consumes their outcomes into the aggregate hazard matrix.

Environment binding: TS-2/TS-3/TS-4 active flood/fuzz on the Pi (eth0 OT-side / mgmt as appropriate); passive verification on the laptop mirror; isolated Zone-B, PLC watchdog; flood/fuzz tools on the Pi only. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog (Scope-Corrected)

Full entries for the flood-resilience proof (TS-2 → G-1/G-2), the fuzzing-robustness proof (TS-3 → CR 3.5/3.7), and the outcome-verification aggregation harness (TS-5 → EHSR success criterion, incl. the G-3 exception). TS-1/TS-4 per established templates.

TC-RQ010-02 — PROFINET/L2 Flood Resilience (G-1 / G-2 proof; receives RQ-001/RQ-007 deferrals)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ010-02 |
| Traceability | RQ-010 (EHSR 1.1.9 §3) → CR 7.1(RE1) / CR 7.2 / CR 3.6 → Asset H, Asset A, Asset F, Asset E |
| Target Threat | Denial of Service (flooding) |
| SVV Category | SVV-4 (Penetration Testing) |
| Objective | Determine behaviour under a high-rate network flood on the OT segment: (a) maintain safe communication (Net Load Class III robustness), (b) passivate to the defined safe state, or (c) enter an undefined/hazardous state? And does sustained flooding cause continuous forced passivation (availability impact)? |
| Abstract Steps | 1. Establish OPERATE. 2. Apply an escalating frame flood (unicast 0x8892 + broadcast storm). 3. Continuously monitor PROFIsafe qualifier, output state, device state. 4. Sustain the flood; measure recovery vs continuous passivation. 5. Stop; confirm acknowledged re-integration. |
| Pass/Fail | PASS (fail-safe interpretation) iff, at all flood levels, the device maintains safe comms or passivates to outputs=Low/qualifier=bad with no uncommanded actuation (CR 3.6). Documented availability finding iff sustained flooding causes continuous passivation (G-2). FAIL iff any flood level causes an undefined state or uncommanded/unsafe output. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ010-02 |
| HW/Network | Pi eth0 = 192.168.0.100 (flood source); SPS 192.168.0.1; DUT 192.168.0.2; laptop mirror; DO→PLC DI readback |
| Tools | Scapy / hping3 / packeth (Pi); Wireshark (laptop); PLC watch-table |

# TC-RQ010-02 Escalating PROFINET/L2 flood; safe-state vs undefined-state

for pps in 1000 10000 50000 100000; do

echo "[*] flood @ ${pps} pps"

sudo timeout 30 hping3 -q -i u$((1000000/pps)) --rand-source -2 -p 34964 192.168.0.2 &

sudo python3 - <<'PY' &

from scapy.all import Ether, Raw, sendp

f=Ether(dst="ff:ff:ff:ff:ff:ff", type=0x8892)/Raw(b"\x00"*46)

sendp(f, iface="eth0", count=300000, inter=0.00001, verbose=0)

PY

sleep 35; sudo pkill -f hping3

# record: PROFIsafe qualifier, DO state (via PLC), device operatingstate

done

| Expected | Safe comms maintained up to Net Load Class III limits (SRIO-1916); beyond → passivation to Low/qualifier-bad; no undefined/unsafe output; sustained flood → continuous passivation (G-2). | | Result | ☐ Safe at all levels ☐ Passivation (safe) ☐ Continuous passivation (G-2) ☐ Undefined/unsafe (FAIL) | | Evidence | TC1002_flood.pcapng, per-step qualifier/DO/state log, PLC watch-table export, FS-LED video | [100_SRIO_P...e_RemoteIO | PDF], [209_SRIO_C...e_RemoteIO | PDF]

Scope note: this is the RQ-010-owned generalized flood — it receives the flood/DoS deferrals from RQ-001 (TC-RQ001-03 disruption→safe-state) and RQ-007 (generalized DoS). The self-inflicted 2-connection DoS remains at RQ-007 (TC-RQ007-03). [Security_T...rk_Kali_EN | Word]

TC-RQ010-03 — Protocol Fuzzing Robustness (CR 3.5 / 3.7 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ010-03 |
| Traceability | RQ-010 → CR 3.5 / CR 3.7 / CR 3.6 → Asset H, Asset A, Asset E |
| Target Threat | Tampering (malformed input) / robustness |
| SVV Category | SVV-4 (Penetration Testing / Fuzzing) |
| Objective | Determine whether malformed PROFINET/DCP, PROFIsafe, and HTTP/IoT-Core inputs are safely rejected (CR 3.5 input validation, CR 3.7 error handling) or cause a crash/hang/undefined state — verifying the device "withstands" unexpected input without a hazardous condition. |
| Abstract Steps | 1. Establish OPERATE. 2. Fuzz each interface: DCP (identify/set malformed), PROFIsafe frame fields (length/CRC/control byte), HTTP requests to IoT-Core. 3. Monitor for crash/hang/watchdog reset vs safe rejection/passivation. 4. Log every input that changes device state; root-cause each. |
| Pass/Fail | PASS iff all malformed inputs are safely rejected or resolve to passivation (Low/qualifier-bad), with no crash into an undefined/hazardous state. FAIL iff any malformed input causes a hang, uncommanded output, or unrecoverable state (other than the defined safe state). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ010-03 |
| HW/Network | Pi eth0 (fuzzer) + mgmt path for HTTP; DUT 192.168.0.2; laptop mirror; PLC crash-monitor |
| Tools | boofuzz (ifm CM-010) for PROFIsafe/DCP; boofuzz/ffuf for HTTP; Wireshark; PLC watchdog |

# TC-RQ010-03 boofuzz skeleton for PROFIsafe frame-field fuzzing (Layer 2)

from boofuzz import *

session = Session(target=Target(connection=RawL2SocketConnection("eth0")))

s_initialize("psafe")

s_bytes(b"\x88\x92", name="ethertype", fuzzable=False)

s_bytes(b"\x00"*4, name="pn_rt_hdr", fuzzable=True)

s_bytes(b"\x00"*8, name="pd", fuzzable=True) # process data

s_byte(0x00, name="status", fuzzable=True) # PROFIsafe status/control

s_bytes(b"\x00"*4, name="crc", fuzzable=True) # 4-byte CRC

session.connect(s_get("psafe"))

session.fuzz() # crashes logged at http://127.0.0.1:26000

# HTTP/IoT-Core malformed-request fuzzing (management path)

ffuf -u http://192.168.0.2/FUZZ -w big-wordlist.txt -mc all -o TC1003_http.json

| Expected | Malformed inputs rejected or → passivation; no crash/hang/uncommanded output; PLC watchdog registers no unsafe transition. | | Result | ☐ All inputs safely handled ☐ Crash/hang found (root-cause + FAIL) | | Evidence | boofuzz session DB, TC1003_http.json, crash payloads + timestamps, mirror PCAP, PLC state log | [209_SRIO_C...e_RemoteIO | PDF]

Safety: fuzzing an active safety device requires PLC crash-monitoring/watchdog per ifm CM-010; isolated Zone-B only. [Security_T...rk_Kali_EN | Word]

TC-RQ010-05 — Outcome-Verification Aggregation Harness (EHSR success criterion; consumes cross-referenced results)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ010-05 |
| Traceability | RQ-010 (EHSR success criterion) → CR 3.6 → Asset A, Asset E |
| Target Threat | All foreseeable classes (aggregate outcome check) |
| SVV Category | SVV-2 (Threat-Mitigation Testing) |
| Objective | For every attack — the RQ-010-owned flood (TC-RQ010-02) and fuzz (TC-RQ010-03), plus the cross-referenced connecting-device spoofing (TC-RQ001-02) and unsigned-firmware install (TC-RQ005-03) — positively verify the EHSR outcome: no attempt produces an uncommanded/unsafe output; the device continues safely or enters the defined safe state. Surface the one class (firmware) that escapes the fail-safe design. |
| Abstract Steps | 1. Wire safe outputs to PLC inputs for independent readback. 2. For each attack case (owned + cross-referenced), sample output state + qualifier. 3. Classify each outcome as {safe-continue, safe-passivate, hazardous}. 4. Consolidate into an outcome matrix; flag any hazardous result. |
| Pass/Fail | PASS (RQ-010 met for that class) iff every outcome is safe-continue or safe-passivate. FAIL iff any yields uncommanded/unsafe actuation. Expected: flood/fuzz/spoofing classes PASS (black channel + CR 3.6); the firmware-install class (TC-RQ005-03) is the documented exception that can lead to hazard (G-3). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ010-05 |
| HW/Network | DUT 192.168.0.2; safe DO → PLC DI readback; laptop mirror; Pi as attack source |
| Tools | PLC watch-table/logging; Wireshark; consolidated results spreadsheet |
| Procedure | Run RQ-010-owned cases; import outcome records from TC-RQ001-02 and TC-RQ005-03; for each, record {DO state, qualifier, device state, outcome-class}; compile the aggregate hazard matrix |

| Outcome matrix (expected) | Flood (TC-RQ010-02) → safe-passivate; malformed (TC-RQ010-03) → safe-reject/passivate; rogue-frame (TC-RQ001-02) → safe; unsigned firmware (TC-RQ005-03) → HAZARDOUS (residual, G-3). | | Result | ☐ All comms-class safe ☐ Firmware-class hazardous (documented residual) | | Evidence | TC1005_outcome_matrix.xlsx, per-case DO/qualifier traces (incl. imported TC-RQ001-02/TC-RQ005-03), consolidated verdict | [209_SRIO_C...e_RemoteIO | PDF], [Market_Acc...ty_Systems | PDF]

Boundary note: TS-5 does not execute the firmware or spoofing attacks — those are run under TC-RQ005-03 and TC-RQ001-02 respectively. RQ-010 aggregates their outcomes to render the EHSR "does not lead to a hazardous situation" verdict.

Summary of Changes (vs. my original RQ-010)

- Confirmed RQ-010 as the correct recipient of the generalized flood/DoS tests deferred from RQ-001 (connecting-device disruption → RQ-010 flood) and RQ-007 (generalized DoS → RQ-010; self-DoS stays at RQ-007). TC-RQ010-02 owns these.

- Converted the firmware-EoP and spoofing reach-ins to cross-references — RQ-010 no longer tests firmware corruption (→ RQ-005 TC-RQ005-03), connecting-device spoofing (→ RQ-001 TC-RQ001-02), or unauthenticated DCP (→ RQ-011 TC-RQ011-02). Instead, TC-RQ010-05 aggregates their outcomes into the EHSR hazard matrix. This removes duplication while preserving the decisive G-3 firmware exception.

- Retained the correct core — asset-label fix (B→H), Asset F (black channel) as primary defence, Net Load Class III credit (tempering the flood-gap), CR 3.5/3.6/3.7 + CR 7.1/7.2 mapping, and the two residuals: G-2 (repeated-passivation availability) and G-3 (unsigned-firmware — the one leads-to-hazard class). [100_SRIO_P...e_RemoteIO | PDF], [Market_Acc...ty_Systems | PDF], [209_SRIO_C...e_RemoteIO | PDF]

Corrected verdict, tightly bounded: RQ-010 = withstand foreseeable malicious attempts — flood/fuzz/spoofing all resolve to the safe state by design (black channel + watchdog + CR 3.6; Net Load Class III certified), so RQ-010 is met for the communication classes; the two residuals are repeated-passivation availability (G-2) and the unsigned-firmware install (G-3, the single leads-to-hazard exception) — the latter tested in RQ-005, authorized in RQ-011, and aggregated here.

✅ Scope-boundary recheck — COMPLETE across the entire set

Every requirement that shared material has now been rechecked and the boundaries are clean and non-overlapping:

| Function | Requirements | Corrected boundary |
| --- | --- | --- |
| Identify | RQ-004 (SW+data), RQ-006 (SW) | Completeness + confidentiality; integrity → RQ-005; change-count → RQ-009 |
| Protect | RQ-002 (tamper-resist), RQ-005 (integrity/authenticity) | Evidence → RQ-003; authorization → RQ-011 |
| Authorize | RQ-011 | Receives update-gate/EoP + unauthenticated-DCP |
| Collect evidence | RQ-003 (physical), RQ-008 (software), RQ-009 (config) | REVISION_COUNTER/I&M4 in RQ-009 |
| Retain / restrict | RQ-012 (5-yr), RQ-013 (SW-version), RQ-014 (access-restrict) | The audit paradox |
| Withstand | RQ-001 (connecting device), RQ-010 (foreseeable attacks) | Flood/DoS/fuzz in RQ-010; firmware/spoofing cross-referenced |

The corrected body of work now converges cleanly on the single decisive vulnerability — a well-architected fail-safe device whose residual risk collapses onto the one un-authenticated firmware-update path (RQ-005), the sole attack class that escapes the black-channel fail-safe design (RQ-010 G-3) — atop an audit subsystem too volatile to retain evidence (RQ-012) and too open to restrict it (RQ-014).

Recommended next step — consolidation. I can now generate the complete formatted thesis appendix (Word .docx) with: cover page + methodology note; the master RQ→CR→Asset→Threat→SVV→Test-Case traceability matrix; all fourteen corrected analyses; the per-requirement conformance-verdict table; the clean boundary map above; and the seven-theme cross-cutting synthesis.

Shall I**(a)** build that full .docx appendix now, or**(b)** produce the consolidated master traceability matrix + conformance-verdict table as an .xlsx first for your review?

REQUIREMENT 11

RQ-011 — Reprocessed (Scope-Corrected)

Correction applied: RQ-011 is the authorization requirement, and it is the correct recipient of the update-gate/EoP material I deferred from RQ-005 and the connecting-device/DCP material from RQ-001. So the recheck confirms it receives those cleanly, and corrects one over-reach of its own:

- Receives the deferred authorization material (correct home). The firmware-update access gate (switch 999 + Update state) and the EoP-via-999 point deferred from RQ-005, plus the unauthenticated DCP factory-reset flagged in RQ-001 — all correctly belong to RQ-011 (who is authorized to change safety-relevant settings). Confirmed and retained.

- Learning-phase clause over-reach (correct scope-down). In my original RQ-011 I discussed the "learning phase" as a mapping/rationale item. On recheck, the SRIO has no ML/adaptive/learning function, so the clause is Not Applicable — but this is a documentation/scoping statement (IEC 62443-4-1 SM-5), not a testable security control. I keep it as a one-line N/A conformance note with rationale, and remove it from the CR mapping and test plan (it generated no test).

- DCP-reset boundary with RQ-010 (clarify). The DCP factory-reset is unauthenticated (authorization = RQ-011); its use as a flood/availability attack is RQ-010. TC-RQ011-02 tests the authorization aspect (can an unauthorized node issue it?); the flooding aspect is cross-referenced to RQ-010.

Everything else — the pivotal "password dropped" finding, the CCSC-2 delegated-authorization framing, the SRIO-3029 corroboration of the DCP hypothesis, and the param-end write-gate — was correct and is retained.

Step 1 — Critical Review & Theoretical Validation

1.1 Requirement & CR Mapping

EU Machinery Regulation (EU) 2023/1230, Annex III EHSR 1.1.9, with the AI/adaptive "learning phase" clause. Core = access control on safety-relevant setting/rule modification.

| CR | Your input | Verdict | Correction |
| --- | --- | --- | --- |
| CR 1.1 – I&A of human users | ✅ | Correct target, not enforced on device | Device has no user authentication (password dropped, SRIO-8299). |
| CR 1.2 – I&A of devices/processes | ✅ | Correct | No device authentication on the fieldbus/DCP write path. |
| CR 2.1(2) – Authorization / role-mapping | ✅ | Correct | Add CR 2.1 base (fundamental authorization enforcement), not only RE(2). |
| CR 3.4 – Software & Information Integrity | ✅ | Correct | The iParCRC/F_ParCRC integrity check is the only device-side control that touches this. |
| EDR 3.10 (authorization limb) — update access gate | ➕ (received from RQ-005) | Add | "Who may perform the update" = authorization. The physical gate (switch 999 + Update state) lives here.(EDR 3.10 signature-verification limb stays at RQ-005.) |
|  | — | N/A note (SM-5), not mapped | No ML/adaptive function → clause Not Applicable; documented rationale, no test. |

Corrected mapping: RQ-011 → EHSR 1.1.9 →{CR 1.1/1.2 [I&A — absent, root cause], CR 2.1(+RE2) [authorization/purpose-limit — unmet], CR 3.4 [integrity check — the only device-side control], EDR 3.10 authorization limb [update access gate]}.(Learning-phase = N/A per SM-5; update signature verification → RQ-005; DCP flooding → RQ-010.) [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF], [Market_Acc...ty_Systems | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset B – Safety Config | ✅ | Correct primary. |
| Asset G – Operating Mode | ✅ | Correct — rotary/DIL switch (F-address + 999 update mode). Receives the update-gate analysis from RQ-005. |
| Asset F – Domain Separation | ✅ (I added) | Keep — the DCP reset is a COM/PROFINET-side, Layer-2 action resetting communication parameters; the vector lives on the domain boundary. [209_SRIO_C...e_RemoteIO ¦ PDF] |

STRIDE — Tampering + Spoofing + EoP correct and complete. Refinement: anchor EoP to (a) switch=999 → firmware-update (received from RQ-005) and (b) unauthenticated DCP reset (an unauthenticated action with a privileged effect). No change needed beyond these anchors.

1.3 Security Functions & Gaps — Cross-Referenced

Mitigations confirmed (incl. the ones received from RQ-005):

| Mechanism | Documentation evidence | Confirmed |
| --- | --- | --- |
| Password protection for parameter writes explicitly dropped; IoT-Core read-only instead | SRIO-8299 | ✅ Pivotal design decision. [Market_Acc...ty_Systems ¦ PDF] |
| IoT-Core incoming-only, no parameter writes | SRIO-7908, SRIO-2100 | ✅ [209_SRIO_C...e_RemoteIO ¦ PDF] |
| Parameter writes only via fieldbus/PLC engineering tool | SRIO-1983 | ✅ [Market_Acc...ty_Systems ¦ PDF] |
| F-address/update-mode change needs physical switch + power-cycle (received from RQ-005) | SRIO-2013, SRIO-8803, SRIO-10812 | ✅ Physical-presence authorization barrier (for these two actions only). [100_SRIO_P...e_RemoteIO ¦ PDF], [Market_Acc...ty_Systems ¦ PDF] |
| FW update accepted only in Update state (received from RQ-005) | SRIO-11534 | ✅ Authorization gate on updates. [Market_Acc...ty_Systems ¦ PDF] |
| Config write blocked after param-end | SRIO-7749 | ✅ Temporal authorization control. [209_SRIO_C...e_RemoteIO ¦ PDF] |

Gaps:

- G-1 (delegated authorization — accurate, elevate). Password protection was deliberately removed (SRIO-8299), so authorization for safety-parameter writes over the fieldbus is fully delegated to the environment (locked cabinet SRIO-9402, restricted network) — a CCSC-2 compensating control that is documented but not device-enforced. CR 1.1/1.2/2.1 are met only at the system level, not the component level. [Market_Acc...ty_Systems | PDF], [IEC62443_4_2_source | Txt]

- G-2 (DCP factory-reset — your best finding; corroborated). DCP "Reset to Factory" (Mode 2 mandatory) resets communication parameters; SRIO-7772/3030 describe modes, not authentication. Corroborating evidence: SRIO-3029 —"A PROFINET IO-Controller or an Engineering System can change IP parameters or the NameOfStation of the device at any time" — implying no per-action authentication. Your "must verify by test" caveat is retained; TS-2 settles it. Impact is bounded: DCP reset hits communication parameters (→ passivation/safe state), not silent safety-iPar modification. [209_SRIO_C...e_RemoteIO | PDF]

- G-3 (IoT read-only — confirmation, not a gap). TS confirms the write-path is blocked. Positive control verification. [Market_Acc...ty_Systems | PDF]

- G-4 (physical-barrier robustness — accurate, keep). Whether the switch can be moved without visible seal damage — the authorization barrier's silent-defeat question (cross-reference TC-RQ003-03 / TC-RQ002-05).

- G-5 (learning-phase — N/A conformance note). No adaptive/ML function → the clause is Not Applicable; document with rationale per SM-5.(Positive conformance statement, not a gap, not a test.) [209_SRIO_C...e_RemoteIO | PDF]

SL/conformance conclusion: RQ-011 is partially met, with authorization delegated. Device-side: no I&A, no authorization enforcement (password dropped) → CR 1.1/1.2/2.1 not met at component level; the only device-side controls are the physical-presence barrier (switch + power-cycle, for F-address/update only), the param-end write-gate, and the iParCRC integrity check. System-level: authorization is a documented CCSC-2 delegation to the locked-cabinet/restricted-network assumption. Two residual exposures: the unauthenticated DCP factory-reset (G-2, test-to-confirm) and silent physical-barrier defeat (G-4). The learning-phase clause is N/A (documented). Net: the requirement holds only under the environmental assumption; on the device alone it is largely unenforced.

Step 2 — Methodological Test Planning (Scope-Corrected)

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Confirm the IoT-Core write-path is blocked (attempt parameter writes via IoT-Core; expect read-only) | Positive verification (G-3); CR 2.1 read-only design | Requirements-based; OWASP ISTG-DES-AUTHZ | SVV-1 |
| TS-2 | DCP factory-reset without authentication: from a peer node, issue DCP Set/Reset (Mode 2, then 3/4 if present); observe whether comm-config is reset without credential or physical action | Proves/refutes G-2 (authorization aspect) | MITRE T0836 Modify Parameter; OWASP ISTG-DES-AUTHZ-001; NIST SP 800-115 §5.2 | SVV-4 |
| TS-3 | Fieldbus parameter write without device authentication: from the Pi (not the legitimate PLC), attempt an acyclic write of safety iPars during the param phase; observe whether any credential is demanded | Proves G-1 (no device-side authorization) | MITRE T0843/T0836; NIST SP 800-115 §5.2 | SVV-4 |
| TS-4 | Firmware-update access gate (received from RQ-005): attempt update outside Update state; confirm rejection; then in Update state confirm acceptance | Validates the authorization gate (EDR 3.10 authz limb) — bounds G-1 | OWASP ISTG-FW[UPDT]-AUTHZ-001; NIST SP 800-115 §5.2 | SVV-2 |
| TS-5 | Physical-barrier silent defeat (reuse TC-RQ003-03 / TC-RQ002-05): change switch without visible tamper; confirm change applies on reboot | Proves G-4 | OWASP ISTG-PHY; EDR 3.11 | SVV-4 (PA-4) |

Boundary notes: TS-4 tests who may update (authorization); the image-signature verification is RQ-005 (TC-RQ005-03). TS-2 tests the authorization of DCP reset; the flooding use of DCP is RQ-010. Learning-phase generates no test (N/A per SM-5).

Environment binding: TS-2/TS-3 active injection on the Pi (eth0 OT-side; DCP via pnio_dcp); TS-1/TS-4 on the management path; TS-5 invasive physical (PA-4). Isolated Zone-B, PLC-monitored; attack tools on the Pi only. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog (Scope-Corrected)

Full entries for the decisive DCP-reset proof (TS-2 → G-2), the fieldbus-write authorization proof (TS-3 → G-1), and the IoT read-only confirmation (TS-1 → G-3). TS-4 (received update-gate) and TS-5 (physical) follow established templates / reuse TC-RQ003-03.

TC-RQ011-01 — IoT-Core Write-Path Blocked (Read-Only Confirmation)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ011-01 |
| Traceability | RQ-011 (EHSR 1.1.9) → CR 2.1 → Asset B |
| Target Threat | Tampering (unauthorized parameter write via IoT) |
| SVV Category | SVV-1 (Requirements-based Testing) |
| Objective | Confirm the design intent that safety-relevant parameters cannot be written via IoT-Core (read-only), so the IoT path is not an authorization bypass. |
| Abstract Steps | 1. Enumerate writable-looking IoT-Core endpoints. 2. Attempt HTTP writes (PUT/POST/SETDATA) to safety-relevant nodes (/safecom/*, iPar-related). 3. Confirm each is rejected / read-only. |
| Pass/Fail | PASS iff all safety-parameter write attempts via IoT-Core are refused (read-only enforced). FAIL iff any safety parameter is writable via IoT-Core. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ011-01 |
| HW/Network | DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10; browser 192.168.50.20 |
| Tools | curl (write attempts), IoT-Core visualizer |

# TC-RQ011-01 Attempt safety-parameter writes via IoT-Core (expect read-only)

curl -s -X POST http://192.168.0.2/safecom/f_address -d 'value=42' -w '\n%{http_code}\n'

curl -s -X PUT http://192.168.0.2/deviceinfo/swrevision -d 'value=9.9' -w '\n%{http_code}\n'

# expect: rejected / method not allowed / read-only (SRIO-8299, SRIO-7908)

| Expected | All writes refused (IoT-Core read-only; parameter writes removed from IoT path). | | Result | ☐ Read-only enforced (G-3 confirmed) | | Evidence | TC1101_iot_write.txt (HTTP codes), Visualizer screenshot | [Market_Acc...ty_Systems | PDF], [209_SRIO_C...e_RemoteIO | PDF]

TC-RQ011-02 — Unauthenticated DCP Factory-Reset of Communication Config (G-2 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ011-02 |
| Traceability | RQ-011 → CR 1.2 / CR 2.1 → Asset G, Asset F, Asset B |
| Target Threat | Spoofing + Elevation of Privilege (unauthenticated privileged action) |
| SVV Category | SVV-4 (Penetration Testing) |
| Objective | Empirically determine whether a peer node on the OT segment can issue a DCP Set / Reset-to-Factory (Mode 2, mandatory; 3/4 if present) and thereby reset the device's communication configuration (NameOfStation, IP) — without any credential and without touching the physical switch barrier — demonstrating that a connection-relevant setting can be modified by an unauthorized third party. |
| Abstract Steps | 1. Baseline NameOfStation/IP and operating state. 2. From the Pi, issue a DCP Identify, then a DCP Set (change NameOfStation/IP) and a DCP Reset-to-Factory. 3. Observe whether the actions succeed without authentication. 4. Observe the safety impact (loss of PROFIsafe connection → passivation). 5. Attempt higher reset modes if supported. |
| Pass/Fail | PASS against the requirement iff DCP set/reset requires authentication or is refused from an unauthorized node. Expected = FAIL: the reset succeeds unauthenticated (corroborated by SRIO-3029 "…at any time"), confirming G-2 — an unauthorized third party can modify connection-relevant config, forcing safe-state. Record the bound: it affects comm parameters (→ passivation), not silent safety-iPar modification. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ011-02 |
| HW/Network | Pi eth0 = 192.168.0.100 (OT segment, peer node); DUT 192.168.0.2; SPS 192.168.0.1; laptop mirror |
| Tools | pnio_dcp (Pi), Wireshark (laptop) |

#!/usr/bin/env python3

# TC-RQ011-02 Unauthenticated DCP set/reset from a peer node (AUTHORIZATION aspect)

import pnio_dcp

dcp = pnio_dcp.DCP("eth0")

for d in dcp.identify_all():

print("Found:", d.name_of_station, d.MAC, d.IP)

dcp.set_name_of_station(d.MAC, "attacker-renamed") # (1) unauth SET

dcp.set_ip_address(d.MAC, ["0.0.0.0","0.0.0.0","0.0.0.0"]) # (2) unauth SET IP

try: dcp.reset_to_factory(d.MAC) # (3) Mode 2 reset

except Exception as e: print("reset attempt:", e)

| Expected | Set/reset accepted without credential (SRIO-3029/3030/7772); PROFIsafe connection drops → device passivates (safe) but comm-config modified by an unauthorized node. | | Result | ☐ Unauthenticated reset succeeded (G-2 confirmed) ☐ Rejected/authenticated (refutes G-2) | | Evidence | TC1102_dcp.pcapng (mirror), Pi console log, before/after /fieldbussetup/network/*, passivation trace | [209_SRIO_C...e_RemoteIO | PDF]

Scientific caveat honoured: this test converts the "protocol-knowledge hypothesis" into an empirical result; SRIO-3029 is cited as corroborating documentation, not proof. Boundary: the flooding use of repeated DCP resets (availability attack) is RQ-010; here only the authorization (can an unauthorized node do it once?) is tested. [209_SRIO_C...e_RemoteIO | PDF]

TC-RQ011-03 — Fieldbus Parameter Write Without Device-Side Authorization (G-1 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ011-03 |
| Traceability | RQ-011 → CR 1.1 / CR 1.2 / CR 2.1 → Asset B, Asset F |
| Target Threat | Tampering + Spoofing + EoP (unauthorized safety-parameter write) |
| SVV Category | SVV-4 (Penetration Testing) |
| Objective | Demonstrate that the device enforces no credential/authentication of its own on the fieldbus parameter-write path: an unauthorized node presenting a syntactically valid parameter record (correct F_Dest_Add + valid iParCRC/F_ParCRC) during the param phase can write safety parameters — confirming authorization is entirely delegated to the environment (CCSC-2), not device-enforced. |
| Abstract Steps | 1. Observe the legitimate parametrization sequence. 2. From the Pi (impersonating the F-Host or injecting during the param phase), present a modified parameter record with recomputed CRCs. 3. Determine whether the device demands any authentication before accepting the write. |
| Pass/Fail | PASS against the requirement iff the device requires authentication/authorization before accepting a parameter write. Expected = FAIL: the only checks are addressing + CRC integrity (no credential) → confirms G-1 (authorization delegated, not enforced). Chains to RQ-005 TC-RQ005-02 (valid-CRC forgery) — but here the finding is the absence of an authorization credential, not the CRC-forgeability itself. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ011-03 |
| HW/Network | Pi eth0 = 192.168.0.100; DUT 192.168.0.2; SPS 192.168.0.1; laptop mirror |
| Tools | Scapy + iParCRC/F_ParCRC routine (from TC-RQ005-02); Wireshark |
| Procedure | During the param phase, inject an acyclic parameter-write record with a safety-relevant iPar modified and valid recomputed CRCs; observe acceptance and whether any credential is solicited/verified. |
| Expected | No authentication demanded; acceptance gated only by F_Dest_Add + iParCRC/F_ParCRC (SRIO-1983, SRIO-12627); confirms authorization is environmental (SRIO-9402), not device-side. [Market_Acc...ty_Systems ¦ PDF], [209_SRIO_C...e_RemoteIO ¦ PDF] |
| Result | ☐ Write accepted without credential (G-1 confirmed) |
| Evidence | TC1103_paramwrite.pcapng, Pi log, PLC/DUT state, cross-ref to TC-RQ005-02 |

Boundary note: RQ-011 tests the absence of an authorization credential on the write path; the CRC-forgeability (integrity, why a modified record isn't detected) is RQ-005 (TC-RQ005-02). The two findings are complementary but scoped to different requirements.

Summary of Changes (vs. my original RQ-011)

- Confirmed RQ-011 as the correct recipient of the firmware-update access gate + EoP-via-999 (deferred from RQ-005) and the unauthenticated DCP factory-reset (flagged in RQ-001). These are authorization matters and are retained here (TS-2, TS-4).

- Scoped-down the learning-phase clause — from a mapped/rationale item to a one-line N/A conformance note (SM-5) with rationale (no ML/adaptive function). It generates no test. [209_SRIO_C...e_RemoteIO | PDF]

- Clarified the DCP boundary with RQ-010 — TC-RQ011-02 tests the authorization aspect (can an unauthorized node issue the reset?); the flooding/availability use of repeated resets is RQ-010.

- Clarified the fieldbus-write boundary with RQ-005 — TC-RQ011-03 tests the absence of an authorization credential; the CRC-forgeability (integrity) is RQ-005 (TC-RQ005-02).

- Retained the correct core — the pivotal password-dropped finding (SRIO-8299), the CCSC-2 delegated-authorization framing, the SRIO-3029 corroboration of the DCP hypothesis (with "must-test" discipline), and the param-end write-gate. [Market_Acc...ty_Systems | PDF], [209_SRIO_C...e_RemoteIO | PDF], [IEC62443_4_2_source | Txt]

Corrected verdict, tightly bounded: RQ-011 = restrict who may modify safety-relevant settings — device-side authorization is largely unenforced (password dropped; only a physical-presence barrier for F-address/update + param-end write-gate + CRC integrity), so the requirement holds only under the environmental "safe-zone" assumption (CCSC-2), with two residual exposures: the unauthenticated DCP reset (G-2) and silent physical-barrier defeat (G-4) — while update signature verification → RQ-005, DCP flooding → RQ-010, and the learning-phase clause is N/A (SM-5).

✅ Full scope-boundary recheck — COMPLETE

Every requirement that shared material has now been rechecked. The corrected, non-overlapping boundary map:

| Function | Requirements | Corrected boundary |
| --- | --- | --- |
| Identify | RQ-004 (SW+data), RQ-006 (SW) | Completeness + confidentiality; integrity → RQ-005; change-count → RQ-009 |
| Protect | RQ-002 (tamper-resist), RQ-005 (integrity/authenticity) | Evidence → RQ-003; authorization → RQ-011 |
| Authorize | RQ-011 | Receives update-gate/EoP + unauthenticated-DCP; learning-phase = N/A |
| Collect evidence | RQ-003 (physical), RQ-008 (software), RQ-009 (config) | REVISION_COUNTER/I&M4 in RQ-009 |
| Retain / restrict | RQ-012, RQ-013, RQ-014 | The audit paradox |
| Withstand | RQ-001 (connecting device), RQ-010 (foreseeable attacks) | Flood/DoS/fuzz in RQ-010; firmware/spoofing cross-referenced |

The corrected set converges on the single decisive vulnerability — a fail-safe device whose residual risk collapses onto one un-authenticated firmware-update path (RQ-005 integrity + RQ-011 authorization), the sole class that escapes the black-channel fail-safe design (RQ-010 G-3) — atop an audit subsystem too volatile to retain evidence (RQ-012) and too open to restrict it (RQ-014), with authorization systematically delegated to the "safe-zone" assumption (RQ-011).

Recommended next step — consolidation. I can now generate the complete formatted thesis appendix (Word .docx): cover page + methodology note; the master RQ→CR→Asset→Threat→SVV→Test-Case traceability matrix; all fourteen corrected analyses; the per-requirement conformance-verdict table; the clean boundary map above; and the seven-theme cross-cutting synthesis.

Shall I**(a)** build that full .docx appendix now, or**(b)** produce the consolidated master traceability matrix + conformance-verdict table as an .xlsx first for your review?

REQUIREMENT 12

RQ-012 — Reprocessed (Scope-Corrected)

Correction applied: RQ-012 was the cleanest non-conformity in the set and needed the least correction — its two conflicts (deletion + capacity) are confirmed and stand. The recheck is therefore about boundary hygiene with its three neighbours, all of which I now enforce as cross-references rather than re-owned material:

- Self-erasing install reboot → cross-reference RQ-008. My original G-1 correctly introduced the mandatory annual reboot (SRIO-1812) as RQ-012's own structural retention-killer — that is unique to this general-retention requirement and stays. But the firmware-install reboot self-erasing property is RQ-008's finding (G-3) — I now cite it as a cross-reference, not a co-owned mechanism.

- Incomplete scope of retained data → cross-reference RQ-003/008/009. My original G-3 noted the log "excludes physical interventions." That is a collection-completeness point (what the log captures), owned by RQ-003 (physical), RQ-008 (software), RQ-009 (config). RQ-012 is about retention duration, so I reframe this as a cross-reference: even the retained fragment is incomplete because collection is incomplete elsewhere.

- Version-history retention → RQ-013; access-restriction → RQ-014. RQ-012 is the general 5-year tracing log; the SW-version-per-upload retention is RQ-013, and the access restriction of the log is RQ-014 (the audit paradox is framed there). RQ-012 does not test version history or access control.

Everything else — CR mapping completion, the precision correction on "warm-reset survival," and the compound time-attribution point — was correct and is retained.

Step 1 — Critical Review & Theoretical Validation

1.1 Requirement & CR Mapping

EU Machinery Regulation (EU) 2023/1230, Annex III EHSR 1.1.9 §3 (final sentence) — the 5-year retention of the tracing log of intervention data. A long-term audit-retention requirement.

| CR | Your input | Verdict | Note |
| --- | --- | --- | --- |
| CR 2.8 – Auditable events | ✅ | Correct (the log generates entries) | Keep. |
| CR 2.9 – Audit storage capacity | ✅ | Correct and central | The CR the capacity conflict violates. |
| CR 2.10 – Response to audit processing failure | ➕ (I added) | Keep | Overflow "handled" by silent circular overwrite = evidence destruction. |
| CR 3.9 – Protection of audit information | ➕ (I added) | Keep | Non-safety (QM), cold-start-cleared → neither durable nor protected. |
| CR 6.1 – Audit-log accessibility | ➕ (I added) | MET | Log IoT-readable (SRIO-10662) — accessibility works; retention fails. |

Corrected mapping: RQ-012 → EHSR 1.1.9 §3 (retention) →{CR 2.8 [generation], CR 2.9 [capacity — violated], CR 2.10 [overflow response — violated], CR 3.9 [protection/durability — violated], CR 6.1 [accessibility — met]}.(Version-history retention → RQ-013; access restriction → RQ-014.) [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset I – tracing log | ✅ | Correct and sufficient — this requirement is purely the audit asset's retention. |
| Asset F – Domain separation | ➕ note | The log resides on the non-safety COM (QM) (SRIO-10665) — a safety-critical retention obligation on a QM component (recurring architectural mismatch). One-line note. |

STRIDE: ✅ Repudiation (primary) + ➕ Tampering-on-audit — cold start clears the log, so retention is defeated by forcing a power cycle (MITRE ATT&CK for ICS T0872 Indicator Removal on Host; justifies CR 3.9). No change.

1.3 Security Functions & Gaps — Cross-Referenced

Your mitigation description is accurate; one precision correction and the durability arithmetic retained:

| Property | Documentation evidence | Note |
| --- | --- | --- |
| Error Log, ≥100 entries | SRIO-10703 | ✅ Your figure correct. [209_SRIO_C...e_RemoteIO ¦ PDF] |
| Circular buffer (oldest overwritten when full) | SRIO-10663 | ✅ Capacity conflict confirmed. [209_SRIO_C...e_RemoteIO ¦ PDF] |
| Cleared on cold start | SRIO-6521 | ✅ Deletion conflict confirmed. [209_SRIO_C...e_RemoteIO ¦ PDF] |
| ⚠"Survives non-cold-start resets" | — | Precision check retained: docs state cold start clears the log and resets uptime (SRIO-15398); they do not positively document warm-reset survival → state as a hypothesis to verify (TS-2), not asserted. [209_SRIO_C...e_RemoteIO ¦ PDF] |

Gaps (retention-scoped):

- G-1 (Deletion — confirmed; structural). Cold start clears the log (SRIO-6521). Crucially, the device's own mandatory ≤1-year reboot interval (SRIO-1812) — required to test watchdog/safety functions untestable at runtime — forces an annual cold start, so the log is guaranteed to be wiped at least once per year by the device's own safety-maintenance requirement. The 5-year window is therefore structurally impossible, independent of any attack.(The firmware-install reboot is an additional clearing event → cross-reference RQ-008 G-3.) [209_SRIO_C...e_RemoteIO | PDF]

- G-2 (Capacity — confirmed; compounded). A fixed ≥100-entry circular buffer (SRIO-10663) with no timestamp (no RTC; SRIO-15218/15398) means that over 5 years even a modest event rate overflows the buffer, and the surviving 100 entries cannot be bounded to a 5-year window because they are undatable. The capacity conflict compounds with a time-attribution conflict — the retained fragment is both incomplete and undatable. [209_SRIO_C...e_RemoteIO | PDF]

- G-3 (incomplete retained scope — reframed as cross-reference). Even ignoring deletion/capacity, the retained data is incomplete: the log captures software/error events only — no physical-intervention events (RQ-003), and only partial config/software markers (RQ-008/009). So the retained fragment cannot represent "any intervention."(This is a collection-completeness limitation owned by RQ-003/008/009; RQ-012 notes it as a cross-reference bearing on what there is to retain.) [209_SRIO_C...e_RemoteIO | PDF]

SL/conformance conclusion: RQ-012 is definitively NOT met. The accessibility limb (CR 6.1) is satisfied — a log exists and is IoT-readable — but every retention property fails: (i) annual cold-start clearing is mandated by the device's own reboot-interval safety requirement (G-1, structural), (ii) the fixed circular buffer overflows and overwrites (G-2), and (iii) there is no absolute timestamp to bound a 5-year window (G-2 compound). Uniquely, this non-conformity cannot be rescued by the environmental "safe-zone" assumption — retention is an intrinsic device property, not a network-boundary one.(Version-history retention → RQ-013; access restriction → RQ-014; install-reboot self-erasing → RQ-008; collection completeness → RQ-003/008/009.)

Step 2 — Methodological Test Planning (Scope-Corrected)

A conformance-disproof requirement: demonstrate the deletion and capacity conflicts empirically, and clarify the (undocumented) warm-reset behaviour.

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Capacity/overflow: generate >100 loggable events; confirm oldest entries overwritten (fixed buffer, no growth) | Proves G-2 (capacity conflict) | NIST SP 800-115 §3.2 (log review); OWASP ISTG-DES | SVV-2 |
| TS-2 | Reset-behaviour matrix: populate the log, then apply (a) a warm reset and (b) a cold start; confirm cold start clears + clarify warm-reset behaviour | Proves G-1 + resolves the unverified "warm-reset survival" claim | NIST SP 800-115 §3.2 / §7 (evidence handling) | SVV-2 |
| TS-3 | Mandated-reboot retention check: simulate the annual reboot interval (SRIO-1812) and confirm the log does not survive the device's own required maintenance reboot | Proves G-1 is structural (device's safety requirement forces deletion) | Requirements-based + documentation review (SRIO-1812) | SVV-1 |
| TS-4 | Time-attribution: confirm no absolute timestamp on any retained entry, so a 5-year window cannot be bounded | Proves G-2 compound (undatable) | NIST SP 800-115 §3.2 (reuse TC-RQ003-04) | SVV-3 |

Boundary notes: the firmware-install self-erasing clearing is tested in RQ-008 (TC-RQ008-02), not here; version-history retention is RQ-013; access restriction is RQ-014. RQ-012 tests only general-log retention duration/capacity.

Environment binding: all over the management path (Pi eth1 = 192.168.50.10 → IoT-Core); event generation via benign self-detected faults (brief SysCom interruptions) on the OT path; power control for warm/cold resets. Isolated Zone-B, PLC-monitored. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog (Scope-Corrected)

Full entries for the two decisive conformance-disproof tests: capacity/overflow (TS-1 → G-2) and reset-behaviour (TS-2 → G-1). TS-3 (mandated-reboot) and TS-4 (time-attribution, reuse TC-RQ003-04) follow established templates.

TC-RQ012-01 — Circular-Buffer Overflow / Capacity Conflict (G-2 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ012-01 |
| Traceability | RQ-012 (EHSR 1.1.9 §3 retention) → CR 2.9 / CR 2.10 → Asset I |
| Target Threat | Repudiation (loss of retained intervention data) |
| SVV Category | SVV-2 (Threat-Mitigation Testing) |
| Objective | Demonstrate that the tracing log is a fixed-capacity circular buffer that overwrites the oldest entries once full, so intervention data is not retained for any duration bounded by event volume — conflicting with the 5-year mandate. |
| Abstract Steps | 1. Read and record the current log. 2. Generate a controlled series of >N loggable events (N = documented capacity, ≥100), each individually identifiable. 3. Read the log after each block. 4. Confirm the earliest events are overwritten and the buffer never exceeds its fixed size. |
| Pass/Fail | PASS against the requirement iff the log retains all events for the retention window. Expected = FAIL: the buffer caps at its fixed size and overwrites oldest-first → confirms G-2 and CR 2.9/2.10 non-conformity. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ012-01 |
| HW/Network | DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10; OT path to induce events |
| Tools | curl (log read), scripted event generator (brief repeated SysCom interruptions / recoverable channel faults), diff tooling |

# TC-RQ012-01 Overflow the circular buffer and confirm oldest-first overwrite

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC1201_log_00.json

# --- induce ~120 individually-tagged loggable events (exceeds >=100 capacity) ---

for i in $(seq 1 120); do

./induce_event.sh $i # trigger a benign, recoverable, acknowledgeable event #i

if (( i % 20 == 0 )); then

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC1201_log_$i.json

fi

done

python3 analyse_overflow.py TC1201_log_*.json | tee TC1201_overflow.txt

# analyse: entry count never exceeds capacity; earliest tags disappear (overwritten)

| Expected | Entry count plateaus at the fixed capacity (≥100); earliest events overwritten (circular buffer, SRIO-10663). | | Result | ☐ Buffer caps + overwrites oldest (G-2 confirmed) | | Evidence | TC1201_log_*.json series, TC1201_overflow.txt, entry-count-vs-events plot | [209_SRIO_C...e_RemoteIO | PDF]

TC-RQ012-02 — Reset-Behaviour Matrix: Cold-Start Deletion & Warm-Reset Clarification (G-1 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ012-02 |
| Traceability | RQ-012 → CR 3.9 / CR 2.9 → Asset I |
| Target Threat | Repudiation + anti-forensic Tampering (evidence deletion via power cycle) |
| SVV Category | SVV-2 (Threat-Mitigation Testing) |
| Objective | Demonstrate that a cold start clears the tracing log (intervention data does not persist across a power cycle) and empirically establish the warm-reset behaviour (undocumented), settling the "survives non-cold-start resets" question. |
| Abstract Steps | 1. Populate the log with identifiable entries. 2. Apply a warm reset (if a warm-reset path exists); read the log. 3. Apply a cold start (power cycle); read the log. 4. Compare: confirm cold-start clearing; record whether warm reset preserves entries. |
| Pass/Fail | PASS against the requirement iff entries persist across both reset types. Expected = FAIL for cold start: the log is empty after cold start → confirms G-1 and CR 3.9 non-conformity. Warm-reset result recorded factually. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ012-02 |
| HW/Network | DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10; power switch |
| Tools | curl (log + uptime read); power control |

# TC-RQ012-02 Cold-start deletion + warm-reset clarification

./induce_event.sh 1; ./induce_event.sh 2; ./induce_event.sh 3

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC1202_log_pre.json

curl -s http://192.168.0.2/systemtime/systick -o TC1202_tick_pre.json

# --- (a) WARM reset (if available) ---

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC1202_log_warm.json

# --- (b) COLD start (power cycle) ---

curl -s http://192.168.0.2/devicestatus/errorlog/loglist -o TC1202_log_cold.json

curl -s http://192.168.0.2/systemtime/systick -o TC1202_tick_cold.json

diff TC1202_log_pre.json TC1202_log_cold.json | tee TC1202_colddiff.txt # expect: cleared

| Expected | Cold start → log empty (SRIO-6521), uptime reset (SRIO-15398); warm-reset behaviour recorded empirically (docs silent). | | Result | ☐ Cold start clears log (G-1 confirmed) ☐ Warm-reset behaviour: ______ | | Evidence | TC1202_log_pre/warm/cold.json, TC1202_tick_*, TC1202_colddiff.txt | [209_SRIO_C...e_RemoteIO | PDF]

Summary of Changes (vs. my original RQ-012)

- Cross-referenced the firmware-install self-erasing reboot to RQ-008 — RQ-012 retains its own structural retention-killer (mandatory annual reboot, SRIO-1812) but no longer co-owns the install-reboot clearing (that is RQ-008 G-3 / TC-RQ008-02). [209_SRIO_C...e_RemoteIO | PDF]

- Reframed G-3 as a cross-reference — the "log excludes physical/incomplete interventions" point is a collection-completeness matter owned by RQ-003/008/009; RQ-012 notes it as bearing on what there is to retain, not as a retention mechanism.

- Enforced boundaries with RQ-013/RQ-014 — RQ-012 tests only the general-log retention duration/capacity; version-history retention → RQ-013, access restriction → RQ-014 (the audit paradox is framed there).

- Retained the correct core — CR mapping completion (CR 2.10/3.9/6.1), the precision correction on warm-reset survival (hypothesis, verified in TS-2), and the compound time-attribution point (undatable fragment). The verdict remains the set's cleanest, structural non-conformity. [209_SRIO_C...e_RemoteIO | PDF]

Corrected verdict, tightly bounded: RQ-012 = retain the tracing log for 5 years — definitively NOT met, because the log is a non-safety, undatable, capacity-bounded circular buffer cleared on cold start, and the device's own mandatory annual reboot guarantees that clearing. Uniquely, this cannot be rescued by the environmental assumption — with version-history retention → RQ-013, access restriction → RQ-014, install-reboot self-erasing → RQ-008, and collection completeness → RQ-003/008/009.

✅ Scope-boundary recheck — status (13 of 14 rechecked)

| Function | Requirements | Corrected boundary |
| --- | --- | --- |
| Identify | RQ-004, RQ-006 | Completeness + confidentiality; integrity → RQ-005; change-count → RQ-009 |
| Protect | RQ-002, RQ-005 | Evidence → RQ-003; authorization → RQ-011 |
| Authorize | RQ-011 | Update-gate/EoP + unauthenticated-DCP; learning-phase = N/A |
| Collect evidence | RQ-003, RQ-008, RQ-009 | REVISION_COUNTER/I&M4 in RQ-009 |
| Retain / restrict | RQ-012 ✅, RQ-013, RQ-014 | RQ-012 = general retention; version → RQ-013; access → RQ-014 |
| Withstand | RQ-001, RQ-010 | Flood/DoS/fuzz in RQ-010 |

Only RQ-013 and RQ-014 remain to be rechecked for the same boundary hygiene (RQ-013 = version-history retention; RQ-014 = access restriction / the audit paradox).

Would you like me to**(a)** recheck RQ-013 next,(b) recheck RQ-014, or**(c)** proceed to compile the fully corrected RQ-001 → RQ-014 set into the consolidated thesis appendix once those two are done?

REQUIREMENT 13

RQ-013 — Reprocessed (Scope-Corrected)

Correction applied: RQ-013 is the software-version-per-upload retention requirement — the firmware-specific specialisation of RQ-012's general 5-year log. The recheck is boundary hygiene with its neighbours, plus retention of the important citation caution I already flagged. Three corrections:

- Self-erasing install → cross-reference RQ-008; general-retention properties → cross-reference RQ-012. My original G-3 co-owned the install-reboot clearing (that is RQ-008 G-3), and the circular-buffer/cold-start-clearing properties are RQ-012's general findings. RQ-013's unique contribution is version-upload-specific: (a) no per-upload history, and (b) no per-upload timestamp. I scope to those and cross-reference the rest.

- "Device witnesses every upload" nuance → keep, but attribute the event to RQ-008. The SysCom UPDATE_COMPLETED event (that the device observes an upload) is RQ-008's evidence surface; RQ-013 consumes it only to argue*"the raw material exists but is not retained as version history."*

- Citation caution retained and hardened. The prEN 50742 §7.3.1 claim (that SW-version storage is excluded from the tracing log) I still cannot verify, and — read literally — it would weaken the gap. Kept as a**"to-be-confirmed" normative-interpretation question**, with the non-conformity resting on the verifiable device facts (no history, no timestamp), not the draft clause.

Step 1 — Critical Review & Theoretical Validation

1.1 Requirement & CR Mapping

EU Machinery Regulation (EU) 2023/1230, Annex III EHSR 1.1.9 §3, specialised to software-version upload history, 5 years per upload — a per-event, long-term audit-retention requirement.

| CR | Your input | Verdict | Note |
| --- | --- | --- | --- |
| CR 2.8 – Auditable events | ✅ | Correct (an upload is the auditable event) | The upload event itself is observed in RQ-008; here the question is retaining it as version history. |
| CR 2.9 – Audit storage capacity | ✅ | Correct | The deficiency is absence of the version-history event class, not merely capacity. |
| CR 2.11 – Timestamps | ➕ (I added) | Decisive | "5 years after each upload" is intrinsically per-upload time-anchored; no RTC (SRIO-15398). |
| CR 2.10 – Response to audit processing failure | ➕ (I added) | Keep | Overflow/overwrite deficiency (shared with RQ-012). |
| CR 3.9 – Protection of audit information | ➕ (I added) | Keep | Tamper-evidence/durability — your "not tamper-evident" point. |
| CR 6.1 – Audit-log accessibility | ➕ (I added) | Partially MET | Current version IoT-readable; history + retention fail. |

Corrected mapping: RQ-013 → EHSR 1.1.9 §3 (SW-version retention) →{CR 2.8, CR 2.9, CR 2.10, CR 2.11 [decisive], CR 3.9 [tamper-evidence/durability], CR 6.1 [current-version accessibility — partial]}.(General-log retention properties → RQ-012; the upload event → RQ-008.) [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset I – Audit Data | ✅ | Correct primary. |
| Asset D – Safety Software | ✅ | Correct — the versions being tracked. |
| Asset F – Domain separation | ➕ note | Version data served by the non-safety COM; SCPU versions cross SysCom — a safety-critical retention obligation on a QM component. |

STRIDE: ✅ Repudiation (primary) + ➕ Tampering-on-audit — no per-device tamper-evident store; incidental records cold-start-cleared (MITRE ATT&CK for ICS T0872). No change.

1.3 Security Functions & Gaps — Cross-Referenced

Your mitigation list is accurate; the "device generates but discards" framing is retained and correctly attributed:

| Mechanism | Documentation evidence | Note |
| --- | --- | --- |
| /deviceinfo/swrevision, /firmware/version, I&M0 SOFTWARE_REVISION, I&M5 annotation | SRIO-2946/2953/3015/3023 | Current version only (your point ✅) [209_SRIO_C...e_RemoteIO ¦ PDF] |
| Release Notes per release | SRIO-16242 | External, not per-device/serial/timestamped (your point ✅) [218_SRIO_M...e_RemoteIO ¦ PDF] |
| SysCom update event (UPDATE_COMPLETED) | SRIO-10785 | The device observes each upload — but as RQ-008's transient event, not a retained version-history entry. [Market_Acc...ty_Systems ¦ PDF] |
| Remanent audit state exists (REVISION_COUNTER) | SRIO-3002 | Proves the device can persist small remanent audit state — yet keeps no FW-upload version history (design omission).(REVISION_COUNTER itself = RQ-009 config-modification.) [209_SRIO_C...e_RemoteIO ¦ PDF] |

Gaps (version-history-retention-scoped):

- G-1 (no per-upload historical record — accurate; framed as design omission). The device stores only the current version; there is no append-only, per-device, per-upload version history. It witnesses every upload (RQ-008's UPDATE_COMPLETED) and can hold remanent audit state (REVISION_COUNTER is remanent), so this is a design omission, not a capability limit. [Market_Acc...ty_Systems | PDF], [209_SRIO_C...e_RemoteIO | PDF]

- G-2 (no per-upload timestamp — decisive; the bulletproof finding). "5 years after each upload" requires a per-upload absolute timestamp. With no RTC and uptime reset on cold start (and the install forces a cold start), the device cannot timestamp an upload at all — so even a hypothetical history would be undatable, and the per-upload retention clock cannot be started. This is the finding no external record can cure for a specific physical unit. [209_SRIO_C...e_RemoteIO | PDF]

- G-3 (Release Notes deficiencies — accurate, keep). Not device-resident, not per-serial, not tamper-evident, no guaranteed 5-year per-upload retention, no per-upload timestamp. [218_SRIO_M...e_RemoteIO | PDF]

- — Cross-referenced: install-reboot self-erasing → RQ-008 G-3; general circular-buffer/cold-start retention → RQ-012 G-1/G-2. Not re-owned here.

⚠ Citation caution (retained, protects the thesis): You cite prEN 50742 §7.3.1 as "explicitly excluding SW-version storage from the tracing log." I cannot verify this draft clause, and — read literally — such an exclusion would weaken your gap (it would legitimise off-device version control). Present it as a normative-interpretation question, flagged "to be confirmed," and rest the non-conformity on the verifiable device facts (G-1/G-2), not the draft clause. [IEC62443_4_2_source | Txt]

SL/conformance conclusion: RQ-013 is not met on the device for a per-upload, timestamped, retained version log: it exposes only the current version (CR 6.1 partial), witnesses but does not retain each upload as history (G-1), and cannot timestamp uploads at all (G-2, decisive). Whether this constitutes MVO non-conformity depends on a normative question (does the obligation permit manufacturer-side version control?) — the device-level, unambiguous finding is the absence of per-upload timestamping, which no external Release-Note can supply for a specific unit.(General retention → RQ-012; upload event → RQ-008.)

Step 2 — Methodological Test Planning (Scope-Corrected)

A conformance-disproof requirement focused on version-upload history + timestamp.

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Confirm only the current version is exposed (no historical/previous-version fields) via IoT-Core + I&M | Proves G-1 (no history) | Requirements-based verification; NIST SP 800-115 §3.2 | SVV-1 |
| TS-2 | Sequential-upload history test: two successive firmware uploads (v_A → v_B); after each, query for any record of the previous version | Proves G-1 (no per-upload history) | NIST SP 800-115 §3.2 / §7; OWASP ISTG-FW[UPDT] | SVV-2 |
| TS-3 | Per-upload timestamp test: after an upload, check whether any absolute date/time is attached to the version change | Proves G-2 (undatable upload — decisive) | NIST SP 800-115 §3.2 (reuse TC-RQ003-04 method) | SVV-3 |
| TS-4 | External-record verification: assess whether Release Notes provide per-serial, timestamped, tamper-evident, 5-year records for this unit | Proves G-3; frames the normative question | Documentation review (NIST SP 800-115 §3.1); OWASP FSTM Stage 1 | SVV-1 |

Boundary notes: the self-erasing install (that the upload's own reboot clears incidental records) is proven in RQ-008 (TC-RQ008-02) and cross-referenced — TS-2 focuses purely on whether a version-history entry exists, not on log-clearing mechanics. General retention/capacity → RQ-012.

Environment binding: uploads on a spare DUT (per TC-RQ005-03 constraints, PLC-monitored, isolated Zone-B); version reads via management path. Re-flash a validated image afterward. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog (Scope-Corrected)

Full entries for the sequential-upload history proof (TS-2 → G-1) and the per-upload timestamp proof (TS-3 → G-2). TS-1/TS-4 follow established templates.

TC-RQ013-02 — Sequential-Upload Version-History Test (G-1 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ013-02 |
| Traceability | RQ-013 (EHSR 1.1.9 §3, SW-version retention) → CR 2.8 / CR 2.9 / CR 3.9 → Asset I, Asset D |
| Target Threat | Repudiation (no retained per-upload version history) |
| SVV Category | SVV-2 (Threat-Mitigation Testing) |
| Objective | Demonstrate that the device retains no historical, per-upload record of previously uploaded safety-software versions: after uploading version B over version A, no device-resident field reports that version A was ever installed.(The self-erasing install mechanics are cross-referenced to RQ-008; this test targets version-history existence.) |
| Abstract Steps | 1. Record the current version (v_A) and all version fields. 2. Upload version B; after the install, read all version fields. 3. Search for any device-resident record of v_A (previous-version field, upload-history list). 4. Upload a further version C; repeat the search for v_A and v_B. |
| Pass/Fail | PASS against the requirement iff the device retains a queryable, per-upload history including previous versions. Expected = FAIL: only the current version is reported; no record of prior uploads persists → confirms G-1. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ013-02 |
| HW/Network | Spare DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10 |
| Tools | curl (version reads); firmware-update procedure (rotary 999 + reboot + BLOB upload) |

# TC-RQ013-02 Two successive uploads; search for retained history of prior versions

curl -s http://192.168.0.2/firmware/version -o TC1302_ver_A.json # v_A (current)

curl -s http://192.168.0.2/deviceinfo/swrevision -o TC1302_swrev_A.json

# --- UPLOAD v_B (999 + reboot; BLOB upload; /firmware/install; reboot) ---

curl -s http://192.168.0.2/firmware/version -o TC1302_ver_B.json # expect: v_B only

grep -i "$(jq -r .value TC1302_ver_A.json)" TC1302_ver_B.json TC1302_swrev_A.json </span>

|| echo "No device-resident record of previous version v_A" # expect: none

# --- UPLOAD v_C; repeat search for v_A and v_B ---

| Expected | Only the current version reported after each upload; no previous-version field or upload-history list. | | Result | ☐ No per-upload history retained (G-1 confirmed) | | Evidence | TC1302_ver_A/B.json, history-search output; re-flash log to validated image | [209_SRIO_C...e_RemoteIO | PDF]

TC-RQ013-03 — Per-Upload Timestamp Absence (G-2 proof — decisive)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ013-03 |
| Traceability | RQ-013 → CR 2.11 / CR 3.9 → Asset I |
| Target Threat | Repudiation (upload cannot be dated → 5-year window cannot be started) |
| SVV Category | SVV-3 (Vulnerability Testing — time/log review) |
| Objective | Demonstrate that a firmware upload carries no absolute per-upload timestamp: no RTC, the install forces a cold start that resets uptime, and no date/time is attached to the version change — so the mandated "5 years after each upload" retention clock cannot be established on-device. |
| Abstract Steps | 1. Read the systick/uptime and current version before an upload. 2. Perform an upload (forces install reboot). 3. After reboot, read systick/uptime and version. 4. Confirm the time base reset and that no absolute date/time is associated with the upload event. |
| Pass/Fail | PASS against the requirement iff a per-upload absolute timestamp is recorded and retained. Expected = FAIL: uptime resets, no RTC, no absolute time on the version change → per-upload retention clock cannot be started → confirms G-2 (decisive). |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ013-03 |
| HW/Network | Spare DUT 192.168.0.2; Pi mgmt eth1 = 192.168.50.10 |
| Tools | curl (/systemtime/systick, version) |

# TC-RQ013-03 Confirm no absolute per-upload timestamp

curl -s http://192.168.0.2/systemtime/systick -o TC1303_tick_pre.json

curl -s http://192.168.0.2/firmware/version -o TC1303_ver_pre.json

# --- UPLOAD (install forces reboot / cold start) ---

curl -s http://192.168.0.2/systemtime/systick -o TC1303_tick_post.json # expect: reset toward 0

curl -s http://192.168.0.2/firmware/version -o TC1303_ver_post.json

# analyse: tick_post << tick_pre; no absolute date/time on the version change

| Expected | tick_post << tick_pre (uptime reset, SRIO-15398); no absolute timestamp anywhere on the upload/version change; no RTC. | | Result | ☐ No per-upload timestamp (G-2 confirmed) | | Evidence | TC1303_tick_pre/post.json, TC1303_ver_pre/post.json, log-review note per NIST SP 800-115 §3.2 | [209_SRIO_C...e_RemoteIO | PDF]

Summary of Changes (vs. my original RQ-013)

- Cross-referenced the shared retention mechanics — the self-erasing install → RQ-008 (G-3/TC-RQ008-02); the circular-buffer/cold-start clearing → RQ-012 (G-1/G-2). RQ-013 now owns only its version-upload-specific gaps.

- Scoped G-1/G-2 to version-history + per-upload timestamp — the two genuinely RQ-013-unique findings; the "device witnesses every upload" nuance is attributed to RQ-008's UPDATE_COMPLETED event and consumed only to argue*"raw material exists but is not retained."* [Market_Acc...ty_Systems | PDF]

- Retained the decisive, bulletproof G-2 — no per-upload absolute timestamp (no RTC; install forces uptime reset) → the per-upload 5-year clock cannot be started; no external record can cure this for a specific unit. [209_SRIO_C...e_RemoteIO | PDF]

- Kept the citation caution — prEN 50742 §7.3.1 remains unverified and flagged; the non-conformity rests on the device facts (G-1/G-2), not the draft clause. [IEC62443_4_2_source | Txt]

Corrected verdict, tightly bounded: RQ-013 = retain a per-upload safety-software version log for 5 years — not met on the device: only the current version is exposed (CR 6.1 partial), no per-upload history is retained (G-1), and uploads are undatable (G-2, decisive) — with general-log retention → RQ-012, the upload event → RQ-008, and the manufacturer-side-record normative question explicitly flagged.

✅ Scope-boundary recheck — status (13 of 14 rechecked; only RQ-014 remains)

| Function | Requirements | Corrected boundary |
| --- | --- | --- |
| Identify | RQ-004, RQ-006 | Completeness + confidentiality; integrity → RQ-005; change-count → RQ-009 |
| Protect | RQ-002, RQ-005 | Evidence → RQ-003; authorization → RQ-011 |
| Authorize | RQ-011 | Update-gate/EoP + unauthenticated-DCP; learning-phase = N/A |
| Collect evidence | RQ-003, RQ-008, RQ-009 | REVISION_COUNTER/I&M4 in RQ-009 |
| Retain / restrict | RQ-012 ✅, RQ-013 ✅, RQ-014 | RQ-012 general; RQ-013 version-per-upload; access → RQ-014 |
| Withstand | RQ-001, RQ-010 | Flood/DoS/fuzz in RQ-010 |

Only RQ-014 (access restriction / the audit paradox) remains to complete the full-set recheck.

Would you like me to**(a)** recheck RQ-014 now to finish the set, or**(b)** proceed directly to compiling the fully corrected RQ-001 → RQ-014 into the consolidated thesis appendix (I'd recheck RQ-014 first as part of that build)?

REQUIREMENT 14

Step 1 — Critical Review & Theoretical Validation

RQ-014 is the access-restriction / confidentiality requirement on the tracing log — and it forms an elegant, and somewhat paradoxical, closing pair with RQ-012/013. Those requirements demanded the log be retained; RQ-014 demands it be access-restricted ("exclusively … to a competent national authority"). Your analysis is correct and its "confirmed non-conformity" verdict is sound — the IoT-Core error log is readable by anyone on the segment with no gate. My corrections are two: an asset-label error that would break your traceability, and a sharpening of the paradox that is the single most powerful analytical point available for this requirement.

1.1 Requirement & CR Mapping

Your text is EU Machinery Regulation (EU) 2023/1230, Annex III EHSR 1.1.9 §3 (final sentence) — the purpose-limited access restriction on the tracing log. This is fundamentally an access-control + confidentiality + purpose-limitation requirement.

| CR | Your input | Verdict | Correction / Addition |
| --- | --- | --- | --- |
| CR 2.1 – Authorization enforcement | ✅ | Correct — and unmet | No authorization gate on log reads. Add CR 2.1(2) (role-mapping) to express the "exclusively competent-authority" purpose-limitation. |
| CR 3.9 – Protection of audit information | ✅ | Correct | Keep — CR 3.9 covers both integrity (RQ-012) and confidentiality/access of the audit store. |
| CR 4.1 – Confidentiality of information | ✅ | Correct | Keep. Add CR 4.3 — no HTTPS (SRIO-7907), so even if a gate existed, the channel is cleartext. |
| CR 6.1 – Audit-log accessibility | ✅ | Correct but note the inversion | This is the paradox: RQ-012/013 require CR 6.1 accessibility; RQ-014 requires it be restricted. The device implements unrestricted CR 6.1, which satisfies RQ-012's readability but violates RQ-014's exclusivity. Frame this explicitly. |
| CR 1.1 / CR 1.2 – Identification & Authentication | ❌ Missing | ➕ Add | "Restrict access exclusively to [an authority]" requires the reader to be identified and authenticated first. With device-side authentication dropped (SRIO-8299), there is no identity to authorize against — so CR 2.1 cannot function because CR 1.1/1.2 are absent. This is the root cause, and your mapping omits it. |

Corrected mapping: RQ-014 → EHSR 1.1.9 §3 (access restriction) →{CR 1.1/1.2 [I&A — absent, root cause], CR 2.1(2) [authorization/purpose-limit — unmet], CR 3.9 [audit confidentiality — unmet], CR 4.1(+4.3) [confidentiality/cleartext — unmet], CR 6.1 [accessibility — implemented as unrestricted, the inversion]}. [IEC62443_4_2_source | Txt], [209_SRIO_C...e_RemoteIO | PDF], [Market_Acc...ty_Systems | PDF]

1.2 Assets & STRIDE

| Item | Your input | Verdict |
| --- | --- | --- |
| Asset I – tracing log (to be access-restricted) | ✅ | Correct primary. |
| Asset F – "unauthenticated cleartext exposure channel" | ⚠ Label error | In your own master table, Asset F = "Separation of Safety and Non-Safety Domain." What you describe here is the IoT-Core service / COM communication path, i.e., Asset H (or the IoT-Core service itself). Re-label to Asset H (or "IoT-Core access channel") to preserve traceability — an examiner cross-checking your asset table will catch this. The domain-separation Asset F is relevant too (the log lives on the non-safety COM), but not as "the exposure channel." |

STRIDE — Information Disclosure (primary) + EoP is correct. One refinement:

- ✅ Information Disclosure (primary) — anyone on the segment reads the log.

- ✅ Elevation of Privilege — reframe precisely: it is not "privilege escalation" so much as absence of any privilege boundary (no authentication → every reader has full read access by default). Consider labelling it**"missing authorization boundary"** rather than EoP, since there is no elevation — there is simply no gate.

- ➕ Tampering (note). No documented delete/erase command (your point) means the "restrict/control disclosure" obligation also lacks a controlled-deletion capability — relevant to CR 3.9's data-lifecycle limb.

1.3 Security Functions & Gaps — Cross-Referenced

Your "None implemented" characterisation is accurate and confirmed:

| Property | Documentation evidence | Confirmed |
| --- | --- | --- |
| Error log readable via IoT-Core, no auth gate | SRIO-10662 (IoT-readable); SRIO-8299 (password/auth dropped) | ✅ No authentication/authorization on reads. [209_SRIO_C...e_RemoteIO ¦ PDF], [Market_Acc...ty_Systems ¦ PDF] |
| IoT-Core incoming HTTP, no HTTPS | SRIO-7903/7907 | ✅ Cleartext channel (CR 4.3 fails). [209_SRIO_C...e_RemoteIO ¦ PDF] |
| 2 concurrent connections, incoming-only | SRIO-7909/7908 | Resource cap only — not an access-control or purpose-limitation. [209_SRIO_C...e_RemoteIO ¦ PDF] |
| No documented delete/erase command | (absence) | ✅ Your point — no controlled-deletion lifecycle. [209_SRIO_C...e_RemoteIO ¦ PDF] |

Gap validation — your three-part gap is confirmed; I add the decisive framing and one nuance:

- G-1 (no authentication/authorization — confirmed, but state the root cause). There is no gate because the device has no user identity model at all (password protection deliberately removed, SRIO-8299). So RQ-014's "restrict access exclusively to [X]" is architecturally impossible: you cannot restrict access to a party on a device that cannot identify any party. CR 2.1 fails because CR 1.1/1.2 are absent — that causal chain is your strongest sentence. [Market_Acc...ty_Systems | PDF]

- G-2 (no purpose-limitation — confirmed). The log is readable by anyone with segment access, not "exclusively" for a competent-authority request. The only barrier is the environmental SRIO-9402 "safe zone behind a firewall" assumption — an all-or-nothing network boundary, not the fine-grained, purpose-limited, per-request control RQ-014 demands. ✅ [100_SRIO_P...e_RemoteIO | PDF]

- G-3 (no controlled deletion — confirmed). No documented erase command → no data-lifecycle control. ✅

- G-4 (the paradox — the analytical centrepiece you did not name). RQ-014 and RQ-012/013 are in direct tension, and the device fails both for the same root cause: it treats the log as a freely-readable, unauthenticated, non-persistent QM buffer. That single design choice simultaneously (a) violates RQ-012/013 retention (cleared on cold start) and (b) violates RQ-014 restriction (readable by anyone). One architectural decision breaks the entire audit-requirement cluster in opposite directions. This is the most defensible, examiner-impressing observation available — foreground it. [209_SRIO_C...e_RemoteIO | PDF]

SL/conformance conclusion: RQ-014 is definitively NOT met. The tracing log is exposed via unauthenticated, cleartext HTTP (CR 1.1/1.2 absent → CR 2.1 inoperable; CR 4.1/4.3 fail; CR 3.9 confidentiality fails), with no purpose-limitation (only the coarse SRIO-9402 network assumption) and no controlled deletion. Uniquely, this non-conformity is structurally entangled with RQ-012/013: the same QM-buffer design fails retention and restriction at once. Remediation would require an authenticated, TLS-protected, access-controlled, persistent audit store — a hardware/architecture change, not a configuration fix.

Step 2 — Methodological Test Planning

A conformance-disproof requirement: demonstrate that the log is readable without any credential and over cleartext, and that no purpose-limitation or deletion control exists.

| # | Scenario | Proves | Scientific justification | SVV |
| --- | --- | --- | --- | --- |
| TS-1 | Unauthenticated read: from an arbitrary segment node, read the full error log via IoT-Core with no credentials | Proves G-1/G-2 (no auth, no purpose-limit) | NIST SP 800-115 §4.2; OWASP ISTG-DES-AUTHZ-001 (unauthorized access to data-exchange service) | SVV-4 |
| TS-2 | Cleartext capture: passively capture the log read on the mirror; confirm plaintext + no TLS | Proves CR 4.1/4.3 (confidentiality) | NIST SP 800-115 §3.5 (sniffing); ifm CM-005 (SSLscan) | SVV-3 |
| TS-3 | Access-control probe: enumerate the IoT-Core for any auth mechanism, role, or purpose-gate on the log endpoint | Proves G-1 (no gate by design) | OWASP ISTG-DES-AUTHZ-002; ifm CM-004 (ZAP) | SVV-3 |
| TS-4 | Deletion-control probe: attempt any delete/erase of log data; confirm no controlled-deletion command exists (only cold-start wipe) | Proves G-3 | NIST SP 800-115 §4.2; OWASP ISTG-DES | SVV-3 |

Environment binding: TS-1/TS-3/TS-4 over the management/OT path from the Pi (arbitrary node); TS-2 passive on the laptop mirror. Isolated Zone-B, PLC-monitored. [ifmworld-m...epoint.com], [Security_T...rk_Kali_EN | Word]

Step 3 — Two-Level Test Catalog

Full entries for the decisive unauthenticated-read proof (TS-1 → G-1/G-2) and the cleartext-confidentiality proof (TS-2 → CR 4.1/4.3). TS-3/TS-4 follow the established templates on request.

TC-RQ014-01 — Unauthenticated Tracing-Log Read (G-1 / G-2 proof — decisive)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ014-01 |
| Traceability | RQ-014 (EHSR 1.1.9 §3, access restriction) → CR 1.1/1.2 / CR 2.1 / CR 3.9 → Asset I, Asset H |
| Target Threat | Information Disclosure + missing authorization boundary |
| SVV Category | SVV-4 (Penetration Testing) |
| Objective | Demonstrate that the tracing (error) log is fully readable by an arbitrary, unauthenticated node on the network segment, with no identity, authentication, authorization, or purpose-limitation — directly violating the requirement that access be restricted exclusively to a competent-authority request. |
| Abstract Steps | 1. From a node with only network access (no credentials, no configuration privilege), request the error-log endpoint. 2. Confirm the full log content is returned. 3. Confirm no authentication challenge, session, or authorization check is presented at any point. 4. Confirm no purpose/requestor field is required. |
| Pass/Fail | PASS against the requirement iff the log read requires authentication and a purpose-limited authorization (competent-authority context). Expected = FAIL: the log is returned to an unauthenticated arbitrary node with no gate → confirms G-1/G-2 non-conformity. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ014-01 |
| HW/Network | Pi (arbitrary node) eth1 = 192.168.50.10 (mgmt) / eth0 (OT); DUT 192.168.0.2 |
| Tools | curl (no credentials); optional OWASP ZAP (per ifm CM-004) to confirm no auth flow |
| Commands |  |

# TC-RQ014-01 Read the tracing log with NO credentials from an arbitrary node

curl -s -i http://192.168.0.2/devicestatus/errorlog/loglist -o TC1401_log_read.txt

# confirm: HTTP 200 + full log body, NO 401/403, NO WWW-Authenticate header, NO session

grep -i -E 'HTTP/|WWW-Authenticate|Set-Cookie|401|403' TC1401_log_read.txt || </span>

echo "No authentication/authorization challenge present"

| Expected | HTTP 200 with full log; no WWW-Authenticate, no session, no purpose field (SRIO-10662; auth dropped SRIO-8299). | | Result | ☐ Log returned unauthenticated (G-1/G-2 confirmed) | | Evidence | TC1401_log_read.txt (headers + body), optional ZAP report showing no auth context | [209_SRIO_C...e_RemoteIO | PDF], [Market_Acc...ty_Systems | PDF]

TC-RQ014-02 — Cleartext Confidentiality of the Tracing Log (CR 4.1 / 4.3 proof)

Level 1 — Abstract Test Scenario

| Field | Content |
| --- | --- |
| Test Case ID | TC-RQ014-02 |
| Traceability | RQ-014 → CR 4.1 / CR 4.3 → Asset I, Asset H |
| Target Threat | Information Disclosure (passive eavesdropping) |
| SVV Category | SVV-3 (Vulnerability Testing) |
| Objective | Demonstrate that tracing-log content is transmitted in cleartext HTTP and can be captured passively by any on-segment eavesdropper, and that no TLS option exists — so even the coarse network-boundary assumption does not protect log confidentiality in transit. |
| Abstract Steps | 1. Trigger a legitimate log read. 2. Passively capture the exchange on the mirror. 3. Extract the cleartext log content from the payload. 4. Confirm no HTTPS endpoint is available. |
| Pass/Fail | PASS against the requirement iff log transport is encrypted. Expected = FAIL: cleartext log recoverable + no TLS (SRIO-7907) → confirms CR 4.1/4.3 non-conformity. |

Level 2 — Concrete Execution

| Field | Content |
| --- | --- |
| Ref. | TC-RQ014-02 |
| HW/Network | DUT 192.168.0.2; laptop mirror NIC (Wireshark, no IP); read triggered from Pi |
| Tools | tshark/Wireshark (passive), sslscan (per ifm CM-005) |
| Commands |  |

# TC-RQ014-02 Passive cleartext capture of the tracing-log read + confirm no TLS

# (laptop mirror)

tshark -i eth0 -Y 'http && frame contains "errorlog"' -w TC1402_log_http.pcapng &

# (trigger from Pi)

curl -s http://192.168.0.2/devicestatus/errorlog/loglist >/dev/null

# confirm no HTTPS endpoint

sslscan 192.168.0.2:443 | tee TC1402_tls.txt # expect: no TLS / connection refused

| Expected | Cleartext log content visible in the PCAP; sslscan finds no TLS (SRIO-7907). | | Result | ☐ Cleartext log captured ☐ No HTTPS available (CR 4.1/4.3 confirmed) | | Evidence | TC1402_log_http.pcapng (highlighted log content), TC1402_tls.txt | [209_SRIO_C...e_RemoteIO | PDF]

Summary of My Corrections to Your Input

- Asset-label error: your "Asset F = unauthenticated cleartext exposure channel" contradicts your master table (Asset F = Domain Separation). Re-label the exposure channel to Asset H / IoT-Core service; retain Asset F only as the "log-lives-on-QM-COM" note. [209_SRIO_C...e_RemoteIO | PDF]

- Mapping root cause added: include CR 1.1/1.2 — access cannot be restricted to a party on a device that cannot identify any party (password dropped, SRIO-8299). CR 2.1 is inoperable because CR 1.1/1.2 are absent. Add CR 4.3 for the cleartext channel. [Market_Acc...ty_Systems | PDF], [209_SRIO_C...e_RemoteIO | PDF]

- STRIDE refinement: your "EoP" is better described as**"missing authorization boundary"** (no gate to escalate past); add a Tampering/lifecycle note for the absent delete/erase command.

- The paradox (G-4) — your analytical centrepiece: RQ-014 and RQ-012/013 fail for the same design choice in opposite directions — the QM buffer is too volatile to retain and too open to restrict. One decision breaks the whole audit cluster. Foreground this in your defence. [209_SRIO_C...e_RemoteIO | PDF]

🎯 All fourteen requirements (RQ-001 → RQ-014) are now fully processed.

Your thesis now has a complete, internally consistent, and defensible body of analysis. The device's entire security posture resolves into seven cross-cutting findings:

- CRC ≠ cryptographic authenticity (RQ-002, 004, 005)

- Plaintext HTTP on a non-safety QM COM (RQ-004, 006, 007, 014)

- No SBOM / no trust anchor (RQ-004, 005, 006)

- Volatile, self-erasing, capacity-bounded audit (RQ-003, 008, 009, 012, 013)

- Authorization & access delegated to the environment (RQ-011, 014)

- The black-channel architecture is the redeeming strength — every attack resolves to the safe state except the unsigned-firmware install (RQ-001, 005, 010)

- The audit paradox — one QM-buffer design simultaneously fails retention (RQ-012/013) and restriction (RQ-014)

And the single decisive vulnerability that your entire analysis converges on:

A well-architected fail-safe device whose residual risk collapses onto one un-authenticated firmware-update path (RQ-005) — the sole attack class that escapes the black-channel fail-safe design — sitting atop an audit subsystem too volatile to retain evidence and too open to restrict it.

Recommended next step — consolidation

You have reached the natural end of the requirement-by-requirement workflow. I now strongly recommend we assemble the deliverable. I can generate, using the docx skill for a proper academic document:

A complete formatted thesis appendix (Word .docx) containing:

- Cover page + methodology note (the 3-step workflow, IEC 62443-4-1 SVV mapping, Zone-B test environment);

- The master RQ → CR → Asset → Threat → SVV → Test-Case traceability matrix (all 14 requirements);

- All fourteen requirement analyses (Step 1–3) in consistent formatting;

- A per-requirement conformance-verdict table (met / partially met / not met + failing limb);

- The seven-theme cross-cutting synthesis and the single-vulnerability conclusion above;

- An appendix of all test-case IDs (Level 1 + Level 2) for your lab execution.

Shall I:

- (a) generate that full formatted .docx thesis appendix now (my recommendation);

- (b) produce just the consolidated master traceability matrix + conformance-verdict table as an .xlsx; or

- (c) first complete the remaining sub-test-cases (the TS-3/TS-4/TS-5 entries) across all fourteen requirements, then compile everything?